State of Libya

Sabha University

Faculty of Engineering

Department of Petroleum Engineering

Thesis Submitted in fulfillment of the requirements for degree of  bachelor's:

Determine the best solvents to avoid emulsion in crude oil using different types of crude oil

Student achievement:

Mohaned Muftah Sassi              Salah aldin Youssef Tanfara

22180099                                              22120054

Supervised by:

Mr. Ibrahim Al-Dukali

Mr. Hamad Abu Azoum

Academic year:

2024-2025

# الآية القرآنية

بِسْمِ اللَّهِ الرَّحْمَٰنِ الرَّحِيمِ

﴿وَقُل رَّبِّ زِدْنِي عِلْمًا﴾

(سورة طه، الآية 114)

شكر وتقدير

بسم الله الرحمن الرحيم

الحمد لله الذي أنعم علينا بنعمة العلم، ووفقنا للوصول إلى هذه المرحلة من مسيرتنا التعليمية. نتقدم بخالص الشكر والامتنان إلى عائلاتنا الكريمة الذين كانوا السند والداعم الأساسي لنا، ولكل من ساهم في تعليمنا وتوجيهنا منذ بداياتنا وحتى هذه اللحظة.

كما نعبر عن عميق شكرنا وتقديرنا للمشرفين الكرام على هذا البحث:

الأستاذ/ حمد أبو عزوم شلش

الأستاذ/ إبراهيم الدوكالي

لما قدموه من دعم علمي وتوجيهات بنّاءة كان لها الأثر الكبير في إنجاز هذا العمل.

ونتقدم بخالص الشكر والتقدير إلى الأستاذة/ مسعودة فرحات على دعمها المتواصل، وتشجيعها المستمر، وملاحظاتها القيمة.

ولا يفوتنا أن نتوجه بالشكر إلى إدارة حقل الشرارة النفطي، وعلى رأسها الأستاذ/ أبوبكر الأنصاري، على ما قدموه من تعاون كريم، وحسن استقبال.

# Abstract

Emulsification is one of the most common challenges in the petroleum industry. The demand for emulsifiers is increasing every day. Almost every company needs them in large quantities, but the source of emulsifiers is limited. Therefore, the search for new natural materials must be exported worldwide to reach buyers. The formation of emulsifiers in the petroleum industry, processing facilities, and pipelines has been a costly problem for the emulsion industry. Numerous studies have been conducted on this phenomenon. Some chemical methods and techniques have been used to prevent or predict emulsion formation in pipelines. The aim of this research is to study the problem of emulsion formation in the flow lines of processing facilities, which leads to crude oil processing. Various types of solvents, demulsifiers, natural material (MLX), and crude oil were used, and basic data from daily operating conditions were entered into a centrifuge incubation experiment. In this experiment, two different types of crude oil, M.F. and S.F., were used as relatively stable water-oil emulsions, and the density and temperature of each were measured.  A centrifuge was used to test the oil, using 100 ml tubes from the machine, the tubes were filled with 100 ml of each oil and placed in the machine, the speed was set at 1500 rpm, the time was set at 10 minutes, and the temperature was set at 60°C.

Key word: centrifugal device, emulsion, solvent, oil transportation lines, predict.

# List of contents

الآية القرآنية	ii

Abstract	iv

List of contents	v

List of Figure	ix

List of tables	xi

Chapter1

1. Introduction	1

1.1 The emulsion problem	1

1.2 Research Objectives	3

1.3 Project outline:	4

Chapter2

2. Literature Review	6

2.1 The origin of oil	6

2.2 Reservoirs	6

2.2.1 Types of reservoir fluids	6

2.2.1.1 Black Oils	7

2.2.1.2 Volatile Oils	8

2.2.1.3 Condensate (Retrograde Gas)	9

2.2.1.4 Wet Gas	11

2.2.1.5 Dry Gas	13

2.3 Definition emulsion:	13

2.3.1 Types of Emulsions	14

2.3.2 Formation of Emulsions	15

2.3.3 Emulsion stability	16

2.4 Demulsification	16

2.4.1 Chemical Demulsification	17

2.4.2 Factors affecting emulsion chemical demulsification	17

Chapter 3

3. Experimental Apparatus and Materials	20

3.1 Apparatus	20

3.1.1 Centrifuge	20

3.1.2 Test tube	21

3.1.3 Micropipette	21

3.1.4 Analytical Balance	22

3.1.5 Hydrometer	23

3.2 Materials	24

3.2.1 Demulsifiers	24

3.2.2 Solvents	26

3.2.3 polymer	28

3.2.4 Natural material	30

3.2.5 Crude oil	31

3.3 Experiment Procedure:	32

3.3.1 Procedure of polymer testing	33

3.3.2 Procedure of Solvent application	33

3.3.3 Testing of natural material	33

3.3.4 Demulsifiers evalution method	33

Chapter 4

4. Results and Discussion	36

4.1 Basic crude oil samples:	36

4.2. Effect of Demo (AS and AM)	37

4.2.1 Demo (AS and AM) - Concentration 0.25%	37

4.2.2 Demo (AS and AM) - Concentration 0. 5%	38

4.3 Organic Solvents (Xylene and Hexane)	40

4.3.1 Xylene and Hexane Solvent- Concentration 0.25%	41

4.3.2 Xylene and Hexane Solvent- Concentration 0. 5%	42

4.3.3Xylene and Hexane Solvent- Concentration 1%	45

4.4 Effect of Polymers A and B concentrations 0.5%	46

4.4.1 Polymer A and B - Concentration 0.25%	47

4.4.2 Polymer A and B - Concentration 0.5%	48

4.4.3 Polymer A and B - Concentration 1%	50

4.5. Polymer A and B - Concentration 1 %	53

4.5.1 Polymer A and B - Concentration 0.25%	53

4.5.2 Polymer A and B - Concentration 0. 5%	55

4.5.3 Polymer A and B - Concentration 1%	56

4.6 Nature material (MLX)	58

4.6.1. Basic crude oil samples	58

4.6.2 Nature material (MLX) at Concentration 0.25%	59

4.6.3. Nature material (MLX) at Concentration 0. 5%	61

4.6.4. Nature material (MLX) at Concentration 1%	62

Chapter 5

5.1 Conclusion	66

5.2 Recommendation	67

# List of Figure

Figure (2.1):  Black oil	8

Figure (2.2):  Volatile oil	9

Figure (2.3) : Condensate gas	11

Figure (2.4):  Wet get	12

Figure (2.5) :dry gas	13

Figure (2.6) :oil-in-water emulsion	14

Figure (2.7) : water-in-oil emulsion	14

Figure (2.8) : water-in-oil-in-water emulsion	15

Figure (3.1) :Centrifuge	20

Figure (3.2) : Test tube	21

Figure (3.3) : Micropipette	22

Figure (3.4) : Analytical balance	22

Figure (3.5) : Hydrometer	23

Figure (3.6) : Hydrometer Cylinder	24

Figure (3.7) : Demo AS	25

Figure (3.8) : Demo AM	25

Figure (3.9) : Hexane	26

Figure (3.10) : Xylene	27

Figure (3.11) : polymer  A	29

Figure (3.12) : polymer B	29

Figure (3.13) : Natural marerial (MLX)	30

Figure (3.14) : Two samples of crude oil used in this study.	31

Figure (3.15) : Crude oil samples	32

Figure (4.1) : Basic crude oil samples	36

Figure (4.2) : Demo(AS and AM) - Concentration 0.25%	38

Figure (4.3) : Demo(AS and AM) - Concentration 0. 5%	40

Figure (4.4) : Xylene and Hexane Solvent- Concentration 0.25%	42

Figure (4.5) : Xylene and Hexane Solvent- Concentration 0. 5%	44

Figure (4.6) : Xylene and Hexane Solvent- Concentration 1%	46

Figure (4.7) : Polymer A and B - Concentration 0.25%	48

Figure (4.8) : Polymer A and B - Concentration 0.5%	50

Figure (4.9) : Polymer A and B - Concentration 1%	52

Figure (4.10) : Polymer A and B - Concentration 0.25%	54

Figure (4.11) : Polymer A and B - Concentration 0. 5%	56

Figure (4.12) : Polymer A and B - Concentration 1%	58

Figure (4.13) : Basic crude oil samples (MLX)	59

Figure (4.14) :  (MLX) at Concentration 0.25%	60

Figure (4.15) : (MLX) at Concentration 0. 5%	62

Figure (4.16) :  (MLX) at Concentration 1%	64

# List of tables

Table (3.1) Types of crude oil, their densities (API) and temperature	32

Table (3.2) Concentration of polymers A and B	33

Table (4.1) Basic crude oil samples	36

Table (4.2) Demo (AS and AM) - Concentration 0.25%	37

Table (4.3) Demo(AS and AM) - Concentration 0. 5%	39

Table (4.4) Xylene and Hexane Solvent- Concentration 0.25%	41

Table (4.5) Xylene and Hexane Solvent- Concentration 0. 5%	43

Table (4.6) Xylene and Hexane Solvent- Concentration 1%	45

Table (4.7) Polymer A and B - Concentration 0.25%	47

Table (4.8) Polymer A and B - Concentration 0.5%	49

Table (4.9) Polymer A and B - Concentration 1%	51

Table (4.10)  Polymer A and B - Concentration 0.25%	53

Table (4.11) Polymer A and B - Concentration 0. 5%	55

Table (4.12) Polymer A and B - Concentration 1%	57

Table (4.13) Basic crude oil samples	59

Table (4.14) Nature material (MLX) at Concentration 0. 25%	60

Table (4.15) Nature material (MLX) at Concentration 0. 5%	61

Table (4.16) Nature material (MLX) at Concentration 1%	63

# Chapter1

Introduction

# 1. Introduction

Crude oil is seldom produced alone because it generally is commingled with water. The water creates several problems and usually increases the unit cost of oil production. The produced water must be separated from the oil, treated, and disposed of properly. all these steps increase costs. furthermore, sellable crude oil must comply with certain product specifications, including the amount of basic sediment and water and salt, which means that the produced water must be separated from the oil to meet crude specifications.

Produced water may be produced as "free" water (i.e., water that will settle out fairly rapidly), and it may be produced in the form of an emulsion. A regular oilfield emulsion is a dispersion of water droplets in oil. Emulsions can be difficult to treat and may cause several operational problems in wet-crude handling facilities and gas/oil separating plants. Emulsions can create high-pressure drops in flow lines, lead to an increase in demulsifier use, and sometimes cause trips or upsets in wet-crude handling facilities. The problem is usually at its worst during the winter because of lower surface temperatures. These emulsions must be treated to remove the dispersed water and associated inorganic salts to meet crude specifications for transportation, storage, and export and to reduce corrosion and catalyst poisoning in downstream processing facilities.
Emulsions occur in almost all phases of oil production and processing: inside reservoirs, wellbores, and wellheads; at wet-crude handling facilities and gas/oil separation plants; and during transportation through pipelines, crude storage, and petroleum processing [1].

# 1.1 The emulsion problem

The problem of separating water from produced crude oil is as old as the oil industry itself. At the beginning of the oil industry, the water separation problem was handled by settling the free water from oil in open tanks or pits and the sludge (an intermediate phase between clean water and clean oil) was disposed of, ordinarily by burning [2]. It was not more than a century ago that attention was drawn to the fact that” sludge” is an emulsion of crude oil and water, and that substantial amount of merchantable oil can be recovered from the emulsion [3]. By crude oil emulsions, we are referring to water-in-oil (W/O) emulsions because most emulsions are this type [4]. Although oil-in-water (O/W) emulsions also form and are encountered in the industry, they are generally resolved in the same way W/O emulsions are resolved, except electrostatic treaters cannot be used on O/W emulsions [5]. At the time when crude oil and water are leaving the wellbore of an oil well, tight w/o emulsions can form due to the turbulence in the choke valve at the wellhead [6]. The formation of emulsion during crude oil production is a very costly operational problem. It occurs when hydrocarbon and formation water in the reservoir and in production pipes are extremely mixed under shear/turbulence, and in the presence of surface-active agents [7].

The continuous phases of these emulsions depend on the water to oil ratio, the natural emulsifier systems contained in the oil, and the origin of the emulsion. The emulsifiers are complex chemically, and they come in different shapes and sizes. As new oil fields are developed and as production conditions change in older fields, there is a constant need for new, effective demulsification methods. The emulsion must be separate before the crude oil can be accepted for transportation, to meet the residual salt and water content quality criteria for a delivered crude oil.

# 1.2 Research Objectives

- Determine and evaluate the best solvents that can be used to avoid emulsification in crude oil, by testing a group of solvents on crude oil.

- Analysis the effect of each solvent on different types of crude oil, since the properties of crude oil can affect the solvents effectiveness in preventing emulsions.

- Applying a natural material to be used as a solvent to avoid emulsification in crude oil.

# 1.3 Project outline:

The present thesis work contains five chapters. In the Chapter 1 a brief introduction is presented to give a general view to the reader to know what the project will talk about; this includes the main and specific objectives of the work. The Chapter 2 involves a review of the literature in the relevant topics to this study Chapter 3 includes the prepared data, which has been use on this thesis and the methodology and procedures. Chapter 4 describes result and dissection that has been done, In Chapter 5 includes the conclusion and recommendation for the thesis.

# Chapter2

Literature Review

# 2. Literature Review

# 2.1 The origin of oil

The oldest formations of the world's oil have about 500 million years and it is a result of the slow process of nature, which produced deposits of sediment in large depressions in the bottom of seas and lakes, accumulating for thousands of years, successive layers of sedimentary rock containing microorganisms, animals and plants.  The action of heat and weight of these layers on the deeper sedimentary deposits has been transforming the organic matter through thermochemical reactions in kerogen, the initial stage of oil, then by the action of higher temperatures and pressures; they were broken, making it on deposits of oil and gas [8].

# 2.2 Reservoirs

A petroleum reservoir or oil and gas reservoir is a subsurface pool of hydrocarbons contained in porous or fractured rock formations, Petroleum reservoirs are broadly classified as conventional and unconventional reservoirs. In conventional reservoirs, the naturally occurring hydrocarbons, such as crude oil or natural gas, are trapped by overlying rock formations with lower permeability, while in unconventional reservoirs; the rocks have high porosity and low permeability, which keeps the hydrocarbons trapped in place, therefore not requiring a cap rock. Reservoirs are found using hydrocarbon exploration methods [9].

## 2.2.1 Types of reservoir fluids

Typically, there are five main types of reservoir fluids: Black oil, volatile oil, condensate (retrograde gas), wet gas, and dry gas. Each of these fluid types require different approaches when analyzing the reservoir, so it is important to identify the correct fluid type early on in the reservoir's life [10].

### 2.2.1.1 Black Oils

Black oils are made up of a variety of components including large, heavy, and non-volatile hydrocarbons. The phase diagram is shown below. When the reservoir pressure lies anywhere along line 1 → 2, the oil is said to be under saturated - meaning the oil could dissolve more gas if more gas were present. If the pressure is at 2, the oil is at its bubble point, and is said to be saturated - meaning the oil contains the maximum amount of dissolved gas and can't hold any more gas. A reduction in pressure at this point will release gas to form a free gas phase inside the reservoir. Additional gas evolves from the oil as it

moves from the reservoir to the surface. This causes some shrinkage of the oil. Black oil is often called low shrinkage crude oil or ordinary oil as shown in the Figure (2.1).

Black oils usually yield gas-oil ratios (GOR) between 200–700 scf/STB and oil gravities (API Gravity) of 15 to 40 API. The stock tank oil is usually brown to dark green in color.

Properties of black oil type of crude oil:

Oil formation volume factor less than 1.2 bbl./STB.

Gas-oil ratio less than 200 scf/STB.

Oil gravity less than 35°API.

Black or deeply colored.

Figure (2.1):  Black oil [10]

### 2.2.1.2 Volatile Oils

Volatile oils contain fewer heavy molecules and more intermediate components (ethane through hexane) than black oils. Volatile oils generally have initial gas-oil ratios in the 2000 to 3300 scf/Bbl. range, and the stock tank gravity is usually 40° API or higher. The color is generally lighter than black oil – brown, orange, or green. Gas associated with volatile oils tends to be very rich and similar to retrograde condensate gas.

The phase envelope for a volatile oil tends to cover a much narrower temperature range when compared to black oil; but like black oil, the reservoir temperature is always lower than the critical temperature for the fluid. As the reservoir temperature approaches the critical temperature a volatile oil will become more gas-like such that with even moderate depletion, a volatile oil reservoir can flash mainly to gas and have a relatively low liquid content as shown in the Figure (2.2)[10].

The characteristic properties of volatile oil include:

Oil formation volume factor less than 2 bbl./STB.

Gas-oil ratios between 2,000–3,200 scf/STB.

Oil gravities between 45–55°API.

Lower liquid recovery of separator conditions.

Greenish to orange in color.

Figure ( 2.2): Volatile oil [10]

### 2.2.1.3 Condensate (Retrograde Gas)

Condensate gas is very similar to volatile oils in terms of the color (green, orange, brown, even clear) and gravity (40° to 60° API) of the produced oil.

However, the reservoir temperature of a condensate gas reservoir is greater than the critical temperature of the fluid, and so where a volatile oil is a liquid at original reservoir pressure and temperature, a condensate gas is a gas.

As pressure is reduced in a condensate gas reservoir, the fluid will pass through the dew point and large volumes of liquid will condense in the reservoir. Since the gas flows preferentially to oil, much of this oil will be unrecoverable. Consequently, it is important to recognize that a reservoir contains a condensate gas and re-inject dry gas to maintain reservoir pressure above the dew point to maximize recovery of the liquids. In the diagram below, the retrograde gas exists completely in a gaseous state inside the reservoir at point 1. As the pressure decreases, the condensate exhibits a dew point at point 2. As the reservoir further depletes and the pressure drops, liquid condenses from the gas to form a free liquid inside the reservoir as shown in the Figure (2.3)[10].

The associated physical characteristics of Condensate:

• Gas-oil ratios between 8,000 to 70,000 scf/STB. Generally, the gas-oil ratio for a condensate system increases with time due to the liquid dropout and the loss of heavy components in the liquid.

• Condensate gravity above 50°API.

• Stock-tank liquid is usually water-white or slightly colored.

Figure ( 2.3): Condensate gas [10]

### 2.2.1.4 Wet Gas

Natural gas that contains significant heavy hydrocarbons such as propane, butane and other liquid hydrocarbons is known as wet gas or rich gas. The general rule of thumb is if the gas contains less methane (typically less than 85% methane) and more ethane, and other more complex hydrocarbons, it is labeled as wet gas. Wet gas exists solely as a gas in the reservoir throughout the reduction in reservoir pressure. Unlike retrograde condensate, no liquid is formed inside the reservoir. However, separator conditions lie within the phase envelope, causing some liquid to be formed at the surface. This surface liquid is normally called condensate, and the reservoir gas is sometimes called condensate-gas, which leads to a lot of confusion between wet gasses and retrograde condensate. The entire phase diagram of a wet gas will lie below the reservoir temperature. Note that the pressure path line does not enter the phase envelope, meaning no liquid is ever formed inside the reservoir. as shown in the Figure (2.4)[10].

Wet-gas reservoirs are characterized by the following properties:

• Gas oil ratios between 60,000 to 100,000 scf/STB

. • Stock-tank oil gravity above 60°API.

• Liquid is water-white in color.

• Separator conditions, i.e., separator pressure and temperature, lie within the                                         two-phase region.

Figure ( 2.4): Wet get [10]

### 2.2.1.5 Dry Gas

Natural gas that occurs in the absence of condensate or liquid hydrocarbons, or gas that had condensable hydrocarbons removed, is called dry gas. It is primarily methane with some intermediates. The hydrocarbon mixture is solely gas in the reservoir and there is no liquid (condensate surface liquid) formed either in the reservoir or at surface. The pressure path line does not enter into the phase envelope in the phase diagram, thus there is only dry gas in the reservoir. Note the surface separator conditions also fall outside the phase envelope (in contrast to wet gas); hence no liquid is formed at the surface separator as shown in the Figure (2.5) [10].

Figure ( 2.5 ):dry gas [10]

# 2.3 Definition emulsion:

An emulsion is a dispersion (droplets) of one liquid in another immiscible liquid. The phase that is present in the form of droplets is the dispersed or internal phase, and the phase in which the droplets are suspended is called the continuous or external phase. For produced oilfield emulsions, one of the liquids is aqueous and the other is crude oil. The amount of water that emulsifies with crude oil varies widely from facility to facility. It can be less than 1% and sometimes greater than 80%[11].

## 2.3.1 Types of Emulsions

Produced oilfield emulsions can be classified into three broad groups: water-in-oil, oil-in-water, and multiple or complex emulsions. Water-in-oil emulsions consist of water droplets in a continuous oil phase, and oil-in-water emulsions consist of oil droplets in a water-continuous phase the two basic (water-in-oil and oil-in-water) types of emulsions. In the oil industry, water-in-oil emulsions are more common (most produced oilfield emulsions are of this kind); therefore, the oil-in-water emulsions are sometimes referred to as "reverse" emulsions as shown in the figures (2.6) (2.7)[5].

Figure ( 2.7): water-in-oil emulsion [5]

Figure (2.6): oil-in-water emulsion [5]

Multiple emulsions are more complex and consist of tiny droplets suspended in bigger droplets that are suspended in a continuous phase. For example, a water-in-oil-in-water emulsion consists of water droplets suspended in larger oil droplets that, in turn, are suspended in a continuous water phase as shown in the Figure (2.8)[5].

Figure ( 2.8): water-in-oil-in-water emulsion [5]

## 2.3.2 Formation of Emulsions

Crude oil emulsions form when oil and water (brine) come into contact with each other, when there is sufficient mixing, and when an emulsifying agent or emulsifier is present. The amount of mixing and the presence of emulsifier are critical for the formation of an emulsion. During crude oil production, there are several sources of mixing, often referred to as the amount of shear, including flow through reservoir rock; bottom hole perforations/pump; flow through tubing, flow lines, and production headers; valves, fittings, and chokes; surface equipment; and gas bubbles released because of phase change. The amount of mixing depends on several factors and is difficult to avoid. In general, the greater the mixing, the smaller the droplets of water dispersed in the oil and the tighter the emulsion.

The second factor important in emulsion formation is the presence of an emulsifier. The presence, amount, and nature of the emulsifier determines, to a large extent, the type and "tightness" of an emulsion. The natural emulsifiers in a crude are resident in the heavy fraction. Because there are different types of crudes and because these crudes have different amounts of heavy components, the emulsifying tendencies vary widely. Crude with a small amount of emulsifier forms a less stable emulsion and separates relatively easily. Other crudes contain the right type and amount of emulsifier, which lead to very stable or tight emulsions[12].

## 2.3.3 Emulsion stability

The crude oil emulsions are stable because there exists an interfacial network (usually a thin and rigid film) surrounding the droplets of water, which prevents the droplets from assembling and coalescing [13]. The compounds responsible for the formation and stabilization of the crude oil emulsions are not explicitly known because the nature and the composition of crude oil vary depending on reservoir location and composition of the reservoir's rocks. Moreover, it is believed that colloidal asphalt, which includes all asphalts and similar substances present in crude oil colloidal dispersions with surfactant properties, can stabilize crude oil emulsions. The crude oil emulsions form due to the asphaltenes, and resins compounds found in the crude oil The stability of crude oil emulsion depends on the rheological properties of the emulsion (high viscosity of the interfacial film), the concentration of the asphaltenes and resins compounds as well as the oil concentration [14]. As the viscosity of the interfacial films increases, it reduces the drainage rate of the films during the 'droplets' coalescence by creating a repulsive force, which consequently lowers the rate of the emulsion droplet's breakdown [15].

# 2.4 Demulsification

Demulsification is the breaking of a crude oil emulsion into oil and water phases. From a process point of view, the oil producer is interested in three aspects of demulsification: the rate or the speed at which this separation takes place, the amount of water left in the crude oil after separation, and the quality of separated water for disposal. A fast rate of separation, a low value of residual water in the crude oil, and a low value of oil in the disposal water are obviously desirable. Produced oil generally has to meet company and pipeline specifications. For example, the oil shipped from wet-crude handling facilities must not contain more than 0.2% BS&W and 10 pounds of salt per thousand barrels of crude oil. This standard depends on company and pipeline specifications. The salt is insoluble in oil and associated with residual water in the treated crude. Low BS&W and salt content is required to reduce corrosion and deposition of salts. The primary concern in refineries is to remove inorganic salts from the crude oil before they cause corrosion or other detrimental effects in refinery equipment. The salts are removed by washing or desalting the crude oil with relatively fresh water [16].

## 2.4.1 Chemical Demulsification

Chemical demulsification consists in adding chemicals called demulsifiers to accelerate the coalescence of the dispersed phase of the emulsion[17][18], in addition, these chemicals are designed to neutralize the effect of surfactants that stabilized the emulsion, and displace them from the interfacial film surrounding the emulsion droplets to enhance destabilization. Effective chemical demulsification requires careful selection of demulsifiers for a given emulsion (emulsion characteristics vary from reservoir to reservoir), an appropriate amount of demulsifiers, proper mixing, and sufficient retention time to allow the phases to separate.

## 2.4.2 Factors affecting emulsion chemical demulsification

To obtain the desired demulsification efficiency, some factors that affect the emulsion chemical demulsification have to be optimized. The demulsification process is governed by Stake's law [19]. The demulsifies act directly on the film separating oil and water phases, so as to disrupt the interfacial tension between the droplets, and hence increase thermal velocity and speed up the coalescence. The process of demulsification is affected by the temperature, phase volume ratio, residence time, emulsifier dosage, crude oil properties, and aging of emulsions [20].

# Chapter 3

Methodology & Procedures

# 3. Experimental Apparatus and Materials

## 3.1 Apparatus

### 3.1.1 Centrifuge

It is used to separate liquids or suspended solids within petroleum samples using centrifugal force. The device rotates at high speed, creating a centrifugal force that pushes the heavier components (such as water and sediment) down and keeps the lighter components (oil) on top as shown in the Figure (3.1).

Figure ( 3.1): Centrifuge

### 3.1.2 Test tube

The tube capacity is 100 ml, and it was used to place the samples inside the centrifuge as shown in the Figure (3.2).

Figure ( 3.2 ):Test tube

### 3.1.3 Micropipette

The pipette is used to draw precise amounts of materials to prepare different concentrations. The pipette capacity ranges from 200 microliters to 1000 microliters as shown in the Figure (3.3).

Figure (3.3): Micropipette

### 3.1.4 Analytical Balance

A balance designed for highly precise measurements in laboratories, especially when preparing solutions or weighing very small chemicals as shown in the Figure (3.4).

Figure (3.4): Analytical balance

### 3.1.5 Hydrometer

Figure ( 3.5):  Hydrometer

A hydrometer is a laboratory instrument used to measure the specific gravity of liquids as shown in the Figure (3.5).

3.1.6 Hydrometer Cylinder

Plastic, 700 ml cylinder for holding liquids during density or API gravity tests. Transparent and chemical-resistant as shown in the Figure (3.6).

Figure ( 3.6):  Hydrometer Cylinder

## 3.2 Materials

### 3.2.1 Demulsifiers

Two types of demulsifiers were used in this study, demulsifiers are chemical agents used to break water-in-oil emulsions by separating the water phase from the oil.

#### 3.2.1.1 Demo AS

Figure ( 3.7): Demo AS

#### 3.2.1.2 Demo AM

Figure ( 3.8 ):Demo AM

### 3.2.2 Solvents

In this research, two organic solvents were used to break the water-oil emulsion and facilitate the separation process, as follows:

#### 3.2.2.1 Hexane

Hexane is a colourless organic solvent belonging to the alkanes group.

Molecular formula: C₆H₁₄

Boiling point: approximately 69°C

Polarity: Nonpolar. as shown in the Figure (3.9).

Figure ( 3.9): Hexane

3.2.2.2 Xylene

Xylene is a clear, aromatic organic solvent belonging to the aromatic hydrocarbon group.

Molecular formula: C₈H₁₀

Boiling point: 138–144°C

Polarity: Semi polar. as shown in the Figure (3.10).

Figure ( 3.10): Xylene

### 3.2.3 polymer

#### 3.2.3.1 Synthesis of Cellulose/Poly (AAc-co-AAm) Ag NPs

one-pot method was used to synthesize the nano silver-containing biodegradable superabsorbent polymer/cellulose composite. An amount of 7.11 g acrylamide was dissolved in 50 ml distilled water and mixed in a three necked round bottom flask and heated to 40 °C under nitrogen protection in a water bath. The solution is then mixed for 20 minutes, then 2.5 ml acrylic acid was added to the solution. 0.05 g of cellulose was added into the flask while stirring, followed by 0.05 g of MBA as crosslinking agent. An amount of 0.1 g KPS as initiator dissolved in 10 ml distilled water, was slowly added into the flask to initiate the polymerization process. Silver nitrate solutions of different dosages (5, 10 and 15 ml) and glucose (5,10, and 15ml) were added into the system and mixed at 65 C° for 1 h to reduce Ag+ into silver particles. The solution was then heated at 80 C° while mixing for 2 hours. The polymers thus formed are composites of biodegradable cross-linked cellulose poly (AAc-co-AAm)/silver nanoparticles (SAP/AgNPs). The obtained SAP composites were dewatered using ethanol and then poured into a petri dish and dried in the oven for 24 hrs at 60 C°. Finally, the dry samples were crushed into powder and sifted for subsequent experiments and tests.

Figure ( 3.11 ):polymer  A

Figure ( 3.12):  polymer B

### 3.2.4 Natural material

MLX is a naturally derived oil-based compound, extracted from natural sources using environmentally friendly methods. It was used in this research as a topical agent within the experimental framework. as shown in the Figure (3.13).

Figure ( 3.13):  Natural marerial (MLX)

### 3.2.5 Crude oil

Two types of crude oil were used in this study, as shown in the Figure (3.14).

The first crude oil (S.F) had an API value of 30.5° API.

The second crude oil (M.F) had an API value of 43.5° API.

Figure ( 3.14): Two samples of crude oil used in this study.

## 3.3 Experiment Procedure:

In this experiment, two different types of crude oil M.F. and S.F. were used containing relatively stable water-in-oil emulsions, the density and temperature of each were measured.

Table (3.1) Types of crude oil, their densities (API) and temperature

| Type Oil | API | Temperature (F) |
| --- | --- | --- |
| M.F | 43.5 | 113 |
| S. F | 30.5 | 99 |

A centrifuge was used for oil testing, using 100ml tubes from the device the tubes were filled with 100ml of each oil and placed in the device the speed was set at 1500 rpm, the time was set at 10 minutes, and the temperature was set at 60°C.

Using the device, the three layers were determined the percentage of free water, the percentage of emulsion layer, and the crude oil as shown in the Figure (3.15).

Figure (3.15 ):Crude oil samples

This was used to determine the separation rate and the extent of emulsion layer removal after adding different proportions of materials.

### 3.3.1 Procedure of polymer testing

Two types of polymers were prepared, Polymer A and Polymer B. Two concentrations of 0.5% and 1% were prepared in a 50 ml tube. They were dissolved by adding distilled water. Three concentrations of 0.25%, 0.5%, and 1% were taken from a 100 ml tube, according to the specified concentration, and added to each oil separately.

Table (3.2) Concentration of polymers A and B

| Polymer (A) | 0.5% | 0.25g | 1% | 0.5g |
| --- | --- | --- | --- | --- |
| Polymer (B) | 0.5% | 0.25g | 1% | 0.5g |

### 3.3.2 Procedure of Solvent application

Xylene and hexane solvents were used, and three concentrations of 0.25%, 0.5%, and 1% were taken from each solvent.

### 3.3.3 Testing of natural material

A natural substance (MLX) was used, and three concentrations were taken from it: 0.25%, 0.5%, and 1%.

### 3.3.4 Demulsifiers evalution method

Two different concentrations of commercial demulsifiers were also taken: 0.25% and 0.5%.

These substances were used at these different concentrations on each type of oil using a centrifuge at 1500 rpm for 10 minutes and a temperature of 60°C.

These conditions were used in each experiment according to international standards.

The separation rate and the extent of emulsion layer removal were monitored for each concentration of the aforementioned substances, and the results were recorded.

# Chapter 4

Result and Discussion

# 4. Results and Discussion

## 4.1 Basic crude oil samples:

Determining the proportion of free water and emulsion layers in crude oil samples (M.F and S.F) in order to evaluate the effectiveness of the applied solvents and materials in separating water and breaking the emulsion layer as shown in the Figure (4.1).

Table (4.1) Basic crude oil samples

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| Before addition | M.F | 0.05 | 5.95 | 6 |
| Before addition | S.F | 0.01 | 14.99 | 15 |

S.F

Figure ( 4.1):  Basic crude oil samples

## 4.2. Effect of Demo (AS and AM)

Demulsifiers AM and AS were tested on crude oil at concentrations of 0.25%, 0.5% to evaluate their effectiveness in breaking emulsions.

### 4.2.1 Demo (AS and AM) - Concentration 0.25%

We observed the effect of adding demulsifiers Demo (AS) and Demo (AM) at a concentration of 0.25% to two oil samples, M.F and S.F. 6 ml of water was separated in the M.F sample using both types, without any emulsion remaining, indicating a high efficiency in breaking the emulsion. As for the S.F sample, a smaller amount of water was separated, as Demo (AS) separated 3.5 ml, while Demo (AM) separated only 1.5 ml, with a large amount of emulsion remaining. This means that the emulsion in the S.F sample is more stable and more difficult to break, as shown in the Figure (4.2).

Table (2.2): Demo (AS and AM) - Concentration 0.25%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Demo (AS) | M.F | 6 | 0 | 6 |
| After addition Demo (AS) | S.F | 3.5 | 11.5 | 15 |
| After addition Demo (AM) | M.F | 6 | 0 | 6 |
| After addition Demo (AM) | S.F | 1.5 | 13.5 | 15 |

Figure ( 4.2 ):Demo(AS and AM) - Concentration 0.25%

### 4.2.2 Demo (AS and AM) - Concentration 0. 5%

We note the effect of adding the demulsifiers Demo (AS) and Demo (AM) at a concentration of 0.5% to the same two samples, M.F and S.F. The results showed an improvement in water separation, especially in the S.F sample compared to the previous concentration. In the M.F sample, the entire amount of water, amounting to 6 ml, was separated when using both types, without any emulsion remaining, confirming the continued effectiveness of the demulsifier. As for the S.F sample, performance improved when using Demo (AS), as 8 ml of water was separated, leaving 7 ml of emulsion, while Demo (AM) separated a smaller amount, amounting to 5 ml, leaving 10 ml of emulsion. This indicates that the emulsion in the S.F sample is still more stable, but less stable than before, thanks to the increased concentration of the demulsifier. As shown in the Figure (4.3)

Table (4.3): Demo(AS and AM) - Concentration 0. 5%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Demo (AS) | M.F | 6 | 0 | 6 |
| After addition Demo (AS) | S.F | 8 | 7 | 15 |
| After addition Demo (AM) | M.F | 6 | 0 | 6 |
| After addition Demo (AM) | S.F | 5 | 10 | 15 |

Figure ( 4.3 ):Demo(AS and AM) - Concentration 0. 5%

## 4.3 Organic Solvents (Xylene and Hexane)

Solvents xylene and hexane were tested on crude oil at concentrations of 0.25%, 0.5%, and 1% to evaluate their effectiveness in breaking emulsions.

### 4.3.1 Xylene and Hexane Solvent- Concentration 0.25%

We note the effect of adding the solvents xylene and hexane at a concentration of 0.25% to two oil samples, M.F and S.F. The solvents achieved lower separation efficiency compared to the emulsion-breaking method. In the M.F sample, xylene separated 5 ml of water, leaving 1 ml of emulsion, while hexane separated a smaller amount of water (4.5 ml) and left 1.5 ml of emulsion. In the S.F sample, weaker results were achieved, as xylene separated only 3 ml of total water compared to 12 ml of emulsion. Hexane was less efficient, separating only 2.8 ml of water compared to 13.2 ml of emulsion, indicating that the effectiveness of solvents in breaking emulsions is limited, especially with the S.F sample, as shown in the Figure (4.4).

Table (4.4) Xylene and Hexane Solvent- Concentration 0.25%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Xylene | M.F | 5 | 1 | 6 |
| After addition Xylene | S.F | 3 | 12 | 15 |
| After addition Hexane | M.F | 4.5 | 1.5 | 6 |
| After addition Hexane | S.F | 2.8 | 13.2 | 15 |

Xylene

Hexane

Figure ( 4.4): Xylene and Hexane Solvent- Concentration 0.25%

### 4.3.2 Xylene and Hexane Solvent- Concentration 0. 5%

We note the effect of adding the solvents xylene and hexane at a concentration of 0.5% to the same two samples, M.F and S.F. The results showed an improvement in water separation, especially in the M.F sample compared to the other samples.

In the M.F sample, when xylene was used, almost all of the water (5.8 ml) was separated, with only a very small amount of emulsion (0.2 ml) remaining, demonstrating the efficiency of xylene in breaking the emulsion. When hexane was used, 5 ml of water was separated, leaving 1 ml of emulsion, indicating that xylene was more effective.

In the S.F sample, the solvents did not show the same high efficiency, as a large amount of emulsion remained with either xylene or hexane. 4 ml of water and 11 ml of emulsion were separated with  xylene, while hexane separated a smaller amount of water (3.5 ml) with 11.5 ml of emulsion remaining, indicating that the emulsion in this sample is more stable and difficult to break. as shown in the Figure (4.5).

Table (4.5): Xylene and Hexane Solvent- Concentration 0. 5%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Xylene | M.F | 5.8 | 0.2 | 6 |
| After addition Xylene | S.F | 4 | 11 | 15 |
| After addition Hexane | M.F | 5 | 1 | 6 |
| After addition Hexane | S.F | 3.5 | 11.5 | 15 |

Xylene

Hexane

Figure ( 4.5 ):Xylene and Hexane Solvent- Concentration 0. 5%

### 4.3.3Xylene and Hexane Solvent- Concentration 1%

At a 1% concentration, xylene demonstrated good emulsion breaking efficiency, especially in the M.F sample, where water was completely separated without any remaining emulsion, a performance similar to or better than the previous concentration.

In the S.F sample, a significant portion of the emulsion remained after adding xylene, but its performance was relatively better than that of hexane.

Hexane at a 1% concentration demonstrated acceptable efficiency in the M.F sample but was significantly less effective in the S.F sample, where the amount of remaining emulsion increased compared to the previous concentration, indicating that the increased hexane concentration contributed to emulsion formation and stability rather than breaking it as shown in the Figure (4.6).

Table (4.6): Xylene and Hexane Solvent- Concentration 1%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Xylene | M.F | 6 | 0 | 6 |
| After addition Xylene | S.F | 4.5 | 10.5 | 15 |
| After addition Hexane | M.F | 5.5 | 0.5 | 6 |
| After addition Hexane | S.F | 1.8 | 13.2 | 15 |

Hexane

Xylene

Figure (4.6): Xylene and Hexane Solvent- Concentration 1%

## 4.4 Effect of Polymers A and B concentrations 0.5%

Polymer A and Polymer B were tested at a concentration of 0.5%, and three concentrations of each polymer (0.25%, 0.5%, and 1%) were taken to determine their effectiveness in breaking and separating emulsions.

### 4.4.1 Polymer A and B - Concentration 0.25%

We observed the effect of adding Polymer A and Polymer B at a concentration of 0.25% (taken from a 0.5% solution) to the same two samples, M.F. and S.F. The results showed an improvement in water separation compared to the condition before addition, especially in the M.F. sample.

In the M.F. sample, when Polymer A was used, 1 ml of water was separated, leaving 5 ml of emulsion. When Polymer B was used, the amount of separated water increased to 2.2 ml, and the emulsion decreased to 3.8 ml, indicating that Polymer B is more effective at separating water and reducing the amount of emulsion in this sample.

In the S.F sample, the amount of separated water was relatively small, as only 0.5 ml was separated when using Polymer A, and 0.6 ml when using Polymer B, with a large proportion of the emulsion remaining in both cases (14.5 ml and 14.4 ml, respectively), indicating that the emulsion in the S.F sample is more stable and more difficult to separate than in the M.F sample, despite the slight improvement resulting from the use of polymers, as shown in the Figure(4.7).

Table (4.7): Polymer A and B - Concentration 0.25%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Polymer A | M.F | 1 | 5 | 6 |
| After addition Polymer A | S.F | 0.5 | 14.5 | 15 |
| After addition Polymer B | M.F | 2.2 | 3.8 | 6 |
| After addition Polymer B | S.F | 0.6 | 14.4 | 15 |

Polymer A

Polymer B

Figure ( 4.7):  Polymer A and B - Concentration 0.25%

### 4.4.2 Polymer A and B - Concentration 0.5%

Based on Table (4.8), which shows the effect of adding Polymer A and Polymer B at a concentration of 0.5% to the same two samples, M.F and S.F, we observe a significant improvement in water separation, particularly in the M.F sample compared to the lower concentration (0.25%)  as shown in the Figure (4.8).

In the M.F sample, Polymer A demonstrated good efficiency, separating 1.6 ml of water while leaving 4.4 ml of emulsion. Polymer B's efficiency increased even further, separating 3 ml of water while leaving only 3 ml of emulsion. This indicates that Polymer B is more efficient at water separation at this concentration, achieving a clear balance between water and emulsion.

In the S.F sample, the emulsion remained relatively stable, with the results not changing much compared to the previous concentration. 0.6 ml of water was separated when using Polymer, A and 1 ml when using Polymer B, while the emulsion amounts remained high at 14.4 ml and 14 ml, respectively, indicating that the S.F sample is still more resistant to separation despite the increased concentration, but there is a slight improvement when using Polymer B.

Table (4.8): Polymer A and B - Concentration 0.5%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Polymer A | M.F | 1.6 | 4.4 | 6 |
| After addition Polymer A | S.F | 0.6 | 14.4 | 15 |
| After addition Polymer B | M.F | 3 | 3 | 6 |
| After addition Polymer B | S.F | 1 | 14 | 15 |

Figure (4.8): Polymer A and B - Concentration 0.5%

### 4.4.3 Polymer A and B - Concentration 1%

Based on Table (4.9), it is noted that increasing the concentration of Polymer A and B to 1% resulted in a clear improvement in water separation compared to the 0.5% concentration. In the M.F sample, the amount of separated water increased after adding Polymer A to 3 ml compared to 1.6 ml previously, while Polymer B showed better performance by separating 4 ml compared to 3 ml previously. As for the S.F sample, the performance of Polymer A remained almost constant, separating only 1 ml, while the performance of Polymer B improved by separating 2.2 ml compared to 1 ml at the previous concentration. This indicates that increasing the concentration enhanced the separation efficiency, especially in the M.F sample, with Polymer B continuing to outperform Polymer A, as shown in the Figure (4.9).

Table (4.9): Polymer A and B - Concentration 1%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Polymer A | M.F | 3 | 3 | 6 |
| After addition Polymer A | S.F | 1 | 14 | 15 |
| After addition Polymer B | M.F | 4 | 2 | 6 |
| After addition Polymer B | S.F | 2.2 | 12.8 | 15 |

Figure ( 4.9):  Polymer A and B - Concentration 1%

## 4.5. Polymer A and B - Concentration 1 %

Polymer A and Polymer B were tested at a concentration of 1%, where three different concentrations of each polymer (0.25%, 0.5%, and 1%) were taken in order to evaluate their effectiveness in breaking and separating emulsions.

### 4.5.1 Polymer A and B - Concentration 0.25%

We note the effect of adding Polymer A and Polymer B at a concentration of 0.25% (taken from a 1% base solution) to the M.F and S.F samples, as the results showed a clear improvement in water separation compared to the baseline values ​​before addition. In the M.F sample, the amount of water increased to 0.9 and 1.6 ml when using Polymer, A and B, respectively, with a decrease in the emulsion, indicating the effectiveness of both polymers in breaking the emulsion, especially Polymer B. As for the S.F sample, 0.5 and 0.6 ml of water were separated when using A and B, respectively, while the amount of emulsion remained high, indicating that the effect of the two polymers in S.F is less compared to M.F, but it indicates the beginning of relative stability, as shown in the Figure(4.10).

Table (4.10):  Polymer A and B - Concentration 0.25%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Polymer A | M.F | 0.9 | 5.1 | 6 |
| After addition Polymer A | S.F | 0.5 | 14.5 | 15 |
| After addition Polymer B | M.F | 1.6 | 4.4 | 6 |
| After addition Polymer B | S.F | 0.6 | 14.4 | 15 |

Figure (4.10):  Polymer A and B - Concentration 0.25%

### 4.5.2 Polymer A and B - Concentration 0. 5%

We observed the effect of adding Polymer A and Polymer B at a concentration of 0.5% to the M.F and S.F samples, achieving better water separation results compared to the previous concentration of 0.25%.

In the M.F sample, 1.5 ml of water was separated when Polymer A was used, while results improved further with Polymer B, which separated 2.6 ml of water, demonstrating its efficiency in breaking emulsions and reducing the amount of water to be emulsified (only 3.4 ml).

In the S.F sample, 0.6 ml of water was separated with Polymer A and 1.2 ml with Polymer B. However, a significant amount of emulsion remained (14.4 ml and 13.8 ml, respectively). We concluded that the S.F sample remained more stable, but greater success was achieved when Polymer B was used. as shown in the Figure (4.11).

Table (4.11): Polymer A and B - Concentration 0. 5%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Polymer A | M.F | 1.5 | 4.5 | 6 |
| After addition Polymer A | S.F | 0.6 | 14.4 | 15 |
| After addition Polymer B | M.F | 2.6 | 3.4 | 6 |
| After addition Polymer B | S.F | 1.2 | 13.8 | 15 |

Figure (4.11): Polymer A and B - Concentration 0. 5%

### 4.5.3 Polymer A and B - Concentration 1%

We note the effect of adding Polymer A and Polymer B at a concentration of 1% to the same two samples, M.F and S.F. The results showed an improvement in water separation, especially in the M.F sample compared to the previous concentration.

The total amount of water separated in the M.F sample, 3.5 ml, was achieved when Polymer B was used, compared to only 2.6 ml when Polymer A was used, confirming that Polymer B is more efficient at breaking the emulsion in this sample.

As for the S.F sample, the results also improved, with 2 ml of water separated with Polymer B, compared to only 0.8 ml with Polymer A, leaving 13 to 14.2 ml of emulsion, indicating that the S.F sample remains more stable. as shown in the Figure (4.12)

Table (4.12): Polymer A and B - Concentration 1%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition Polymer A | M.F | 2.6 | 3.4 | 6 |
| After addition Polymer A | S.F | 0.8 | 14.2 | 15 |
| After addition Polymer B | M.F | 3.5 | 2.5 | 6 |
| After addition Polymer B | S.F | 2 | 13 | 15 |

Figure (4.12): Polymer A and B - Concentration 1%

## 4.6 Nature material (MLX)

### 4.6.1. Basic crude oil samples

Crude oil samples before the addition of MLX, showing the measured free water and emulsion layers after centrifugation, as shown in the Figure (4.13).

Table (4.13): Basic crude oil samples

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| Before addition | M.F | 0.05 | 5.95 | 6 |
| Before addition | S.F | 1.1 | 23.9 | 25 |

M.F

M.F

S.F

S.F

Figure (4.13):  Basic crude oil samples (MLX)

### 4.6.2 Nature material (MLX) at Concentration 0.25%

The effect of MLX at a concentration of 0.25% on water separation was observed in both samples, M.F and S.F, where the results showed a noticeable improvement in water separation.

In sample M.F, the amount of separated water increased from 0.05 mL to 1.5 mL, and the emulsion layer decreased from 5.95 mL to 4.5 mL, indicating the material’s effectiveness in reducing emulsification.

In sample S.F, the water content increased to 4 mL, with a slight reduction in the emulsion from 23.9 mL to 21 mL. This suggests a weaker effect compared to M.F, but still shows an improvement in breaking the emulsion, as shown in the Figure (4.14).

Table (4.14): Nature material (MLX) at Concentration 0. 25%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition | M.F | 1.5 | 4.5 | 6 |
| After addition | S.F | 4 | 21 | 25 |

Figure (4.14): (MLX) at Concentration 0.25%

### 4.6.3. Nature material (MLX) at Concentration 0. 5%

It is observed that increasing the concentration of MLX to 0.5% enhanced the efficiency of water separation and reduced the emulsion layer in both M.F and S.F samples compared to the 0.25% concentration. In the M.F sample, the amount of separated water increased from 1.5 mL to 3 mL, while the emulsion layer decreased from 4.5 mL to 3 mL, indicating a significant improvement in demulsification. Similarly, in the S.F sample, the water content increased from 4 mL to 6.8 mL, and the emulsion decreased from 21 mL to 16.2 mL, confirming the higher effectiveness of MLX at this concentration, as shown in the Figure (4.15).

Table (4.15): Nature material (MLX) at Concentration 0. 5%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition | M.F | 3 | 3 | 6 |
| After addition | S.F | 6.8 | 16.2 | 23 |

Figure (4.15): (MLX) at Concentration 0. 5%

### 4.6.4. Nature material (MLX) at Concentration 1%

It is observed that increasing the concentration of MLX to 1% significantly improved the efficiency of water separation and reduced the emulsion layer compared to the previous concentration of 0.5%.

In the M.F sample, the amount of separated water increased from 3 mL to 4.2 mL, and the emulsion decreased from 3 mL to 1.8 mL, indicating higher effectiveness in breaking the emulsion and enhancing phase separation.

In the S.F sample, the water volume rose from 6.8 mL to 10 mL, and the emulsion decreased from 16.2 mL to 13 mL, confirming that increasing the MLX concentration led to better demulsification performance, as shown in the Figure (4.16).

Table (4.16): Nature material (MLX) at Concentration 1%

| Case | Sample | Water | Emulsion | Total |
| --- | --- | --- | --- | --- |
| After addition | M.F | 4.2 | 1.8 | 6 |
| After addition | S.F | 10 | 13 | 23 |

Figure (4.16):  (MLX) at Concentration 1%

# Chapter 5

conclusion and recommendation

# 5.1 Conclusion

1- In the M.F sample, the entire amount of water, amounting to 6 ml, was separated when using both types, without any emulsion remaining, confirming the continued effectiveness of the demulsifier. As for the S.F sample, performance improved when using Demo (AS), as 8 ml of water was separated, leaving 7 ml of emulsion, while Demo (AM) separated a smaller amount, amounting to 5 ml, leaving 10 ml of emulsion.

2- When Polymer B was used, the amount of separated water increased to 2.2 ml, and the emulsion decreased to 3.8 ml, indicating that Polymer B is more effective at separating water and reducing the amount of emulsion in this sample.

3- We concluded that the S.F sample remained more stable, but greater success was achieved when Polymer B was used.

4- that increasing the concentration of MLX to 0.5% enhanced the efficiency of water separation and reduced the emulsion layer in both M.F and S.F samples compared to the 0.25% concentration.

5- the concentration of MLX to 1% significantly improved the efficiency of water separation and reduced the emulsion layer compared to the previous concentration of 0.5%.

6- that increasing the MLX concentration led to better demulsification performance.

# 5.2 Recommendation

1- Using different types of organic solvents.

2- To apply the same experimental on different types of crude oil.

3- Standard centrifugal device in oil industry must be used for   this    experimental.