State of Libya

Ministry of Education and Scientific Research

Sebha University

Faculty of Engineering

Department of Renewable Energy Engineering

This research was submitted in fulfillment of the requirements for a bachelor's degree at the Department of Renewable Energy Engineering, Faculty of Engineering, Sebha University, under the title:

Design of solar LED street light for Sebha city

By student:

Shareefa Al-Ghali Hajri

Under supervision:

Dr. Mohamad Hossin

Academy Year

2024-2025

Declaration

Student Declaration:

I, Shareefa Al-Ghali Hajri, student ID number 22170066, hereby declare that the content of this research is the product of my own efforts, with the exception of those paragraphs attributed to scholarly sources.

Date: ......................................................... Signature: ..............................................................

Supervisor Approval:

Supervisor Name: Dr. Mohamad Hossin

I hereby confirm that I have reviewed the material in this research and that it is ready for discussion.

Date: ......................................................... Signature: .............................................................

Dedication

To all who contributed to the enrichment of this academic research, from my esteemed professors who generously shared their knowledge and guidance, to my dear colleagues who shared my passion for knowledge and exchanged ideas with me, and to my honorable family who supported and assisted me throughout this journey. This work is the fruit of all your efforts, and I hope it will be a valuable addition to the field of this research study.

Acknowledgment

In the name of Allah, the Most Gracious, the Most Merciful, and prayers and peace be upon the noblest of prophets and messengers, our Prophet Muhammad, and upon his family and companions, all of them.

Praise be to Allah, by whose grace good deeds are completed, and by whose favor goals are achieved.

Today, I stand grateful and thankful to Allah Almighty for His guidance and assistance in completing this research. I extend my sincere thanks and appreciation to the one who was a guiding light and beacon for me, the esteemed Dr. Mohammed Abdullah Hussein, who generously provided me with his sound guidance and valuable advice, and was a support and assistance to me in every step. May Allah reward him with the best of rewards on my behalf.

I would also like to express my gratitude and appreciation to everyone who contributed to the success of this work, including professors, colleagues, and friends, who provided me with support and encouragement. Thanks are also due to my honorable family, who were the best support and assistance to me.

I ask Allah to make this work purely for His noble sake, and that it may benefit researchers and those interested in the field.

Student: Shareefa Al-Ghali Hajri.

Table of Contents

CHAPTER 1: INTRODUCTION…………………………………………………………..	1

1.1 Introduction:	2

1.2 Objective of the project:	3

1.3 Research Methodology:	3

1.4 Scope of the research:	4

1.5 Project outline:	5

CHAPTER 2: Literature Review and Solar Street Light Components	6

2.1 Literature Review:	7

2.2 Solar street light components:	7

2.2.1SolarPanels…..……………………………………………………………………8

2.2.1.1 Working principle:………………………………………………………….…………..………….11  2.2.2:  Battery………………………………………………………..…………..…….12

2.2.2.1 Types of batteries used in solar energy systems:.. …………....…………… ………13

2.2.2.2. The structure and operation of Lithium-ion battery:	……………………..13

2.2.3 LED Lamp:………………………….…………………………………………………………………………14

2.2.4 Charge Controller:…..………………………………………..………..………………………………….15

2.2.5 Light Sensor:………………………………………………………………………………………..	……..16

2.2.6 Street Light Pole:………………………………………………….…………………………………………16

2.3 Operation of a solar street lighting system:	17

CHAPTER 3: Existing Sebha Solar street light	18

3.1 Determine the location that was designed:	19

3.2 selecting existing design components:	20

CHAPTER 4: Design of Street Lights	22

4.1 Location of proposed street lights	23

4.2 Study solar irradiance and climate:	24

4.3 Simulate the desired light level using the DIALux application:	24

4.4 Calculation of energy and system sizing and selection of light components	25

4.5 Results and Discussion:	30

4.5.1 Selecting a solar panel:.…………………………………………………………………………..………32

4.5.2 Economic Analysis:..……………………………………………………………………………….……..33

CHAPTER 5: Conclusion	34

5.1 Conclusion:	35

5.2 Difficulties and obstacles:	36

5.3 Usefulness of the research:	36

5.4 Recommendations:	37

References	39

Appendixes	43

List of figures

Page

Figure 2.1: solar LED street light components………………..………..………….…...…..8

Figure 2.2: Types of solar panels……………..………………………..……………...…..11

Figure 2.3: working mechanism of a solar cell.……………………………………...........12

Figure 2.4: A) Lithium-ion battery during discharge. B) Formation of passivation layer (solid-electrolyte interphase, or SEI) on the negative electrode………………………......14

Figure 2.5: (a) series charge controller, (b) shunt charge controller…………..…………..15

Figure 2.6: Diagram of the operation of a solar street lighting system…..…………..……17

Figure 3.1: lighting line installation…………………...………………………………......20

Figure 3.2: Pole of street lighting …….………….…………………………………….….21

Figure 4.1: Location of proposed street lights….…………………...………………..……24

Figure 4.2: Simulate the desired light level….…………...…..……………………..……..26

List of tables

Page

Table 3.1: Existing design specification. …………………………………………….……22

Table 4.1: Technical data related to the lamp………………………………………25

Table 4.2: The average daylight hours in Sebha City. ………………………..……...….. 27

Table 4.3: Summary of each light pole. …………………………………………...…….. 31

Table 4.4: Solar panel specification……………………………………...……… 33

Table 4.5: Characteristics of the two batteries.. ………………………………...…...……34

Abstract

This research presents the design and simulation of a street lighting system with LED solar lamps for the city of Sabha in Libya. The system aims to provide reliable and energy-saving lighting in areas affected by power outages and insufficient network coverage. The proposed solution integrates photovoltaic panels, batteries, charge controller, LEDs and light sensors to ensure independent night operation.

A complete technical design has been developed, including calculations of power consumption, battery size, plate selection, and charging controller classification. Lighting layout using DIALux software has been simulated to ensure proper lighting and standardization based on international lighting standards. A comparative analysis of gel battery systems and lithium-ion batteries was also conducted to assess long-term performance, reliability and value.

An economic analysis showed that the system recovery period is approximately 16.4 years and achieves net savings over 20 years of shelf life. These results confirm the feasibility of implementing solar street lighting as a sustainable, cost-effective and environmentally friendly solution in Sabha and similar areas.

CHAPTER 1

INTRODUCTION

1.1 Introduction:

Today, Energy-efficient lighting is an important factor for sustainable development and energy strategies. Indeed, lighting consumes about 20 percent of the electricity for a nation. Also, renewable energy utilization development permits the reduction of CO2 emissions and contributes to the decrease of fossil energy dependency. In the past few years, developing energy-efficient street lighting with light-emitting diodes (LEDs) has gained enormous interest. LED luminaires also have the potential to increase illumination uniformity and glare reduction, which improves both the eye comfort and the visual discrimination ability of car drivers.

Traditional street lighting technologies, such as high-pressure sodium or mercury, emit light in all directions, and consequently, the light distribution is difficult to control. This is why a common street luminaire usually has defects such as glare, non-uniform light pattern, upward reflected light, light pollution, and waste of energy. The association of solar energy with high-efficiency lighting technology as LEDs (Light Emitting Diodes), contributes to the development of clean energy (Solar) and green lighting technology (LEDs). In comparing LEDs to other lamp technologies, we can say that LEDs are the Greenest lighting choice. High power LEDs (Light Emitting Diode) devices permit the design and fabrication of street lighting units in order to replace existing luminaries which are using sodium or metal halide or CFL (Compact Fluorescent Lamp) lamps.

A simple LED light bulb that can fits standard E40 bulb holders can be applied for solar powered street light system which is totally independent of power mains. The high power LEDs of last generation and technology offer a considerable alternative to a conventional street lighting with energy savings of up to 75 % and an important reduction in carbon emissions. The photometric properties of high power LED street lights adding to their bright, natural light color, give a uniform rectangular beam pattern that is 50% brighter and50 % larger than the oval beam pattern produced by a conventional lamp. This highly focused beam pattern allows LED lights to be spaced at much wider intervals than sodium and CFL lights. Competitors to conventional lead-acid batteries include nickel-cadmium, nickel-metal hydride, and lithium-ion, lithium-polymer, and nickel-zinc technologies. [1]

1.2 Objective of the Project:

The primary goal is to create an effective and dependable solar street lighting system by substituting LED lights with sodium ones.

Specific Objectives:

- Reduce energy consumption and operational costs.

- Utilize renewable energy to decrease carbon footprint.

- Assure longevity and little maintenance.

1.3 Research Methodology:

This study adopts a design-based engineering methodology focused on the real-life application in the city of Sebha, and this process followed several main steps:

- Selection of components Selection of system elements (solar panel, battery, controller, lamp, lamppost, lane and sensor) Based on efficiency, local availability and compatibility.

- Site selection Selecting the location where the design will be implemented and collecting solar radiation data and climatological information for the city of Sebha.

- Mathematical calculations, calculate power consumption, battery capacity and panel size based on lighting requirements and hours of operation.

- Simulation and modeling using comparative analysis software: Optimal light distribution, light levels, pole spacing, and lamp power.

- Comparative Analysis Evaluate different battery types (gel vs. lithium ion) in terms of performance, efficiency, life-cycle cost, and temperature tolerance.

- Verification and validation comparing simulation results with standard lighting requirements technical feasibility.

- Economic feasibility cost analysis to assess the feasibility and sustainability of the system for public infrastructure in Sebha city.

1.4 Scope of the project:

This project is limited to the design and simulation of a solar-powered LED street lighting system for selected areas in the city of Sebha, Libya. The focus is on:

- Designing an independent off-grid solar system.

- Covering specific streets such as Al-Qarda Street and the university area.

- The duration of the lighting operation is about 12 hours per night.

- Selection and simulation of standardized components under real environmental conditions.

- Addressing local energy challenges related to power outages.

- Evaluation of two battery technologies (gel and lithium-ion) for past and present.

It does not include implementation or actual testing of prototypes. It focuses on engineering design, simulation, and economic analysis only.

1.5 Project Outline:

This research is divided into five main chapters:

Chapter 1: Introduction, objectives, methodology, and scope

Chapter 2: Literature review and components

Reviews relevant studies and explains the components of the system (solar panel, battery, controller, LED, sensor, and poles).

Chapter 3: Existing system in Sabha

Reviews the actual application of solar lighting in Sebha, highlighting specifications, components, and impact on safety and sustainability.

Chapter 4: Proposed Design and Simulation

Covers detailed design calculations, simulation of the DIALux system, system sizing, and comparison between gel and lithium-ion batteries.

Chapter 5: Results, Conclusion, and Recommendations

Presents the final conclusions, summarizes the results and economic comparison, discusses the difficulties, and proposes recommendations.

CHAPTER 2

Literature Review and Solar Street Light Components

2.1 Literature Review:

A comparative analysis of the economics of street lighting powered by public electricity utility and solar energy in Nigeria was carried out by (Tsado and Ganiyu, 2012) [2] ; the result showed that the initial cost of installing solar energy for standalone street lighting is N 6,402/h. which is more expensive than using public electricity utility with a cost of N 2,508.9/h. However, over 20 years, such projects become feasible. The feasibility emphasizes the need to supplement and eventually replace the existing conventional street lighting powered by electricity from public utilities. Recently many studies have been made on the feasibility of introducing photovoltaics to the lighting sector. Wu, et al. (2009) [3]. investigated the design of the solar powered LED roadway lighting using high-power LED luminaries (100 W) and estimated the installation cost for a 10 km highway with 2 lanes. This solar-powered LED roadway lighting system can save 75% of lighting energy as compared to the mercury lamp. The payback time for the excess investment of the whole lighting system is 3.3 years for high-power LED using solar power.

Rizaho, China is a solar-powered city. In addition to 99% of households in the central district, most traffic signals and street lights are powered by photovoltaics. The achievement was the result of an unusual convergence of three key factors: a supportive government policy, local solar industries that seized the opportunity, and strong political will among the city's leadership. In addition, government buildings and homes of city leaders were the first to have the panels installed. [4]

2.2 Solar street light components:

The solar street lighting system has several components. In this chapter, we will provide a quick introduction to the solar cell panel, battery, controller, LED light, light sensor, and street lighting poles as shown in Figure 2.1.

Figure (2.1): solar LED street light components.

2.2.1 Solar Panels:

The solar panel is one of the most important components of solar street lights since the solar panel will convert solar energy into electricity in the form of a direct current (DC). A solar panel is made up of solar cells.  Solar cell that is about 10–15 cm squared. This component converts energy from sunlight into electrical energy [5]. Electric power generated by a solar cell is so small that several solar cells must be combined to form a unit of the component, and this unit is called  solar panel or module [6]. Commonly solar cells made from a semiconducting material such as silicon wafer. There are three types of solar panels [7]. shown in figure 2.2, as follows:

Figure (2.2): Types of solar panels.

- Monocrystalline solar panels:

Monocrystalline silicon (or "single-crystal silicon", "single-crystal Si", "mono c-Si", or just mono-Si) is the base material for silicon chips used in virtually all electronic equipment today. Mono-Si also serves as photovoltaic, light absorbing material in the manufacture of solar cells. It consists of silicon in which the crystal lattice of the entire solid is continuous, unbroken to its edges, and free of any grain boundaries. Single-crystal silicon is perhaps the most important technological material of the last few decades—the "silicon era", [8]. because its availability at an affordable cost has been essential for the development of the electronic devices on which the present day electronic and informatic revolution is based [9]. The efficiency of converting the solar energy via the solar panels consisting of this material can be in the range of 15e25% [10],[9] Monocrystalline silicon differs from other allotropic forms, such as the non-crystalline amorphous silicon—used in thin-film solar cells, and polycrystalline silicon, that consists of small crystals, also known as crystallites [9].

- poly-crystalline solar panels:

Polycrystalline technology: Polycrystalline silicon modules are another technology derived from crystalline silicon. These modules have their first efficiency records in 1984. At that time, laboratory tests recorded efficiency values below 15%. Although, nowadays, it is possible to reach efficiency of 22.3% in laboratory environment. and for largescale production efficiency is in the range of 14 - 20%.

Compared with the monocrystalline technology, one can notice lower efficiency levels. Although, the production costs and the silicon residues generated are also reduced.

The low costs in the manufacturing process of the polycrystalline silicon is due to the fabrication method of block smelting. This method does not need the extraction stage for the formation of the ingot. The silicon is melted and poured in a square coated with SiO/SiN graphite crucible. Next the material undergoes a controlled cooling process which produces a solid block of polycrystalline silicon. Then the block is cut into square wafers for the manufacture of the solar cells. The disadvantage of this method is that the higher polycrystalline silicon contact with the crucible allows a greater transfer of impurity from the crucible to the block of polycrystalline silicon. [12]

- Thin-film solar panels:

A thin-film solar cell (TFSC), also called a thin-film photovoltaic cell (TFPV), is a second generation solar cell that is made by depositing one or more thin layers, or thin film (TF) of photovoltaic material on a substrate, such as glass, plastic or metal. Thin-film solar cells are commercially used in several technologies, including cadmium telluride (CdTe), copper indium gallium diselenide (CIGS), and amorphous and other thin-film silicon (a-Si, TF-Si). Film thickness varies from a few nanometers (nm) to tens of micrometers (µm), much thinner than thin-film's rival technology, the conventional, first-generation crystalline. The conversion rate of the mono-crystalline solar panel is much higher than poly-crystalline silicon solar cell (c-Si), which uses silicon wafers of up to 200μm. This allows thin film cells to be flexible, lower in weight, and have less drag. It is used in building integrated photovoltaics and as semi-transparent, photovoltaic glazing material that can be laminated onto windows. Other commercial applications use rigid thin film solar panels (sandwiched between two panes of glass) in some of the world's largest photovoltaic power stations. [9]

### 2.2.1.1 Working principle: A Solar cell is a device which converts photons in solar rays into direct current (DC) and voltage. The associated technology is called solar Photovoltaic (PV). A typical silicon PV cell is a thin wafer consisting of a very thin layer of phosphorous-doped (N-type) silicon on top of a thicker layer of boron-doped (P-type) silicon. An electrical field is created near the top surface of the cell where these two materials are in contact (the P-N junction). When the sunlight hits the semiconductor surface, an electron springs up and is attracted towards the N-type semiconductor material. This will cause more negatives in the n-type and more positives in the P-type semiconductors, generating a higher flow of electricity. This is known as the photovoltaic effect. Fig. 2.3 below shows the working mechanism of a silicon solar cell. [13]

Figure (2.3): working mechanism of a solar cell [13].

Battery: 2.2.2

In this section, we discuss a vital component not only of PV systems but of renewable energy systems in general. Energy storage is very important at both small and large scales in order to tackle the intermittency of renewable energy sources. In the case of PV systems, the intermittency of the electricity generation is of three kinds: first, diurnal fluctuations, i.e. the difference of irradiance during a 24-hour period. Secondly, fluctuations from one day to another occur because of changes in weather, especially changes in cloud coverage. Thirdly, the seasonal fluctuations, i.e. the difference in irradiance between the summer and winter months [14].

Batteries are classified as primary and secondary batteries. [15] A primary lithium battery is an un-rechargeable battery with lithium metal elements as the anode. The primary cell is commonly used in watches, calculators, and other electronic devices. A secondary lithium battery is a rechargeable battery that is usually found in consumer electronic products such as notebooks, personal computers, smartphones, electric bikes, street lighting, and many other electronic gadgets [16].

### 2.2.2.1 Types of batteries

- Lead Acid Batteries:

It is available in several types, the most important of which are: Open Lead Acid, Sealed Lead Acid, AGM, Gel, Deep Cycle Batteries.

- Lead Carbon Battery:

Recently, carbon was added to the negative pole of the battery for the purpose of reducing sulfurization and in return increasing the ability to draw/fast charging, which reaches 60% of its capacity without damaging the poles, and withstands a high temperature of up to 60 degrees Celsius.

- Lithium battery:

It is similar to LED-carbon batteries, where iron-lithium phosphate has been replaced, and it has a long life at a 75% discharge rate, unlike its predecessors with its light weight, and its drawback is that it needs integrated charging regulators that regulate the charging and discharging process. [17]

### 2.2.2.2. The structure and operation of Lithium-ion battery:

A lithium-ion battery consists of a positive electrode made of metallic lithium oxide, a negative electrode, usually graphite, and an electrolyte containing a lithium accessory (such as LiPF₆) dissolved in organic solvents. and a separator that prevents contact between the two electrodes, see figure 2.4.

During discharge, lithium ions move from the negative electrode to the positive electrode through the electrolyte, and the electrons produce an electric current in the external circuit. During charging, the ions return from the positive electrode to the negative and accumulate inside the graphite. This action is known as intercalation, the introduction of ions into a substance without changing its composition. In the first charging cycles, a protective membrane forms inside the cell called the solid-electrolyte interlayer (SEI), which slightly reduces the effective capacity but is essential for the integrity of the battery. [18]

The battery will be mounted inside a waterproof metal box that will be installed under the light pole or buried underground near it to ensure protection from weathering and vandalism.

Figure (2.4):  A) Lithium-ion battery during discharge. B) Formation of passivation layer (solid-electrolyte interphase, or SEI) on the negative electrode. [18]

2.2.3 LED Lamp:

LEDs—light-emitting diodes—are electronic components made of semiconductor-type diode material that produce light. The light present in an LED lamp is electromagnetic energy emitted in the visible part of the spectrum [6]. LED lighting is a modern type of lighting that relies on light-emitting diodes as a light source. It offers significant energy savings compared to traditional bulbs, such as halogen or incandescent bulbs. One of its most notable advantages is its longer lifespan (up to ten years of daily use). It produces less heat and requires no warm-up time, turning on instantly when connected to electricity. It is also shock- and vibration-resistant, making it suitable for outdoor lighting, such as street lighting. It is also an excellent environmental choice as it is free of mercury or other harmful substances.

2.2.4 Charge Controller:

An integral component of a solar street lighting system, the solar charge controller is critical to effectively controlling and allocating energy. The function of the Charge Controller:

1) Prevent overcharging: Prevents overcharging, which can harm the battery and shorten its lifespan.

2) Prevent over-discharging: When the battery voltage falls to a certain point, devices are disconnected to prevent the battery from over-discharging.

3) Control the charge: Utilizes strategies like maximum power point tracking (MPPT) and pulse width modulation (PWM) to control the power transfer from the solar panels to the battery.

We distinguish between series and shunt controllers, as illustrated in Figure 2.5.

Figure (2.5): (a) series charge controller, (b) shunt charge controller. [14]

- Series Charge Controller:

The series charge controller is primarily used to regulate voltage and current flow between the solar panel and battery; further, it can be used to disable current flow when batteries are fully charged and to keep batteries from overcharging conditions, which helps to improve the battery performance [19].

b) Shunt charge controller:

This type of charge controller is a shunt charge controller or (parallel or shunt controller), which can be used to regulate voltage and current flow between the battery and load. Further, it is used to divert the excess charge to the shunt load (like an electric water heater) and to provide continuous power to the load without any interruption [20].

2.2.5 Light Sensor:

Light Dependent Resistor (LDR), as the name suggests, the resistance is dependent upon the light incident on it. The LDR resistance changes with intensity of light; with an increase in light intensity, the resistance offered by the sensor decreases. LDR sensor gives an analog input value to the control circuit. This value can be used to automatically turn on/off the LED streetlight [21].

2.2.6 Street Light Pole:

The pole is the foundation of solar-based street lighting. Based on the shape, the pole is one of two types: single or double-arm. A solar-based street lighting pole needs to be adjusted to road conditions. Related to the height of the solar-based street lighting pole, the wattage of the lamp needs to be assessed, as the amount of lighting can affect the size of the irreversible area. A solar-based street lighting pole could have a height ranging from 5 to 14 m. Moreover, the distance between poles varies from 15 to 40 m. The distance between poles depends on the height of the pole, the type of lamp, and the required light (brightness) .[6], Galvanized steel was chosen from aluminum and iron for technical and economic reasons, and for its high resistance to corrosion and weather conditions thanks to the zinc layer that encapsulates it, which prolongs its shelf life. It also has mechanical rigidity that makes it suitable for use in outdoor installations such as light poles.

2.3 Operation of a solar street lighting system:

This diagram in Figure 2.6  shows how a solar street lighting system works.

Figure (2.6): Diagram of the operation of a solar street lighting system.

During the day, the solar panel converts the sun into electrical energy in the form of direct current (DC), then this energy is transferred to the controller that regulates the flow of electricity and protects the battery from overcharging or over-discharging, the controller charges the battery during the day and stores the energy in it for later use during the night.

When the sun goes down and the light level drops, the light sensor (Light Dependent Resistor - LDR) detects this change in light intensity, and when the light reaches a certain level, the sensor sends a signal to the controller to turn on the LED. This light works using the energy stored in the battery and illuminates the street with high efficiency. In the morning, when the natural light increases, the light sensor also detects this change and sends a signal to the controller to turn off the lamp, starting the cycle all over again.

CHAPTER 3

Existing Sebha Solar street light

3.1 Location of existing solar light

In 2021, the city of Sebha saw the first signs of solar light  project, which is thought to be a cost-effective and safe solution to the ongoing power outage crisis. 1,070 poles were installed to cover the  main road from  Sebha airport and Abed neighborhood ,  and internal roads of Sebha Airport.  Air travel has recently resumed in Sebha city without worrying about power outages, particularly at night.

The municipality of Sebha city, in collaboration with the United Nations Development Program installed a 10-kilometer lighting line  shown on Figure 3.1. which passes through Jamal Abdel Nasser Street in the heart of Sebha, from Sebha Airport to Sihban Mosque in the Abedkafi neighborhood.

Figure (3.1): lighting line installation.

The LED lights pole shown in figure 3.2, powered by solar panels and use battery to store electricity during sun light. and discharge it during night.  
UNDP Resident Representative in Libya Gerardo Noto previously said that the project not only helps achieve the 2030 Sustainable Development Goals, especially its seventh goal, "Affordable and Clean Energy," but also promotes social cohesion by boosting economic activity and enhancing security.

Figure (3.2): Pole of street lighting.

3.2 Existing solar street light components:

The specifications of the solar panel street lighting system project were given by a company called "Insiab Libya Solar" shown in Table 3.1

Table (3.1): Existing design specification

| Components | Specifications |
| --- | --- |
| Solar panel | Mono, 340Wp |
| LED lamp | 40W per lamp |
| Battery | Gel, 120Ah |
| Charge controller | 30A, 24V ,MPPT |
| Sensor | LDR |
| pole | 9m, galvanized steel |

CHAPTER 4

Design of Street Lights

The steps used for designing street lights are summarised as follows:

1. Determine the location and the size of the area to be covered by LED light.

2. Study solar irradiance and climate.

3. Define the required illumination level.

4. Calculation of energy and system sizing and selection of light components.

4.1 Location of proposed street lights:

This project aims to extend the existing solar street light in Sebha to include other areas.  The location of streets to be lighted with solar energy, is shown in Figure 4.1. The light poles will extend 2.7 km along Al-Qardah Street and Sebha university campus.

Figure (4.1):  Location of proposed street lights.

Total area: 470,365.00 m2(5,062,966.75 ft2).

Total distance: 2.76 Km (1.71 mi).

4.2 Study solar irradiance and climate:

The city of Sabha, the capital of southern Libya and one of the three most important cities in the country, is located in the southwestern part of Libya at latitude 27.05 north and longitude 14.42 east, where the highest value of Direct Normal Irradiance (DNI) was recorded at 939.571 W/m².[22]

This desert city and its surroundings are characterized by low rainfall even in winter, where it does not rain much and for long periods of time, and hot and dry winds blow over the area for most of the year.

4.3 Design of street lighting

The design of street lighting is carried out by software called DIALux 4.12, a professional program used to simulate and analyze outdoor and indoor lighting, where all technical data related to lamps, roads, and ambient conditions were entered to achieve an efficient and energy-efficient distribution, as shown in Table 4.1.

Table (4.1): Technical data related to the lamp.

| Design inputs |  |  |
| --- | --- | --- |
| 25.000 m | Pole distance |
| 9.000 m | Light center height |
| 2 | No. of luminaires of pole |
| 15.0° | Boom angle |
| 2.000 m | Pole distance from roadway |
| 1.729 m | Boom length |
| 220 Lm/W | Lumen efficiency |
| 40 W per lamp | Power LED light |

This specification ensures that the road is properly lit and minimizes energy consumption, which supports the project's main objective of improving energy efficiency using sustainable sources. Figure 4.2 shows the design for correcting the desired light level.

Figure (4.2): Simulate the desired light level.

4.4 System sizing and light components:

The selection the size of the PV panel and the battery depending on the load characteristics. The standard 80 W LED street lighting was selected, and the operating voltage is 24 V.

To calculate the daily load, we need to know the number of operating hours, the following formula is used to calculate the daylight hours. [23]:

N=215cos-1-tanφtanδ,                                    (4.1)

where N is the daylight length (hours), φ is the latitude, which in our case is 27°′, δ is the solar declination angle, which can be determined using the Cooper formula [23]:

δ=23.45°sin360284+n365,                                                  (4.2)

where n =1,…365. The monthly average daylight hours in Sebha city are given in Table 4.2. From the table the average daylight hours is12.01 which makes the night hours 12.01 and also the operating hours of the street lighting is 12.01 hours. Then the load has the following specifications: Hours of operations per day are 12 hours, from 18:00 PM to 06:00 AM, the days of autonomy are 3 days, the system voltage level is 24 V, and the LED lamp power rating is 40W. [23]

Table (4.2): The average daylight hours in Sebha City

| Month | Average Daylight (h) | Declination Angle δ |
| --- | --- | --- |
| Jan | 10.8 | -17.8 |
| Feb | 11.4 | -8.7 |
| Mar | 12.3 | 3.6 |
| Apr | 13 | 14.6 |
| May | 13.6 | 21.9 |
| Jun | 13.7 | 23.2 |
| Jul | 13.3 | 18.2 |
| Aug | 12.6 | 8.1 |
| Sept | 11.7 | -3.8 |
| Oct | 11 | -15.1 |
| Nov | 10.4 | -22 |
| Des | 10.3 | -23.1 |
| Average | 12.0 | - |

From table 4.2, the average daylight hours are 12.01, which makes

The operating hours of the street lighting 12 hours, from 18:00

PM to 06:00 AM [24]

In designing solar-based street lighting, the energy requirement, the photovoltaic area coverage, and the operational cost need to be assessed. This serves to determine the amount of energy and specification that must be delivered; thus, it can provide the components required to be employed.

- Total Lamp Power:

Total lamp power can be formulated as follows:

Ptotal= P × n                                         (4.3)

40W × 2 = 80W

Where:

Ptotal: total lamp power (W).

P:  lighting power (W).

n: number of lights.

B.      Lamp Power Every Day:

The power of each day can be formulated by:

Pday  = Ptotal × t                                      (4.4)

80W × 12 h = 960Wh

Where:

Pday  : light power per day (kWh).

t = time (h).

C.     daily power System Losses:

Pday+losses=Pday × SF                                    (4.5)

960 ×1.2 = 1152Wh

Pday+losses ≈ 1200Wh

Where:

Pday+losses = daily power plus System Losses

SF = Safety Factor

D.      Power of Solar Panels:

Psolar =  Pday+losses ÷ T                             (4.6)

= 1200 ÷4 = 300 W

Where:

T: Number of Sun equivalent Hours (h).

It is preferable to choose your slightly higher capacity to extend the life of the board: 350W

depth of discharge (DoD) is an important parameter. It is defined as the percentage of the battery capacity that has been discharged; Permissible discharge of depth lithium-ion batteries 90% (DOD= 0.9).

Daily consumption 1200Wh, a reserve period for two days and a voltage of 24V.

1200Wh ÷ 0.9 = 1333 ≈ 1200Wh (to provide security margin)

E.      Required Battery Capacity:

Cbat =  Pday+losses ÷ Vbat                                      (4.7)

1200Wh  ÷ 24= 50Ah

It is always a need to add a small safety margin to compensate for any loss of efficiency or to ensure reliable performance in inappropriate weather conditions: 60Ah

(Lithium battery, 24V/60Ah)

F.      Calculate The Minimum Current:

I = Psolar ÷ V                                                   (4.8)

= 350W÷ 24V =14.5A

Where:

V: Voltage System (24V)

G.     Charging Controller Rating:

Icontroller= I × SF                                              (4.9)

14.5 × 1.2 =17.5A

(add 25% security margin) = 20A

(MPPT Controller, 24V/20A)

H.     Light Sensor:

- Used to detect sunset and automatically trigger lights.

- Usually integrated with the controller, for the need for sizing.

- Pole Spacing:

- Distance between poles: 25 meters.

- Galvanized steel pole.

Summary of each Light Pole as there are a total of 81 light poles; 73 double poles, and 8 single poles as shown in the Table 4.3:

Table (4.3): Summary of each light pole

| component | Specification |
| --- | --- |
| LED Lamp | (Double Pole: 2 LED × 40W = 80W) (Single Pole: 40W) |
| Lumen Efficiency | 220 LM/W |
| Daily Energy Demand | 1200Wh/day |
| Solar Panel | Mono, 350Wp |
| Battery | Lithium Ion, 60A |
| Charge Controller | 20A , MPPT |
| Operation Time | 12 Hour |
| Pole Spacing | Length 9m, spacing 25m, Galvanized steel |

4.5 Results and Discussion:

The system was simulated using the DIALUX program to evaluate the light intensity distribution and suitability of the two systems at 40 W for street lighting, where the previous system produces 150 LM/W and the system that provides a total illuminance of 220 per watt The simulation showed that the two systems (whether using gel or lithium battery) give the same level of illumination in terms of light distribution and illumination intensity on the road surface (lux), because the light source and geometric location are identical in both designs.

The distances between poles, their height, and the angle of light distribution were corrected according to the standard criteria to ensure a homogeneous distribution of light without dark areas or extreme variations in light intensity The simulation results showed that the system complies with the general road lighting standards in terms of lighting level (luminance) and distribution. Look the Figure A in the appendixes.

Summary (according to EN 13201:2015)

Results for valuation fields

A maintenance factor of 0.80 was used for calculating the installation.

(Maintenance Factor) is a ratio used in lighting design to determine the desired level of illumination after taking into account the natural deterioration of performance.

- Eav: Average Luminance. It is the average intensity of light reaching a given surface and is measured in lux.

- Emin: Minimum Illuminance. It is the minimum intensity of light measured on a given surface and is measured in lux.

- Lav: Average Luminance. the average amount of light emitted from a surface is measured in candela per square meter (cd/m²).

- UO: Overall Uniformity. This coefficient reflects the degree of uniformity of light distribution on a given surface.

- Ul: Longitudinal Uniformity refers to evenly distributed light over a given surface.

- TI: Threshold Increment. It is a measure of increased glare that can affect the visual comfort of drivers and pedestrians.

- REI: Longitudinal Illuminance Uniformity. It is a measure of the evenness of lighting distribution along the road.

However, the difference between the two systems does not appear in the direct lighting results, but rather in the continuity of lighting and its efficiency in the long term. Lithium batteries maintain a more stable voltage and current, which ensures a constant glow without a gradual decrease in lighting intensity over time, while the lighting intensity in the battery-gel system may be affected over time if the battery is repeatedly deep discharged, which leads to a gradual decrease in lamp efficiency. Although the simulation results are equal, the actual reliability of daily lighting is higher in the Lithium-ion system.

4.5.1 Selection of solar panel:

The system design is based on the panel type and specifications shown in Table 4.4.

Table (4.4): Solar panel specification.

| Electrical Data |  |
| --- | --- |
| SPR-X21-350-BLK | Model |
| 96 Cells (Monocrystalline) | Type solar Cells |
| Wp350 | Maximum Power (Pmax) |
| 57.3 V | Rated Voltage (Vmpp) |
| 6.11 A | Rated Current (Impp) |
| 68.2 V | Open-Circuit Voltage (Voc) |
| 6.50 A | Short-Circuit Current (Isc) |
| −0.29% / ° C | Power Temperature Coefficient |
| −167.4 mV / °C | Voltage Temperature Coefficient |
| 2.9 mA / ° C | Current Temperature Coefficient |
| −40° F to +185° F (−40° C to +85° C) | Operating Temperature |
| 21.5% | Panel Efficiency |
| 41 lbs (18.6 kg) | Weight |

4.5.2 Economic Analysis:

Table 4.5 An economic comparison was conducted between Gel batteries and Lithium-ion batteries based on total lifespan cost and efficiency. Look at the Figure B and C in the appendixes.

Table (4.5): Characteristics of the two batteries.

| characteristics | Battery-gel | Battery Lithium-ion |
| --- | --- | --- |
| Efficiency | Lower (about 75-85%) | Higher (above 95%) |
| Lifespan | Charging cycle 500-1200 | Charging cycle 2000-5000 |
| Weight and volume | Heavy and large | Light and compact |
| Discharge of Depth (DOD) | No more than 50% is allowed | Up to 90% without damage |
| Operation Temp. Range | Discharge (-20~55℃) Charge (-20~40℃) Storage (-20~50℃) | Discharge (-20~60℃) Charge (0~60℃) Storage (0~50℃) |
| Maintenance | Moderate | Maintenance-free |
| Cost | Initially lower | Higher, but lasts longer in the long run |
| Compatibility with MPPT system | Limited | Excellent |
| Weight | 25.7 Kg | 10.4 Kg |

Although the initial cost of Lithium-ion system is higher, their lifespan, efficiency, and maintenance advantage make them more economically viable for long-term projects like public street lighting.

CHAPTER 5

Conclusion

5.1 Conclusion and Recommendations:

This research presented a comprehensive design and evaluation of a solar LED street lighting system tailored to the climatic conditions and infrastructure of Sebha city, Libya. The study focused on enhancing the energy efficiency, sustainability, and reliability of public lighting using photovoltaic panels, lithium-ion batteries, LEDs, and smart light sensors

Through detailed calculations, proper system sizing, and advanced simulation using DIALux software, the design ensured that the proposed system could operate autonomously for 12 hours per night with minimal maintenance. The use of lithium-ion batteries proved to be more efficient than conventional phone batteries in terms of longevity, cost, and total life cycle cost

An economic analysis of the solar street lighting system indicates its long-term financial viability. By relying on solar energy and reducing dependence on the public grid, the system is expected to contribute to lower operating and maintenance costs. This option is cost-effective and sustainable, especially in areas with high solar radiation such as the city of Sebha. The project is also in line with energy efficiency strategies and promotes the global shift towards clean energy.

Overall, this project contributes to Libya's renewable energy transition by providing sustainable and scalable delivery of public infrastructure. The results confirm that solar-powered streetlights are not only environmentally responsible, but also technically feasible and economically viable for the city of Sebha and similar urban environments.

5.2 Difficulties and obstacles:

During the course of this project, several challenges, both technical and logistical, were encountered. These include:

- The high cost of components, particularly lithium-ion batteries and MPPT charge controllers, which impacted budgeting and financial analysis.

- Lack of infrastructure in some of the target areas, such as poles or proper street layout, which required an assumption-based design.

5.3 Usefulness of the research:

This research offers tangible benefits, both theoretical and practical:

- It provides a scalable model for solar street lighting in desert and off-grid areas.

- Promotes the adoption of clean energy technologies in public infrastructure, in line with SDG 7.

- Enhances public safety and quality of life, especially in underserved areas where power outages are frequent.

- Reduces government spending on public lighting.

- Contributes to academic knowledge by demonstrating a full cycle of design, simulation, and economic analysis using real site conditions.

5.4 Recommendations:

Based on the findings of this study, the following recommendations are proposed:

1. Adopt lithium-ion batteries in large-scale projects for their longer   life, higher efficiency, and lower maintenance needs.

2. Expand the scope of the project to include more streets and vital public     areas in Sebha and similar cities.

3.  Strengthen training programs on solar energy system design and simulation.

4. Provide training programs for local engineers and technicians to build    capacity in the design and maintenance of solar lighting systems.

5.  Encourage partnerships between governments and donors to finance solar infrastructure.

6.  Develop and integrate smart control systems, such as automatic dimming and motion sensors, to further reduce energy consumption.

Appendixes

Figure A

Summary (according to EN 13201:2015)

Figure B

Figure C