State of Libya

Ministry of Education and Scientific Research

University of Sabha

Faculty of Engineering

Petroleum and Gas Engineering Department

Thesis submitted in fulfillment of the requirements for the degree of Bachelor’s

Entitled:

The title: A Numerical Perspective on the Integration of Enhancement and Storage Techniques in Depleted Reservoirs

Student achievement

Ali Hassan Ali Alannaiyd                          Hamad Mohamed Nasr Belfqera

22200057                                                       22200021

UNDER SUPERVISORS

Mr. Mohammed A. Samba

Academy Year / 2025-2024

{ ‏‏يَرْفَعِ اللَّهُ الَّذِينَ آمَنُوا مِنكُمْ وَالَّذِينَ أُوتُوا الْعِلْمَ دَرَجَاتٍ }

[الآية (11) من سورة  المجادلة]

APPROVAL SHEET

Thesis submitted in fulfillment of the requirements for the degree of Bachelor’s

By

Ali Hassan Ali Alannaiyd                           Hamad Mohamed Nasr

Bachelor program of Petroleum Engineering

Sabha University

Approved as to style and contents Supervising Committee

Date………………

Advisor

Mr. Mohammed A. Samba

………………………

ACKNOWLEDGMENTS

Praise be to Allah who gave us quality and generosity.

We would like to thank our families who have been the primary supporters of reaching this stage and everyone who taught us letters and contributed to our reaching this stage.

Thanks and appreciation to the faculty members in the College of Engineering.

We would like to thank our supervisor Mr. Mohamed Samba for the support and information that he provided for us during our educational journey and during the research period for his idea and his guide to us.

We also express our sincere gratitude to MED Fezzan Center for providing the necessary equipment and for their cooperation during the preparation of this work.

Finally, we would also like to thank everyone who supported us and helped us if with a kind word.

Ali Hassan Ali Alannaiyd

Hamad Mohamed Nasr

# Abstract

# This study investigates the effectiveness of gas storage and secondary oil recovery in depleted reservoirs through specialized reservoir conditioning techniques. A novel method is introduced into the CCUS (Carbon Capture, Utilization, and Storage) framework by introducing a specialized interfacial layer between reservoir phases. This layer is designed to manage fluid interactions, sustain appropriate pressure conditions, enhance fluid displacement, and mitigate premature production issues.

# A comprehensive numerical simulation using CMG software was conducted to evaluate the performance of the specialized interfacial layer in reducing gas solubility in both oil and water phases. The results highlight improved gas containment and an increase in the oil recovery factor. Additionally, a mathematical model based on mass transfer theory and Fick’s Law was developed and solved using MATLAB to analyze gas diffusion dynamics through the specialized interfacial layer.

### Two scenarios were examined: (1) conventional gas injection without a specialized interfacial layer and (2) gas injection with the specialized interfacial layer. The simulation results reveal that the introduction of a polymer layer significantly reduces gas solubility in oil, thereby enhancing both gas storage efficiency and secondary oil recovery performance.

### The findings indicate that specialized interfacial-layer-assisted gas injection presents a promising and practical approach for improving reservoir productivity and supporting the long-term sustainability of subsurface gas storage operations.

### Keywords: Depleted reservoirs, CCUS, gas injection, polymer barrier, secondary recovery, gas solubility, CMG simulation, Fick’s law, gas diffusion, reservoir simulation

Table of contents

ACKNOWLEDGMENTS	iii

List of Table	vi

List of Figure	viii

Abstract	iv

Chapter 1. Introduction	1

1.1 Introduction	1

1.2 Problem Statement	4

1.3 Objectives	5

Chapter 2. Literature Review	6

2.1 Underground Gas Storage (UGS)	6

2.1.1 Definition and Importance	6

2.1.2 Types of Underground Gas Storage Facilities	7

2.1.3 Advantages of Using Depleted Reservoirs for Gas Storage	8

2.1.4 Challenges and Technical Issues	9

2.2 Secondary Recovery Methods	9

2.2.1 Overview and Objectives	9

2.2.2 Water Flooding	10

2.2.3 Gas Injection	10

2.3 Polymer as an EOR Technique	12

2.3.1 Polymer Types and Characteristics	12

2.3.2 Mechanisms of Polymer Flooding	12

2.3.3 Polymer Barriers and Flooding in Reservoir Engineering	13

2.1.5 Quantitative Review of Existing UGS Literature	14

Chapter 3. Methodology and Procedures	23

3.1 Mathematical Model of Polymer Blocking	23

3.2 Detailed Explanation of Governing Equations (1)–(20)	23

3.2.1 Numerical Implementation in MATLAB	28

3.2.2 Model Inputs	30

3.2.3 Simulation Scenarios for the Mathematical Model	31

3.3 CMG Reservoir Simulation	32

3.3.1 Model Construction	32

3.3.2 Input Parameters	33

3.3.3 Simulation Scenarios	34

4.1 Mathematical Model Results	36

4.2 CMG Simulation Results	40

Chapter 5. Conclusions and Recommendations	44

5.1. Conclusions	44

5.2 Recommendations…………………………………………………………………………………………………..…….45

References	47

List of Table

Table 3.1 Symbols Table	28

Table 3.2 Model Input Parameters	30

Table 3.3 Reservoir Properties 	34

List of Figure

Figure 1.1 Classification Oil Recovery Method	4

Figure 2.1 Water Flooding Schematic	10

Figure 2.2 Gas Injection Schematic	12

Figure 2.3 Reservoir Type	16

Figure 2.4 Gas Type Distribution injected or stored	17

Figure 2.5 Distribution of Studies by Research Methodology	19

Figure 2.6 Study Locations geographic distribution	20

Figure 2.7 Evolution of Studies Over the Decades	21
Figure3.1.Matlabcode………………………………………………………………………....29

Figure 3.2 CMG Reservoir Model Schematic	32

Figure 4.1 Concentration in oil ( no barrier)	36

Figure 4.2 Concentration in polymer	37

Figure 4.3 Concentration in oil ( 5oo mm barrier barrier)	38

Figure 4.4 Concentration in oil ( 1000 mm barrier barrier)	40

Figure 4.5 Gas In Place ( scenario A: No barrier)	41

Figure 4.6 Oil Recovery  ( scenario A: No barrier)	42

Figure 4.7 Gas In Place ( scenario B: With barrier)	43

Figure 4.8 Oil Recovery (scenario B: With polymer barrier)……….………………………...43

Chapter 1.  Introduction

Chapter 1. Introduction

1.1 Introduction

Oil reservoirs exist within the subsurface and contain hydrocarbons stored within porous and permeable rocks. Hydrocarbons are created when organic matter is changed into hydrocarbons in source rocks over heat and pressure over millions of years. As kerogen matures within the source rock, hydrocarbons are expelled through a process called primary migration and are able to move into the reservoir by travelling through microfractures and pore space toward a zone of lower pressure[1].

Hydrocarbons accumulate in traps during migration path when a rock layer, typically, shale or dense carbonate, seals the way up and arrest further migration. The hydrocarbons then form a petroleum reservoir at the bottom of a structural or stratigraphic trap[2]. Once hydrocarbons are trapped, oil production progresses through three main stages: primary recovery, which utilizes natural reservoir energy; secondary recovery, involving water or gas injection to maintain pressure; and tertiary recovery (enhanced oil recovery), employing advanced methods like thermal, gas, or chemical injection to maximize extraction, as shown in the Figure 1.1.

Oil production efficiency depends on reservoir properties such as porosity, permeability, fluid saturation, and pressure [3]. Primary recovery occurs when the natural energy of the reservoir pushes hydrocarbons to the surface through natural drive mechanisms, including solution gas drive, gas cap expansion, water drive, gravity drainage, and compaction. However, primary recovery is not usually sufficient to maximize reservoir productivity since it typically recovers between 5% and 30% of original oil in place (OOIP) [4].

Methods of artificial lift play an important role in primary recovery, particularly when natural energy deteriorates. At times, the reservoir pressure is not enough to lift the fluids the entire way to surface but it's sufficient to lift the fluids the length of the wellbore. In these cases, one could use some form of artificial lift system such as sucker rod pumping (SRP), gas lift, electric submersible pumps (ESP), plunger lift, or progressing cavity pumps (PCP). These techniques are designed to reduce the bottom-hole pressure and mechanically help lift fluids to the surface thereby maintaining the production rate, despite a drop in reservoir pressure [5].

Reservoirs can be classified according to their geological and fluid characteristics. Sandstone reservoirs usually possess good porosity and permeability and are better suited for producing oil and gas conventionally Carbonate reservoirs are more complex as a result of a variety of pore structures; in many cases, natural fractures may cause more complicated ways to produce these wells, thus requiring advanced recovery methods. Shale reservoirs are an unconventional type of reservoir characterized by extremely low permeability, and they require sophisticated stimulation methods such as hydraulic fracturing in order to produce oil or gas commercially [6].

Petroleum reservoirs can also be classified by fluid characteristics. Oil reservoirs can be classified as undersaturated, where the pressure in the reservoir is greater than the bubble point and all gas is dissolved in the oil, or saturated, where there is gas free from oil and pressure is at or below the bubble point. A gas reservoir will be found as a predominantly single gas phase at initial conditions, while in a gas-condensate reservoir liquid dropout will occur with changes in reservoir pressure [7].

After primary recovery no longer generates enough energy, water, gas or other external fluids are used to enhance recovery. Water injection is the most prominent form of secondary recovery, where injection water supports reservoir pressure to help oil flow from the pay zone to production wells.  gas injection, gas is injected into the reservoir to form a gas cap that displaces the oil better than water and builds enough pressure to drive oil through the formation. The extent of these secondary recovery methods and the overall recovery factors can be increased to 30%to 50% of OOIP with these methods [8].

Enhanced Oil Recovery (EOR) is usually employed when the primary and secondary recovery methods have depleted to their limit. EOR methods include thermal methods, such as steam injection, gas injection methods (using CO2 or nitrogen), and chemical methods, such as polymer flooding and surfactant injection. EOR methods are considered advanced oil recovery methods as they target oil that is residual and can increase recovery fractions upwards to as much as 80% given favorable conditions [9].

The goal of optimising the transition from primary, thru secondary, to tertiary methods of oil recovery is to raise recovery thresholds on our hydrocarbon extraction, increase the useful life of the reservoir and the economic viability of oil field operations. Understanding reservoir, fluid behaviour, and oil recovery mechanisms is fundamental to developing thorough reservoir management plans which successfully match production goals and economic considerations with the results of your reservoir.

Figure 1.1. Classification Oil Recovery Method.

Figure Classification Oil Recovery Method

1.2 Problem Statement

Gas injection during secondary recovery can result in miscibility between the injected gas and reservoir oil, which may increase environmental concerns during production. Additionally, due to the compressibility of gas, reservoir pressure can decline rapidly. According to the literature review, most existing studies do not provide effective or practical solutions for long-term gas storage; rather, they remain largely theoretical and lack real-world applicability.

This study proposes an ideal solution by introducing a polymer barrier prior to gas injection. The polymer acts to minimize direct gas-oil contact, delay gas migration, and facilitate the formation of a stable gas cap. This mechanism helps maintain reservoir pressure, improves oil displacement efficiency, and reduces gas miscibility with oil, thereby enabling higher gas storage capacity within the reservoir.

1.3 Objectives

This study aims to:

- Investigate the impact of a pre-injected polymer barrier on gas behavior and storage in depleted reservoirs.

- Evaluate the effectiveness of the polymer in reducing gas-oil interaction and enhancing pressure support.

- Develop and analyze a mathematical model using MATLAB to describe gas transport and dissolution with a polymer interface.

- Simulate gas injection scenarios using CMG software, with and without polymer barriers.

Chapter 2. Literature Review

Chapter 2. Literature Review

2.1 Underground Gas Storage (UGS)

2.1.1 Definition and Importance

Underground Gas Storage (UGS) is a crucial strategic process that consists of injecting and storing natural gas in underground geological formations—the most common being depleted oil and gas reservoirs, aquifers or salt caverns—becoming necessary when there is low demand or the production of natural gas has outpaced consumer consumption or demand in particular geographical locations. The stored gas in UGS can be retrieved to live up to obligations requiring gas, e.g. during periods of elevated demand, unintended disruptions in supply, or seasonal shifts in consumption.

UGS serves as a cornerstone for national and regional energy security by acting as a dynamic buffer that enhances the reliability and responsiveness of gas supply networks. It not only ensures energy availability during peak demand seasons (e.g., winter in temperate regions) but also supports pipeline system load balancing, optimizes operational flexibility for suppliers and distributors, and facilitates price stabilization in volatile markets.

Moreover, UGS infrastructure plays a critical role in long-term strategic planning, allowing governments and utility providers to maintain strategic reserves for emergencies such as geopolitical conflicts or natural disasters. It also helps enable the integration of renewable energy sources into the energy mix by compensating for their intermittency, thereby contributing to the resilience, sustainability, and decarbonization of modern energy systems [10].

2.1.2 Types of Underground Gas Storage Facilities

Underground Gas Storage (UGS) is primarily confined to three geological formations: depleted hydrocarbon reservoirs, aquifers and salt caverns. Each of these formations have their own benefits and operational features which delineate their capacity for gas storage.

Depleted hydrocarbon reservoirs are widely regarded as the most favorable due to their prior use for oil or gas production, which ensures pre-existing infrastructure such as wellbores and pipelines, along with well-characterized geological and petrophysical properties. This familiarity translates into lower exploration and development costs, improved safety and predictability, and a more straightforward permitting process. Additionally, the historical production data enables precise reservoir modeling and efficient gas injection and withdrawal operations.

Aquifers, which are water-bearing porous rock formations, represent another viable option for gas storage, particularly in regions lacking depleted reservoirs. However, they generally require the development of new infrastructure and extensive characterization efforts, which can increase capital expenditures and introduce higher uncertainty.

Salt caverns provide excellent deliverability rates and operational flexibility. Because they have extremely low permeability and high mechanical strength, salt caverns have much lower risks of gas leakage and allow for easy cycling in operations, suitable for various peak shaving applications or for short-term periods.

Overall, depleted reservoirs continue to be the favored option for UGS due to their cost, availability, and findings from prior analytical studies [10,11].

2.1.3 Advantages of Using Depleted Reservoirs for Gas Storage

Depleted reservoirs provide economic and operational benefits, making them the foundation of underground gas storage (UGS) projects around the world. From an economic perspective, depleted reservoirs reduce capital investment needed for storage development, largely due to the existing infrastructure of wells, pipelines, and surface facilities, left over from earlier hydrocarbon production activities, so that there is reduced need for costly drilling and construction activities.

From an operational perspective, depleted reservoirs have well-known geological and petrophysical properties. In addition, there are historical production, pressure data, and operational knowledge in place, allowing operators to model and simulate the properties of the reservoir and the injection and withdrawal of gas. Finally, reservoirs marginally suitable for hydrocarbon production in terms of their porosity or permeability may be suitable for gas injection and deliverability, because, over the course of years of hydrocarbon production, the cumulative production has often modified the above parameters, as well.

Another key advantage is the relatively high gas recovery efficiency. The reservoir's pressure behavior and fluid dynamics are already well-understood, which reduces the uncertainty associated with new storage projects. This contributes to operational safety, regulatory compliance, and overall project predictability.

Due to these attributes—cost savings, reliable performance, ease of regulation, and favorable reservoir conditions—depleted reservoirs have become the most commonly used type of underground storage facility on a global scale [10].

2.1.4 Challenges and Technical Issues

While depleted reservoirs have their advantages, the key disadvantages are technical difficulties associated with gas migration, leakage through a fault or fracture system, unintended mixing of cushion and working gas, and geomechanical change or activity affecting storage integrity. There are technical challenges of mitigating these barriers and issues with advanced monitoring technologies and detailed geological evaluation  [12].

2.2 Secondary Recovery Methods

2.2.1 Overview and Objectives

The extractions for secondary recovery methods take place after primary extraction to increase oil recovery from the reservoir, maintain reservoir pressure, and displace oil left behind in the reservoir. The goal of these recovery methods is to optimize recovery and keep the reservoir productive [12, 9].

Typically marks the transition of a reservoir into a depleted state. A depleted reservoir is one in which the natural energy—such as solution gas drive, gas cap expansion, or water influx—is no longer sufficient to sustain economic production. At this point, external energy sources, like water or gas injection, become necessary to maintain reservoir pressure and oil mobility. Thus, the initiation of secondary recovery operations is both a response to depletion and a strategy to prolong the productive life of the reservoir.

2.2.2 Water Flooding

Water flooding involves injecting water into an oil reservoir to displace the remaining oil and maintain pressure as shown in the Figure 2.1. Water flooding is preferable because it is simple to execute and cheap; however, it has drawbacks such as an early breakthrough of water and low sweep efficiency in heterogeneous formations [8].

Figure 2.1.  Water Flooding Schematic [8].

Figure 1Water Flooding Schematic

2.2.3 Gas Injection

Gas injection is a widely utilized secondary recovery method applied in reservoirs that have undergone significant depletion of their natural energy. When primary drive mechanisms such as solution gas expansion, water influx, or gas cap pressure are no longer sufficient to sustain production, gas injection provides an external energy source to restore reservoir pressure and improve oil mobility. This technique involves injecting inert or naturally occurring gases into the reservoir to create a pressure front that pushes residual oil toward production wells.

In the context of secondary recovery, gas injection is primarily aimed at pressure maintenance rather than altering fluid properties or achieving miscibility, as shown in the Figure 2.2. The method is especially useful in maintaining production in mature fields where primary recovery rates have declined substantially.

Operationally, gas injection faces several challenges that can affect its effectiveness. These include early gas breakthrough, where the injected gas reaches the production wells prematurely; gravity override, where lighter gas overrides heavier oil; and low areal or vertical sweep efficiency in heterogeneous or fractured reservoirs. Such challenges necessitate advanced reservoir characterization, proper well placement, and optimized injection patterns to ensure uniform displacement and maximize oil recovery Despite its limitations, gas injection remains an economically viable and technically sound strategy for extending the productive life of depleted oil fields, especially when combined with real-time monitoring and adaptive reservoir management [8].

Figure 2.2. Gas Injection Schematic[8].

2.3 Polymer as an EOR Technique

2.3.1 Polymer Types and Characteristics

Polymer flooding involves injecting water-soluble polymers, such as hydrolyzed polyacrylamide (HPAM) and xanthan gum, to improve water viscosity and mobility ratio, thereby enhancing oil sweep efficiency. Critical polymer properties include molecular weight, rheology, stability under reservoir conditions, and resistance to salinity and temperature variations [9].

2.3.2 Mechanisms of Polymer Flooding

The key mechanism of polymer flooding is to improve the mobility ratio between injected water and reservoir oil, thus reducing viscous fingering and channeling, and improving the overall sweep efficiency. Viscoelastic properties of polymers also help redistribute injected fluids into previously unswept reservoir zones [12, 9].

2.3.3 Polymer Barriers and Flooding in Reservoir Engineering

Polymer-based techniques have long been implemented in enhanced oil recovery (EOR) operations, particularly through polymer flooding. However, the use of polymers as physical or chemical barriers to control gas migration during underground gas storage (UGS) and secondary recovery is an emerging concept with promising implications [9].

Polymer flooding traditionally involves injecting water-soluble polymers—most notably hydrolyzed polyacrylamide (HPAM) and xanthan gum—into the reservoir to increase the viscosity of the injected water. This viscosity enhancement helps reduce the mobility ratio between displacing and displaced fluids, thus improving sweep efficiency and minimizing viscous fingering. The success of polymer flooding largely depends on polymer properties such as molecular weight, thermal and chemical stability, salinity resistance, and viscoelastic behavior [9].

Recent research has extended the application of polymers beyond mobility control to function as barriers between injection gas and oil zones. In this context, polymers act as low-permeability, high-viscosity materials that selectively retard gas flow while allowing pressure transmission. This barrier effect is especially useful in scenarios where early gas breakthrough or gravity override threatens the efficiency of gas injection schemes [9].

The viscoelastic and swelling properties of certain polymers enable them to form a semi-permeable layer within the pore network, decreasing gas diffusion rates into the oil zone. Additionally, polymers can fill high-permeability streaks or fractures, effectively acting as conformance control agents that redirect fluid flow toward unswept zones [6].

Despite the promising attributes, the integration of polymer barriers into UGS and EOR schemes remains underexplored. Few studies have attempted to quantitatively model the gas-polymer-oil interactions using numerical methods or validated such models through laboratory or field data [13]. As such, a significant research gap exists in understanding the long-term performance, degradation behavior, and scalability of polymer barriers under reservoir conditions.

This study addresses this gap by employing both a mathematical model and CMG-based reservoir simulations to analyze the effect of polymer barriers on gas diffusion and oil recovery. Through this approach, the research aims to contribute to the growing field of hybrid EOR-UGS technologies with practical implications for enhanced containment and displacement efficiency [12].

2.1.5 Quantitative Review of Existing UGS Literature

To determine the development and current state of Underground Gas Storage (UGS) research, we systematically quantified 60 peer reviewed publications in the UGS literature over the last 40 years. Each publication was categorized according to important parameters including reservoir type, injected or stored gas type, research method (theoretical, simulation, laboratory, or field study), and geographic location to develop a multidimensional view of the field. The synthesizing of these parameters highlights prevailing trends and established practices, but also indicates research gaps and methodological uncertainties to help define research directions and strategic decisions for researchers and industry alike..

1. Reservoir Type

Among the 60 studies, reservoir typologies were examined in detail to understand their suitability for UGS operations:

Depleted Gas Reservoirs (45%): Mostly chosen for their pre-existent infrastructure, known petrophysical characteristics, and lower geomechanical uncertainty. Many researchers utilized these reservoirs in their studies to evaluate cushion gas dynamics and cyclic injection–withdrawal dynamics under actual field conditions. [14, 17_42]

Carbonate Oil Reservoirs (30%): Chosen for their high contrasts in porosity-permeability and widespread availability in mature fields, this study was set up to evaluate the integrity of caprock in repurposing oil-bearing carbonates for gas storage, with careful attention to diagenetic features that affect caprock seal integrity [15, 44, 50-55, 64-73].

Fractured/Naturally Fractured Systems (15%): These systems were targeted because of their increased permeability pathways, and as a result, these systems posed increased deliverability, along with certain challenging aspects of conformance control. In particular, studies tended to couple geomechanical modeling simulations that evaluated the stability of fractures during cyclic gas loading with fluid flow simulation [16, 56-63].

Other Reservoir Types (10%): Including a partially depleted, mixed sandstone–carbonate and saline aquifer reservoirs. While uncommon so, these studies investigated innovative storage solutions, such as hybrid aquifer–reservoir systems and salt-cavern-enhanced systems

Figure 2.3.  Quantitative Review for Different Reservoir Type.

Figure 2Reservoir Type

In conclusion, these classifications indicate that research has been oriented toward reservoirs with pre-existing infrastructure and anticipated behaviour while considering non-traditional formations to broaden the UGS portfolio. [43, 45-49]

2. Injected or Stored Gas Type

Gas species selection directly influences storage performance, safety, and environmental impact. The breakdown is as follows:

Natural Gas (28.3%): The benchmark choice for seasonal storage and peak shaving applications. Research has examined the impact of gas composition (for example, the effects of methane-rich vs. lean gas mixtures) on cushion gas retention and product gas quality . [15-21, 23-27, 30-31, 33, 40].

CO₂ (35%): %): Used both for its storage capacity and compatibility with EOR. The research focused on CO2-rock-brine interactions, trapping mechanisms, and long-term containment approaches in saline aquifers and depleted reservoirs.[14, 28-29, 32, 34-35, 37, 39, 41-43, 45-46, 51-53, 58, 60, 65, 67, 70, 73].

Hydrogen (25%): Emerging focus area driven by decarbonization goals. Investigations assessed hydrogen’s diffusivity, microbial reactivity, and compatibility with reservoir minerals, as well as blending strategies with natural gas for pipeline transport [22, 62-64, 66, 69, 71-72, 38, 48-50, 59].

Mixed Gases (11.7%): Combinations (e.g., CO2–H2, CH4–N2) explored to optimize cushion gas performance and operational flexibility. These studies often implemented advanced compositional reservoir simulation to model multi-component flow and phase behavior [36, 44, 47, 54, 56-57, 61].

This distribution underscores a transition from conventional natural gas storage toward integrated CO2 sequestration and nascent hydrogen storage frameworks.

Figure 2.4. Gas Type Distribution injected or stored.

3. Study Type

A robust methodological spectrum ensures comprehensive evaluation of UGS processes:

Numerical Simulations (50%): Leveraging reservoir simulators (e.g., ECLIPSE, CMG) for parametric studies on injection strategies, thermodynamic behavior, and field development optimization [14-21, 24-32, 36-35, 38-40, 43-47, 50-51, 53].

Laboratory Experiments (21.7%): Core flooding, rock–fluid interaction tests, and geochemical analyses provided empirical validation for simulation models, especially in CO₂ and hydrogen compatibility assessments [33, 56, 61, 63, 65, 72]

Case Studies (15%): Field-specific analyses combining historical production data, well logs, and operational records to reconstruct injection–withdrawal cycles and assess pilot project feasibility [34, 41, 48, 55, 57, 68]

Reviews & Comparative Analyses (13.3%): Systematic literature syntheses and cross-comparisons of UGS technologies, offering strategic insights on best practices, risk management, and economic viability [22-23, 64, 66-67].

By integrating these approaches, researchers achieve a balance between theoretical rigor, experimental validation, and real-world applicability.

Figure 2.5. Distribution of Studies by Research Methodology.

Figure 3Distribution of Studies by Research Methodology

4. Study Locations

Assessing geographic distribution highlights regional research priorities and infrastructure readiness:

Asia (28.3%): Led by extensive studies in China and Iran, focusing on large-scale gas fields and saline aquifer storage to meet seasonal demand [14-17, 19, 24-25, 53-54, 68].

Europe (25.0%): Concentrated in the North Sea (UK, Norway) and continental Europe, with emphasis on depleted fields and CO₂ storage regulations under EU directives [32, 34, 36, 42-44, 55-57, 59, 65, 69-70].

North America (20.0%): Predominantly USA and Canada, exploring salt cavern and depleted reservoir storage, supported by mature gas markets and regulatory frameworks  [23, 28-29, 33, 37-38, 45, 47, 49, 61, 73].

Middle East & Africa (13.3%): Studies in Saudi Arabia, UAE, and Nigeria investigated UGS in desert and deep aquifer settings for grid stabilization [20-22, 26-27, 31, 66].

Australia & New Zealand (8.3%): Pilot projects on hydrogen blending and saline aquifer storage, driven by emerging renewable integration schemes [46, 51-52, 58, 60].

South America & Others (5%):  Emerging research hubs in Brazil and Argentina, exploring UGS potential in basins with growing gas consumption [35, 30, 18].

This regional overview reflects global intent to deploy UGS technologies, tailored to local geological settings and energy policy drivers.

Figure 2.6. Study Locations geographic distribution

5. Evolution of Studies Over the Decades

To understand how research focus has shifted over time, the 60 studies were categorized by publication period:

Before 2000: 10% of studies [23, 42, 49, 55-56].

2000–2010: 25% of studies [15, 17, 19, 24, 28-30, 34, 37, 40, 43, 45, 54, 57, 60].

2010–2020: 40% of studies [16,  18, 20-21, 25-27, 31-33, 35-36, 39, 41, 46-48, 50-53, 58-59, 62].

2020 to Present: 25% of studies [14, 22, 38, 44, 61, 64-73].

This timeline shows a significant increase in UGS research activity in the last two decades, peaking between 2010 and 2020, and sustained interest into the current decade.

Figure 2.7. Evolution of Studies over the Decades.

6. Key Findings Summary

Summary and Findings from 60 Reviewed Studies:

The reviewed studies collectively demonstrate that utilizing depleted oil and gas reservoirs for underground storage of natural gas, hydrogen, and other gases is both technically and economically viable. Most studies confirmed the feasibility of developing storage facilities in depleted carbonate and sandstone formations.

The geological formation type plays a crucial role in storage performance. Naturally fractured carbonate formations and porous sandstone formations were shown to offer favorable characteristics such as high porosity and permeability, essential for high-capacity storage.

Many researchers emphasized the effectiveness of numerical simulation models (such as ECLIPSE 300) in evaluating the performance of depleted reservoirs for storage. These models enabled accurate simulation of injection and withdrawal cycles, facilitating improved operational strategies and pressure stability.

Several studies highlighted the importance of integrating risk analysis and reliability assessment into storage operations. Techniques such as Monte Carlo simulations were applied to ensure optimal performance and to assess potential risks.

The use of various gases, such as hydrogen or nitrogen-enriched natural gas, was shown to help maintain reservoir pressure and facilitate potential future recovery operations.

Geographically, the studies covered a wide range of regions, including the Middle East, Asia, North America, and Africa. This diversity indicates the global potential for implementing underground storage technologies in depleted fields.

Based on these findings, it is evident that developing underground gas storage in depleted reservoirs is a strategic and sustainable solution that enhances energy security and optimizes the use of existing subsurface resources.

Chapter 3. Methodology and Procedures

Chapter 3. Methodology and Procedures

3.1 Mathematical Model of Polymer Blocking

The mathematical model developed in this study describes the mass transfer of gaseous components into an oil layer across a polymer barrier. As mentioned above, this barrier acts as a selective medium that reduces the diffusion of gas into oil. The formulation assumes a thin polymer film between a gas cap and the oil zone, where gas components diffuse through the polymer layer and dissolve in the underlying oil phase. The derivation of the governing equations is based on fundamental principles of mass transfer and conservation of mass, incorporating Fick's law, oil phase expansion, and the time-dependent behavior of polymer thickness.

3.2 Detailed Explanation of Governing Equations (1)–(20)

Fpit=-Di,pSipAp(Ci,g-Ci,o)Lp                                                          Equation (1)
This equation represents Fick's first law of diffusion for component i through the polymer barrier. It calculates the molar flux (Mols) from the gas into the oil phase through the barrier, where:

Di,p: diffusion coefficient of component i in polymer

Si,p: solubility of i in polymer

Ap: cross-sectional area of polymer

Lp: polymer thickness at time t

Ci,g,Ci,o: concentration of i in gas and oil, respectively

Vio=MiCi,o(t)piV,o                                                                             Equation (2)
This relates the volume of component i in the oil phase to its concentration, molar mass Mi,total oil volumeVi.o, and component density pi.

i=1ncMiCi,o(t)pi=1                                                                                 Equation (3)

This normalization constraint ensures that the total volume fraction of all components in the oil phase sums to 1, enforcing volumetric consistency.

∂vi,o(t)∂t=Mipi∂VotCi,ot∂t=MipiFi,o(t)                                                 Equation (4)

This equation uses the product rule to differentiate the volume of component i in oil over time, accounting for changing concentration and oil volume.

∂VotCi,ot∂t=Fi,o=-Fi,p                                                                Equation (5)

This expresses conservation of mass: the rate of accumulation of component i in the oil equals the inflow from the polymer, which is the negative of its flux out of the polymer.

∂vo(t)∂t=Ao∂Lo(t)∂t=i=1ncMipiFi,o(t)                                                    Equation (6)

This equation calculates the total oil volume change over time due to the net accumulation of all components. It links the rate of swelling of oil to the net influx of mass across the polymer barrier.

Substituted by combining Equations (1), (5), and (6)

i=1ncMipiFi,o(t)=MipiDi,pSipAp(Ci,g-Ci,o)Lp                                                 Equation (7)
Derived from equations (1), (5), and (6), this expression defines how the thickness of the oil ganglion increases over time due to multicomponent diffusion from the gas through the polymer.

∂Lo∂t=-∂Lp∂t                                                                                         Equation (8)

Since the total length of the oil-polymer system is assumed constant, this constraint couples the shrinking of the polymer with the expansion of the oil phase.

(Obtained by substituting Equation (7) into Equation (8))

∂Lp∂t=-i=1ncMipiDi,pSip(Ci,g-Ci,o)Lp                                                          Equation (9) This equation is an explicit form derived from (8) and (7), describing how the polymer barrier thickness decreases over time due to net component transfer into the oil.

Ci,ot∂Vo∂t+Vot∂Ci,o∂t=Fi,o(t)                                                      Equation (10)   Defines the molar accumulation rate of component i in the oil phase as the total derivative of the product of concentration and volume.

Vi.o=AoLo(t)  Equation(11) 
A geometric relation stating that the oil volume is a function of the initial and time-varying thickness, scaled by area.

(Derived by substituting Equations (1), (5), (6), and (11) into the time derivative of Equation (10))

∂Ci,o(t)∂t=Di,pLp(t)Sip(Ci,g-Ci,o)+Ci,o(t)∂Lp(t)∂tLo(t)                                                 Equation (12)

This governs how the concentration of component i in the oil phase evolves over time due to diffusion into the oil and the effect of oil swelling.

Transition to Dimensionless Form (Substitution and Scaling)

To facilitate numerical simulation and generalization of the physical behavior, the governing equations are converted to a dimensionless form by introducing characteristic scales for length, time, diffusivity, and solubility.

The following dimensionless variables are introduced:

Lp*=LpLp,in                                                                                          Equation (13) 
Defines the dimensionless polymer thickness relative to its initial value.

Si=Sco2,PS1,p                                                                                          Equation (14)

Dimensionless solubility of component i, normalized by that of methane.

Di=Dco2,PD1,p                                                                                         Equation (15)

Dimensionless diffusivity of component i.

Ci,g*=MipiCi,g                                                                                  Equation (16)
Dimensionless concentration of component i in the gas phase.

Ci,o*=MipiCi,o                                                                                Equation (17)
Dimensionless concentration of component i in the oil phase.

td*=tD1,pS1,pLp,in^2                                                                                     Equation (18)

Defines dimensionless time scaled by polymer properties.

(Derived by non-dimensionalizing Equation )

∂Lp*∂t=-i=1ncDi,p*Si,p*(Ci,g*-Ci,o*(t))Lp*(t*)                                                           Equation (19)
A key dynamic equation that models the shrinkage of the polymer barrier over time in a normalized framework.

(Derived by non- dimensionalizing Equation)

∂Ci,p*∂t=Di,p*Lp*Si,p*Ci,g*-Ci,o*t*+Ci,o*(t*)∂Lp*∂xt*Lop*+1-Lp*(t*)                     Equation (20)
Final governing equation in dimensionless form, combining mass transfer and the effect of expansion of the oil region.

This model allows for evaluating how polymer barriers can limit gas diffusion into the oil phase and thus serve as effective blocking agents in enhanced gas storage and recovery systems.

Table 3.1 – Symbols Table

| Symbol. | Meaning | Unit |
| --- | --- | --- |
| Di,p | Diffusion coefficient of component i in polymer | m²/s |
| Vio | Volume of component i in the oil phase | m |
| Di,o | Diffusion coefficient of component i in oil | m²/s |
| Ci,o(t) | Concentration of component i in the oil phase | mol/m³ |
| Ci,g(t) | Concentration of component i in the gas phase | mol/m³ |
| V,o | oil volume | m |
| pi | Density of pure component i | kg/m³ |
| Si | Solubility of component i | mol/mol |
| Lp | polymer barrier thickness | m |
| td* | Dimensionless time | m²/s |
| pi | Density of pure component i | kg/m³ |
| Mi | Molar mass of component i | kg/mol |
| Lop* | Dimensionless ratio of oil thickness to water barrier thickness | - |

3.2.1 Numerical Implementation in MATLAB

The final set of governing equations (Equations 19 and 20), formulated in dimensionless form, was solved numerically using MATLAB. The built-in solver ode45 was employed due to its effectiveness in handling non-linear ordinary differential equations (ODEs) with smooth dynamics.

Figure 3.1.Matlab code.

Initial conditions were specified for both the dimensionless polymer barrier thickness Lp*=0 and the initial dimensionless concentrations Co,i*=0, which reflect the undisturbed state of the oil before gas begins diffusing through the polymer. The simulation proceeds until a pre-defined condition such as equilibrium or critical shrinkage of the polymer barrier is reached.

The injected gas used in this model was carbon dioxide Co2, chosen due to its relatively high solubility in liquids and relevance to underground gas storage scenarios. Gas-phase concentrations Cg,i*=1 were kept constant at the interface, assuming continuous supply and negligible gas depletion.

These modeling conditions help evaluate the effectiveness of the polymer barrier in delaying or inhibiting oil dissolution into Co2, which is crucial for assessing the feasibility of gas storage and recovery enhancement strategies in porous media.

This numerical framework bridges the mathematical formulation and physical interpretation, enabling parametric studies and sensitivity analyses to be performed efficiently across different fluid systems.

3.2.2 Model Inputs

Table 3.1 summarizes the physical and thermodynamic parameters used in the simulation. These include calculated values for the solubility and diffusivity of CO2 in the polymer under the specified conditions, as summarized in the following table: The model requires several key physical and thermodynamic input parameters:

- Diffusion coefficient and solubility of CO2 in the polymer.

- Initial thicknesses of the polymer barrier and oil region.

- Molar mass and density of oil-phase components.

- Gas concentration, assumed constant at the interface.

- Simulation time step and total duration.

Table 3.2 – Model Input Parameters

| Parameter. | Symbol | Value | Unit | Description |
| --- | --- | --- | --- | --- |
| Temperature | T | 300 | K | System temperature |
| Pressure | P | 1e5 | Pa | CO2 partial pressure |
| Gas concentration at oil(dimensionless) | Co | 0 | – | Interface gas concentration |
| Gas concentration (dimensionless) | Cg | 1 | – | Interface gas concentration |
| Initial polymer thickness | Lp₀ | 500e-6 | m | Initial thickness of polymer barrier |
| Reference solubility for CH₄ | S_ref | 0.0097 | mol/mol | Used for non-dimensional scaling |
| Reference diffusivity for CH₄ | D_ref | 5.8283e-11 | m²/s | Used for non-dimensional scaling |
| Solubility of CO₂ in polymer | S_CO₂ | 2.9058e-05 | mol/mol | - |
| Diffusivity of CO₂ in polymer | D_CO₂ | 1.0843e-16 | m²/s | - |

3.2.3 Simulation Scenarios for the Mathematical Model

To evaluate the effectiveness of the polymer as a diffusion barrier, several scenarios were designed to reflect realistic gas storage and transport conditions in porous media. These scenarios investigate the interaction of CO2 with oil and polymer barriers under different configurations.

Scenario 1: CO2 with Polymer and CO2 with Oil

A comparative simulation between two systems:

- CO₂ in contact with the polymer barrier only.

- CO₂ in direct contact with the oil phase.

This setup helps isolate the role of the polymer in reducing gas-oil miscibility.

Scenario 2: CO₂ with Polymer Barrier (500 µm)

A simulation representing a moderate-thickness polymer barrier to evaluate its effectiveness in slowing CO₂ diffusion and delaying oil saturation.

Scenario 3: CO₂ with Polymer Barrier (1000 µm)

A simulation with a thicker polymer barrier to investigate how increased resistance affects the mass transfer process and interface dynamics.

These scenarios were solved using the dimensionless mathematical model, and their results are interpreted and discussed in Chapter 4.

3.3 CMG Reservoir Simulation

3.3.1 Model Construction

The CMG (Computer Modelling Group) simulation model was built to represent a simplified section of a oil reservoir undergoing gas injection for storage and enhanced oil recovery. The model focuses on simulating gas behavior, oil saturation changes, and the role of a polymeric barrier in restricting gas flow and evaluating its effectiveness as a gas-blocking mechanism.

Figure 3.2. CMG Reservoir Model Schematic.

Figure 4Cmg Reservoir Model Schematic

A 3D Cartesian grid system was used to represent the reservoir, with dimensions adjusted to capture vertical layering. The layers from bottom to top include:

Oil Zone: The bottom layer representing the initial oil-saturated region.

Polymer Zone: The middle layer representing the polymer barrier.

Gas Injection Zone: The top layer where gas is injected.

Key construction elements:

Grid size: 14 × 14 × 4 cells (horizontal and vertical resolution)

Layer arrangement: oil in layer 3, polymer in layer 2, gas in layer 1

Initial conditions: oil saturation in the bottom layer, polymer saturation in the middle layer, and gas saturation in the top layer.

This model structure enables the evaluation of gas migration through the polymer and its impact on oil displacement, saturation dynamics, and ultimately, the polymer's effectiveness in blocking gas movement and enhancing the overall recovery factor.

3.3.2 Input Parameters

Polymer injection was initiated in 2028 to establish a blocking layer within the reservoir. This was followed by the commencement of gas injection in 2029, after the completion of the primary recovery phase. The simulation was extended through a forecast period up to 2040 to evaluate long-term performance.

The key input parameters employed in the CMG simulation model are summarized in the table below. These parameters primarily include reservoir properties relevant to fluid flow, injectivity, and storage performance.

Table 1 Reservoir Propertie

Table 3.3 – Reservoir Properties

| Property | Symbol | Value | Unit | Description |
| --- | --- | --- | --- | --- |
| Porosity | ϕ | 0.25 | – | Uniform porosity across all layers |
| Permeability (horizontal) | Kx | 1000 | md | Horizontal permeability in x-direction |
| Permeability (horizontal) | Ky | 1000 | md | Horizontal permeability in y-direction |
| Permeability (vertical) | Kz | 500 | md | Vertical permeability |
| Oil Zone | – | Layer 3 | – | Initial oil saturation |
| Polymer Zone | – | Layer 2 | – | Middle layer with polymer injection |
| Gas Injection Zone | – | Layer 1 | – | Top layer gas entry |

3.3.3 Simulation Scenarios

To evaluate the effect of polymer barriers and gas injection strategies on oil recovery and gas containment, several simulation scenarios were implemented using CMG. These scenarios help assess how the presence and thickness of the polymer layer influence the gas-oil interaction, recovery factor, and gas migration.

Scenario 1: Gas Injection without Polymer Barrier

- Gas is injected directly into the oil-saturated reservoir.

- No polymer layer is included in the model.

- Gas injection begins in the year 2029, following the end of the primary recovery phase.

- Used as a base case to evaluate oil recovery without any gas-blocking mechanism.

Scenario 2: Gas Injection with Polymer Barrier

- A polymer barrier is placed in the middle layer (Layer 2).

- The polymer was injected in the year 2028, one year prior to gas injection.

- Gas is injected through the top layer starting in 2029.

- This scenario evaluates the effect of a moderate-thickness barrier on delaying gas penetration into the oil zone.

Chapter 4.  Results and Discusion

Chapter 4.  Results and Discusion

4.1 Mathematical Model Results

Scenario 1: Gas dissolve into Oil and Polymer

Figure 4.1 shows the concentration profile of CO₂ in the oil phase over time for the base case scenario (gas-oil system). The system initially contained zero CO₂ in the oil phase, and diffusion started from a defined gas concentration. The simulation was run for a total time of 17000 seconds.

Figure 4.1. Gas Concentration in oil (no barrier)

Figure 5 Concentration in oil ( no barrier)

The results indicate a rapid increase in oil-phase CO₂ concentration at the beginning of the simulation, followed by a gradual approach toward equilibrium as the system neared saturation. The final concentration approached the theoretical saturation value calculated from the solubility S, which confirms the consistency of the numerical model. The shape of the concentration-time curve is consistent with Fickian behavior in porous media.

Figure 4.2 presents the concentration profile of CO₂ in the polymer phase under the same initial gas concentration and boundary conditions. The x-axis represents time in days, and the y-axis shows the normalized concentration in the polymer. The curve shows a gradual increase over time, reflecting the lower diffusion coefficient and lower solubility capacity of the polymer compared to oil. This indicates that the polymer acts as a retarding medium for CO₂ diffusion, making it a potential barrier or buffer in storage applications.

Figure 4.2.  Concentration in polymer.

Figure Concentration in polymer

Scenario 2: Effect of a Barrier on CO₂ Diffusion (Polymer Thickness = 500×10⁻⁶ m)

To evaluate the effectiveness of a physical or chemical barrier in slowing CO2 migration, a secondary simulation was performed with a polymeric barrier introduced between the gas phase and the oil zone. This scenario aims to replicate realistic conditions where a low-permeability layer is used to restrict gas movement.

The results demonstrated a significant delay in the onset of CO2 accumulation in the oil phase compared to the base case. The polymer layer acted as a resistance interface, reducing the flux of CO2 into the underlying oil. The concentration in the oil phase rose at a much slower rate, indicating the barrier’s potential to prolong storage and limit gas dissolution.

This finding supports the hypothesis that engineered barriers can enhance storage security and modify the gas injection strategy for both underground storage.

Figure 4.3. Concentration in oil (5oo µm polymer barrier)

Concentration in oil ( 5oo mm barrier barrier)

Figure 4.3 shows the concentration profile of CO₂ in the oil phase when a polymeric barrier of thickness 500×10⁻⁶ m was introduced. The results demonstrate an extremely low rate of CO₂ accumulation in the oil zone, with concentrations remaining near-zero for an extended time. This confirms the barrier’s effectiveness in minimizing gas transport, as evident from the very small values on the concentration axis.

Scenario 3: Effect of a Thicker Barrier on CO₂ Diffusion (Polymer Thickness = 1000×10⁻⁶ m)

A third scenario was simulated to assess the impact of increasing the barrier thickness to 1000×10⁻⁶ m. The resulting concentration profile is shown in Figure 4.4. The data illustrate an even greater resistance to CO₂ migration, with gas concentration values at the oil interface remaining at negligible levels throughout the simulation period.

The extended diffusion path and reduced effective permeability resulting from the thicker barrier confirm the trend that increasing barrier thickness enhances gas containment. Such a design could be beneficial in long-term CO2 storage or EOR processes where containment integrity is a priority.

Figure 4.4.  Concentration in oil (1000 µm polymer barrier)

in oil ( 1000 mm barrier barrier)

The results demonstrate an extremely low rate of CO₂ accumulation in the oil zone, with concentrations remaining near-zero for an extended time. This confirms the barrier’s effectiveness in minimizing gas transport, as evident from the very small values on the concentration axis.

4.2 CMG Simulation Results

To complement the mathematical modeling, a series of numerical simulations were conducted using the CMG (Computer Modelling Group) software. These simulations aimed to evaluate the dynamic behavior of natural gas injection into the reservoir under various configurations, including the effect on gas storage capacity and oil recovery efficiency.

Scenario A: Gas Injection without Barrier

In the absence of a barrier, natural gas was injected into the reservoir and allowed to migrate freely into the formation. The simulation results showed that Gas In Place (GIP) decreased sharply at the early stage, reflecting the rapid migration and production of gas due to high mobility and lack of resistance. This behavior led to an early breakthrough of natural gas into production wells, which limited the displacement of oil and resulted in a moderate oil recovery factor.

- Gas in Place: Final GIP was approximately 2.36 × 10⁸ SCF, with a noticeable decline in the early years due to rapid breakthrough.

Figure 4.5. Gas in Place (scenario A: No barrier).

- Oil Recovery: Reached 29.72% OOIP, indicating moderate effectiveness in oil displacement under unrestricted gas flow, (see figure 4.6).

Figure 4.6. Oil Recovery (scenario A: No barrier).)

Scenario B: Gas Injection with Polymeric Barrier

This scenario introduced a low-permeability polymeric barrier at the gas-oil interface. The presence of the barrier effectively delayed the gas front, allowing more time for oil to be swept toward production wells before gas breakthrough occurred. Additionally, the restricted flow path led to a localized accumulation of natural gas, resulting in an increase in Gas in Place (GIP) compared to the case without a barrier.

- Gas in Place: Final GIP was approximately 2.48 × 10⁸ SCF, representing better gas retention than Scenario A.

Figure 4.7. Gas in Place (scenario B: With barrier)

scenario B: With bar

- Oil Recovery: Reached 35.03% OOIP, showing a clear improvement due to delayed gas breakthrough and extended oil displacement (see figure 4.8).

Figure4.8.Oil Recovery (scenario B: With polymer barrier).

Place ( scenario B: With bar

Chapter 5. Conclusions and Recommendations

Chapter 5. Conclusions and Recommendations

5.1. Conclusions

This study has explored polymer barriers' role in enhancing gas containment and oil recovery in secondary gas injection scenarios, resulting in the following key conclusions:

- Polymer barriers significantly delay gas migration into oil zones, reducing gas dissolution rates. Thicker polymer layers (500–1000 µm) demonstrated notable effectiveness.

- Reservoir simulations using CMG indicated polymer barriers improve oil recovery factors by approximately 5% compared to scenarios without barriers.

- Increasing polymer barrier thickness enhances gas retention up to a point, beyond which injectivity and economic feasibility may diminish.

- Polymer barriers effectively mitigate early gas breakthrough, stabilize gas caps, and substantially enhance pressure support during secondary recovery.

In summary, effective integration of polymer barriers with optimized injection strategies, economic feasibility analyses, and continuous monitoring systems is essential to maximizing the potential of polymer barriers in underground gas storage and enhanced oil recovery processes.

5.2 Recommendations

Based on the findings obtained from mathematical modeling and reservoir simulation scenarios presented in Chapters 3 and 4, the following recommendations are proposed:

- Optimization of Polymer Barrier Thickness: Conduct further parametric studies to identify the optimal polymer barrier thickness (~500 µm to 1000 µm) that balances gas containment effectiveness with injection efficiency.

- Long-term Stability and Degradation Studies: Perform comprehensive laboratory and field-scale experiments to assess the polymer barrier's chemical and mechanical stability over extended periods under realistic reservoir conditions.

- Economic and Operational Feasibility Studies: Conduct detailed economic analyses comparing the costs of polymer injection against conventional methods, factoring in polymer production, injection operations, and incremental oil recovery gains.

- Advanced Polymer Formulation Development: Research advanced polymer formulations designed specifically for harsh reservoir environments, focusing on enhancing viscoelasticity, durability, and environmental compatibility.

- Integrated Monitoring and Real-time Management Systems: Implement advanced monitoring technologies (e.g., fiber-optic sensors, tracer tests) combined with real-time analytics for ongoing performance evaluation and proactive management.

- Scale-up and Field Pilot Projects: Initiate pilot projects under actual field conditions to validate lab and simulation results, refine predictive models, and identify potential operational challenges.

- Multicomponent Gas Injection Studies: Investigate the behavior of mixed gases interacting with polymer barriers to enhance understanding of barrier performance under realistic gas storage and recovery scenarios.