State of Libya

Sabha University

Faculty of Mining and Energy

Department of Petroleum and Gas Engineering

Project Title:

Vibration Enhanced Oil Recovery

Student achievement:

Mohammed Omar Ibrahim              Wael Moftah Alhaddad

22140041                                                        22150011

SUPERVISED BY:

Dr. Mohammed .A. Samba

Academic Year

(2025-2024)

بسم الله الرحمن الرحيم

وقل اعملوا فسيرى الله عملكم ورسوله والمؤمنون وستردون إلى عالم                                        الغيب والشهادة فينبئكم بما كنتم تعملون

صدق الله العظيم

سورة التوبة الآية (105)

الإهــــــــــــــــداء

بسم الله الرحمن الرحيم

}قل اعملوا فسيرى الله عملكم ورسوله والمؤمنون{ صدق الله العظيم

إلهي لا يطيب الليل إلا بشكرك ولا يطيب النهار إلا بطاعتك .. ولا تطيب اللحظات إلا بذكرك .. ولا تطيب الأخرة إلا بعفوك .. ولا تطيب الجنة إلا برؤيتك

"الله جل جلاله"

إلى من بلغ الرسالة وأدى الأمانة .. ونصح الأمة .. إلى نبي الرحمة ونور العالمين "سيدنا محمد صلى الله عليه وسلم"

إلى الأرض التي ترعرعنا عليها واستنشقنا نسيم الحياة والحرية فيها وعلمتنا معاني التضحية والفداء

"ليبيا"

إلى من كلله الله بالهيبة والوقار .. إلى من علمنا العطاء بدون انتظار .. إلى من حملنا اسمهم بكل افتخار .. ارجو من الله أن يمد في اعمارهم من لترى ثمارا قد حان قطافها بعد طول انتظار

وستبقى كلماتك نجوم نهتدي بها اليوم وفي الغد وإلى الأبد "أبائنا الأعزاء"

إلى ملاكنا في الحياة .. إلى معنى الحب وإلى معنى الحنان والتفاني .. إلى بسمة الحياة وسر الوجود إلى من كان دعائها سر نجاحنا وحنانها بلسم جراحنا إلى أغلى الحبايب

"امهاتنا"

إلى سندنا وقدوتنا وملاذنا بعد الله إلى من أثرونا على نفسهم إلى من علموني علم الحياة إلى من أظهروا لي ما هو أجمل من الحياة

"أخوتنا"

الأن تفتح الأشرعة وترفع المرساة لتنطلق السفينة في عرض بحر واسع مظلم هو بحر الحياة وفي الظلمة لا يضيء إلا قنديل الذكريات ذكريات الأخوة البعيدة إلى الذين أحببتهم وأحبوني

"أصدقائنا"

ACKNOWLEDGMENTS

Praise be to Allah who gave us quality and generosity.

We would like to thank our families who have been the primary supporters of reaching this stage and everyone who taught us letters and contributed to our reaching this stage.

Thanks and appreciation to the faculty members in the College of Engineering.

We would like to thank our supervisor Mr. Mohammed Samba for the support and information that he provided for us during our educational journey and during the research period for his idea and his guide to us.

Abstract

Vibration-Enhanced Oil Recovery (EOR) has emerged as a modern and promising technique for improving oil extraction efficiency from reservoirs. This project investigates various vibration-based EOR methods, including acoustic, ultrasonic, and mechanical vibrations. A novel technique for generating these vibrations has been developed and successfully implemented. The study has led to innovative insights into the interaction between vibration and injection strategies, with findings currently under the process of publication. Due to the sensitivity and originality of the results, detailed methodologies and specific outcomes are withheld at this stage.

TABLE OF CONTENT

| Title | Page Num |
| --- | --- |
| Acknowledgment | i |
| Thank word | ii |
| Abstract | iii |
| Table Of Content | iv |
| List Of Figures | v |
| List Of Tables | vi |
| Chapter 1: Introduction | 1 |
| 1.1 Introduction | 1 |
| 1.2 Vibration Enhanced Oil Recovery | 5 |
| 1.3 Objective | 6 |
| Chapter 2: Literature Revie | 7 |
| 2.1 Enhanced oil recovery (EOR) | 7 |
| 2.2 Type Enhanced Oil Recovery (EOR) | 7 |
| 2.2.1 Gas Injection | 8 |
| 2.2.2 Thermal Recovery | 9 |
| 2.2.3 Chemical recovery | 10 |
| 2.2.4 Microbial Recovery | 11 |
| 2.2.5 Vibration Enhanced Oil Recovery | 11 |
| 2.3 Vibration Stimulation Historical Background | 12 |
| CHAPTER 3 EXPERIMENT & EQUIPMETN | 17 |
| 3.1 EQUIPMENT | 17 |
| CHAPTER 4. RESULTS and DISCUSSIONS | 25 |
| 4.1 Simulation Results | 25 |
| 4.2 Experimental Results | 27 |
| 4.3 Conclusion | 31 |
| References | 32 |

List of Figures

| Title | Page Num |
| --- | --- |
| Figure (1.1): Chart of oil recovery | 5 |
| Figure (2.1): Flow Chart Enhanced Oil Recovery | 7 |
| Figure (2.2): Gas injection | 9 |
| Figure (2.3): Schematic of heat exchange system | 10 |
| Figure (2.4): Chemical enhanced oil recovery | 10 |
| Figure (2.5): Microbial enhanced oil recovery | 11 |
| Figure (3.1): Digital Sound Level Meter | 17 |
| Figure (3.2): Pipes with different length | 18 |
| Figure (3.3): Piece of rubber with two channels covered with a piece of plastic | 18 |
| Figure (3.4): Two bottles of water of 500 ml | 19 |
| Figure (3.5): Three-dimensional finite differences grid | 21 |
| Figure (3.6): Three-dimensional finite differences grid | 22 |
| Figure (3.7): First scenario | 22 |
| Figure (3.8): Second scenario | 23 |
| Figure (3.9): Third scenario | 23 |
| Figure (3.10): Fourth Scenario | 24 |
| Figure (4.1): First scenario oil recovery factor | 25 |
| .Figure (4.2): Second Scenario Oil Recovery Factor | 26 |
| Figure (4.3): Vibrations results when the injection from 1 side | 27 |
| Figure (4.4): Vibrations results when the injection from 2 different sides | 28 |
| Figure (4.5): Vibrations results when the injection from 2 different sides with cycle injection | 29 |
| Figure (4.6): Vibrations results when the injection from 2 same sides with cycle injection | 30 |

List of Tables

| Title | Page Num |
| --- | --- |
| Reservoir properties | 20 |
| Vibrations results when the injection from 1 side | 27 |
| Vibrations results when the injection from 2 different sides | 28 |
| Vibrations results when the injection from 2 different sides with cycle injection | 29 |
| Vibrations results when the injection from 2 same sides with cycle injection | 30 |

CHAPTER 1 INTRODUCTION

CHAPTER 1 INTRODUCTION

  - Introduction

Oil is a naturally occurring hydrocarbon believed to have originated from the decomposition of animal and plant matter buried in deep sedimentary layers. Being less dense than the surrounding water, petroleum migrated upward from the source beds through permeable rocks such as sandstone and certain types of limestone. This migration continued until it was trapped beneath impermeable layers like shale or dense limestone. These geological formations—comprising the trap, hydrocarbons, and porous media—form what is known as a petroleum reservoir, from which oil is extracted to the surface. The production of oil from reservoirs typically occurs in three distinct phases: primary, secondary, and tertiary recovery, the latter often referred to as Enhanced Oil Recovery (EOR).

Primary recovery relies on the reservoir's natural energy to drive oil to the surface, occasionally supplemented by basic methods such as fluid injection. This approach was the only available technique in the early stages of the petroleum industry and remains the sole method in use in some oil fields, particularly in regions like the Middle East. The driving energy during primary recovery must replace the volume of extracted oil, as the creation of a vacuum within the reservoir is physically impossible. This natural energy originates from five primary mechanisms: (a) solution gas drive, (b) gas-cap drive, (c) water drive, (d) combination drive, and (e) gravity drainage. [1].

Some researchers classify artificial lift techniques as part of primary recovery. These methods are employed when reservoir pressure declines to the point where oil can no longer be produced at an economically viable rate. In such cases, the natural energy of the reservoir is insufficient to lift the oil to the surface, although it may still be capable of moving the oil to the bottom of the wellbore. Artificial lift systems are then introduced to assist in lifting the fluids to the surface. The most widely used artificial lift methods include: Sucker-rod pumping, Gas lift (both continuous and intermittent), Electrical submersible pumping (ESP), and Hydraulic pumping. [1].

Secondary recovery involves the injection of gas or water into a reservoir when a significant portion of crude oil cannot be extracted through primary recovery methods alone. At this stage, additional energy must be supplied to sustain production. Most reservoirs contain gas in a miscible state—comparable to the dissolved gas in a sealed bottle of soda, which begins to escape once the cap is opened. During primary production, as reservoir pressure drops, the dissolved gas is released, further reducing the pressure. To restore or enhance the reservoir pressure, secondary recovery techniques are employed. This is typically achieved by injecting gas or water to replace the produced fluids and maintain or increase reservoir pressure. When gas is used alone, it is often injected at the top of the reservoir, where natural gas tends to accumulate, forming a gas cap. Gas injection can be particularly effective in reservoirs where gravity assists in driving the oil toward the wellbore. However, in cases where gravity segregation is insufficient, alternative recovery strategies must be considered. [1].

One of the most commonly applied secondary recovery techniques is water flooding. In this method, water is first treated to remove any impurities that could hinder its flow within the reservoir. It is then injected into selected wells within the oil field. As the water moves through the porous formation, it displaces and drives the oil toward the remaining production wells. The injection wells are strategically positioned in patterns optimized to maximize the displacement of oil toward producing wells.

Water flooding can significantly enhance oil recovery—often doubling the amount recovered compared to primary methods alone. In certain oil fields, such as the East Texas field, the reservoirs are naturally connected to large, active aquifers within the same formation. In these cases, maintaining reservoir pressure can be achieved simply by injecting water directly into the aquifer. Enhanced Oil Recovery (EOR) techniques are specifically designed to further increase the rate and efficiency of oil extraction. Since many EOR methods target the oil that remains after water flooding, they are commonly referred to as tertiary recovery methods. [1].

Enhanced Oil Recovery (EOR) techniques primarily involve the injection of gases, liquid chemicals, and/or the application of thermal energy to improve oil displacement and recovery. Commonly used gases in EOR operations include hydrocarbon gases, carbon dioxide (CO₂), nitrogen, and flue gases. In the context of this work, a gas-based method is classified as an EOR process if its effectiveness relies on mechanisms beyond simple immiscible frontal displacement, particularly those not dominated by high interfacial tension (IFT) flow characteristics. Various liquid chemicals are also employed in EOR, most notably polymers, surfactants, and hydrocarbon solvents, each contributing to enhanced fluid mobility and improved sweep efficiency.

Thermal EOR methods generally involve injecting steam or hot water into the reservoir or generating heat in situ through the combustion of oil within the reservoir rock. These techniques reduce oil viscosity and facilitate its movement toward production wells, thereby increasing recovery rates. [1].

For over 70 years, multiple observations have reported increased oil production in fields affected by earthquakes and similar seismic events. This phenomenon has drawn scientific interest to the concept of seismic stimulation as a potential technique for enhanced oil recovery [2]. Seismic stimulation aims to improve oil mobility by transmitting seismic waves into the reservoir to dislodge trapped oil and promote flow throughout the formation [3].

Although some analytical and numerical models have been developed to describe this process, experimental data remains limited. The underlying physics governing the interaction between seismic vibrations and multiphase fluid flow in porous media is still not fully understood.

In contrast to other fluid-based EOR techniques [4], seismic stimulation is less affected by geological complexities such as fault blockages, stratification, or phase heterogeneity. A key advantage of this method is that the induced elastic waves propagate effectively in both horizontal and vertical directions without significant attenuation. Additionally, some researchers have utilized multi-channel acoustic wave monitoring systems to investigate hydrate sand production [5], further supporting the applicability of wave-based technologies.

Seismic stimulation shows particular promise in reservoirs with multiple pay zones or significant faulting, and although it has so far been applied mainly alongside water flooding, a deeper understanding of its mechanisms may allow it to be integrated into various stages of the oil recovery process. Despite promising experimental trends, the fundamental physical mechanisms driving this production enhancement remain poorly defined. Continued research is necessary to advance this technique toward reliable, predictable, and field-applicable solutions. Some researchers propose that stress-wave stimulation could evolve into a cost-effective and practical oil recovery method for the oil and gas industry [6], offering an alternative to traditional mechanical or chemical treatments.

Figure (1.1): EOR methods.

  - Vibration Enhanced Oil Recovery

The seismic stimulation method is regarded as an alternative technique for Improved Oil Recovery (IOR) due to its relative simplicity of operation and low cost. It primarily involves various approaches to transmit wave energy from surface or subsurface sources to the underground fluids and rock formations within reservoirs. Four main types of vibration application are commonly utilized: (1) Injection of pulsed fluid from the borehole to generate seismic waves within the oil reservoir, where the fluid (e.g., water) is compressed and then abruptly released in a water hammer-like manner; (2) Use of a surface-based vibrator that generates seismic energy directed toward the oil-bearing zones, typically achieved by periodically striking a heavy weight (such as a hammer) on the ground surface; (3) Employment of sonic or ultrasonic generators to transmit vibro-energy directly into the reservoir; (4) Reactive explosions using chemical or nuclear methods conducted either downhole or at the surface.

For the pulsed fluid injection method, a cycle period—ranging from days to months—is generally required to ensure that vibration energy effectively reaches the oil-bearing zones. This vibration approach is closely associated with “pulsed” or “cyclic” water flooding processes, where water injection rates are varied periodically. The cycle duration typically spans days or months, thus the enhanced oil recovery observed may result from both the periodic variation in water injection rates and the seismic vibration effects [6].

  - Objective of this study

    - Investigate if the Vibration stimulation has the capability to shift the relative permeability curve.

    - Investigate ability to generate sufficient vibration energy by different injection patterns.

    - Create a vibration with low cost.

CHAPTER 2 LITERATURE REVIEWS

CHAPTER 2 LITERATURE REVIEWS

  - Enhanced oil recovery (EOR)

Enhanced Oil Recovery (EOR) refers to any reservoir technique employed to alter one or more properties within the reservoir. Such modifications may involve changes in the interactions between the displacing and displaced fluids, or between the displaced fluid and the reservoir rock, with the goal of increasing the recovery factor. These changes can include the reduction of interfacial tension, lowering of oil viscosity, enhancement of oil swelling, and alteration of reservoir rock wettability [1].

Figure (2.1): Flow Chart Enhanced Oil Recovery

  - Type Enhanced Oil Recovery (EOR)

- Gas Displacement.

- Thermal Recovery.

- Chemical Recovery.

- Microbial Recovery.

    - Gas Injection

Gas injection has been demonstrated to enhance crude oil recovery in reservoirs that have undergone partial or full water flooding. This technique typically involves injecting different types of gases to displace the residual oil within the reservoir. Conventionally, the process aims to reduce the interfacial tension (IFT) specific to each crude oil and gas combination, thereby achieving a miscible flooding mechanism.

Near-miscible gas flooding refers to the injection of gases that do not achieve complete miscibility with the oil but come very close. Examples include condensing–vaporizing gas drives operating at enrichments slightly below the minimum miscibility enrichment or at pressures marginally below the minimum miscibility pressure (MMP). Near-miscible gas flooding is attractive both economically and operationally. Injecting leaner gases—those with lower LPG or NGL content in dry gas—is less costly than injecting richer gases. Furthermore, operating at lower pressures reduces costs due to decreased gas density and lower compression requirements near the miscibility threshold.

Some reservoirs exist at pressures below the MMP, and others experience pressure declines during miscible flooding projects. In such cases, near-miscible operations may provide a more practical and feasible alternative to fully miscible flooding [7].

Gas Displacement classification into:

      - Immiscible Displacement

Injection of fluids immiscible with the oil displacement fluids, such as water and gases—including hydrocarbons, CO₂, nitrogen, and air—has been widely applied in enhanced oil recovery processes [8].

      - Miscible Displacement:

Injection of fluids that are miscible with the oil displacement fluids—such as hydrocarbon solvents, CO₂, flue gas, and nitrogen—plays a significant role in enhanced oil recovery operations [8].

Figure (2.2): Gas injection [9].

    - Thermal Recovery

Thermal recovery is a technique aimed at enhancing oil recovery by injecting heat into the reservoir, thereby reducing the oil’s viscosity or causing phase changes such as vaporization of the oil [10].

      - Hot – water flood.

      - Steam processes.

      - In situ combustion.

Figure (2.3): Schematic of heat exchange system [11].

    - Chemical recovery

Chemical flooding is an isothermal enhanced oil recovery (EOR) process primarily aimed at improving oil extraction by (1) reducing the mobility of the displacing fluid to achieve better mobility control, and/or (2) decreasing the interfacial tension between oil and water [12].

      - Polymer Flooding.

      - Alkaline Flooding.

      - Surfactant Flooding.

Figure (2.4): Chemical enhanced oil recovery [13]

    - Microbial Recovery

      - Acid production helps dissolve the rock matrix, thereby increasing both porosity and permeability Producing gas (basically similar with CO2 flood).

      - Gas production, which operates on principles similar to CO₂ flooding.

      - Producing surfactant basically similar with surfactant flood.

      - solvents are miscible with oil, reduce oil viscosity, and improve the mobility ratio.

      - Surfactant production, which functions similarly to surfactant flooding techniques.

      - Polymer production modifies water mobility by increasing water viscosity, thus effectively controlling water movement within the reservoir [8].

Figure (2.5): Microbial enhanced oil recovery [14].

    - Vibration Enhanced Oil Recovery

In waterflooded fields, wells are often abandoned due to high water-cut, even though significant amounts of oil remain trapped within the formation, rendering production uneconomical. Vibration stimulation emerges as a promising technique to enhance oil production and increase the ultimate economic recovery under such conditions.

The introduction of vibrational energy into the reservoir is believed to facilitate oil movement through several mechanisms: by reducing capillary forces, decreasing adhesion between the rock and fluids, or causing oil droplets to aggregate into “streams” that flow more easily with the water flood.

The significance of vibration stimulation lies in its potential to generate sufficient downhole vibrational energy to improve fluid flow characteristics—a concept that holds considerable interest. The economic potential of vibration stimulation for enhanced oil recovery is substantial. As reported by the Interstate Oil and Gas Compact Commission in 1995, “Immobile oil is held in reservoirs by viscous and capillary forces.” Conventional primary and secondary recovery methods can only recover a small fraction of this immobile oil. Therefore, the approximately 238 billion barrels of immobile oil reserves in the U.S. represent a critical target for enhanced oil recovery technologies [15]. Vibration stimulation has the capability to alter the relative permeability curve, thereby increasing the recovery of these immobile oil reserves as an effective enhanced oil recovery method.

2.4 Vibration Stimulation Historical Background

Russian Investigations: Interest in elastic-wave vibration stimulation—distinct from non-elastic vibrations such as explosions—dates back to the 1950s. This history is well documented in the work of Beresnev and Johnson, who reviewed extensive research conducted in both the USSR and the USA [16]. Their survey covers contributions from over a hundred researchers studying man-made vibrations across a broad frequency range, from ultrasonic frequencies around 5 MHz to barely audible seismic waves near 1 Hz, including those induced by traffic. While the effects of earthquakes on oil production were also examined, conventional and nuclear explosions were excluded from their review.

Most of the documented studies originated from four Soviet institutions, each producing numerous technical papers on laboratory and field investigations. These groups provided general descriptions of the mechanisms and effects of vibration on fluid-saturated porous media. Various theories have been proposed to explain observed changes in fluid flow, encompassing the influences of gravitational and capillary forces, alterations in the wetting phase and its thickness, modified relative permeability, coalescence or dispersion of oil droplets, the effect of connate water salinity, viscosity reduction, temperature increase, frequency and intensity dependencies, continuous or intermittent wave energy application, elastic versus nonelastic effects, resonant and dominant frequencies, and the effectiveness of surface versus downhole seismic stimulation. However, no consensus exists regarding the theoretical basis of elastic vibration effects on multiphase fluid flow in porous media.

V.N. Nikolaevsky and colleagues proposed that ultrasonic oscillations are generated by seismic waves [17]. Their in-situ measurements during surface vibro-stimulation indicated that seismic wave energy is converted to a dominant frequency independent of the source frequency. Their research suggests that vibrations can significantly influence the relative permeability of oil or water phases, partially restoring flow at saturations that would normally restrict phase movement.

Laboratory and pilot field tests of vibroseismic technology demonstrated both increased oil recovery and reduced water-oil ratio (WOR) [18]. Recent reports on the successful application of Soviet-developed seismic stimulation come from the vibro-seismic impact technology (VSIT), a surface stimulation method. V.N. Belonenko reported that elastic waves accelerate filtration processes, enhance accumulation of dispersed oil and gas bubbles, and speed gravitational segregation of gas, oil, and water [19, 20]. Field tests in Indonesia by Caltex Pacific Indonesia (CPI) recorded a cumulative incremental oil production increase of 20.2% over the production baseline [21]. CPI is planning further field studies of this promising enhanced oil recovery technique.

Chinese Studies: Recently, several papers from multiple Chinese institutes have reported laboratory research on sound vibration stimulation. These studies focused on the effects of vibration during core-flooding experiments [22–24]. Mingyuan demonstrated that sound vibration can alter the wettability of oil-saturated cores towards a more water-wet condition, thereby enhancing oil recovery through water flooding combined with sound vibration [25]. Additionally, Wenfei and colleagues proposed a theoretical model coupling wave propagation in porous media with artificial vibration [26]. This line of research, which requires further validation, aims to deepen the understanding of compression and shear wave propagation in both the solid (rock) and liquid (multiphase) components of porous media.

Canadian Heavy Oil Applications: While research on vibration stimulation progressed in various parts of the world, a Canadian research group made significant advances in another enhanced oil recovery (EOR) technique known as pressure pulsing. This discussion highlights the work of Spanos, Dusseault, and colleagues, who developed theoretical explanations for flow enhancements achieved through pressure pulsing [27–29].

They introduced the concept of porosity diffusion induced by low-frequency, high-amplitude pressure waves. Their study analyzed the Biot-Gaussman porous media model and derived the de la Cruz-Spanos porous media models by treating porosity as a dynamic variable.

Initially, this technique was applied to produce cold, heavy, and highly viscous oil from poorly consolidated, young sandstone reservoirs, where sand production was considered beneficial. The method, termed Cold Heavy Oil Production (CHOP), has been successfully implemented in heavy oil sands in Alberta and Saskatchewan, Canada. Several case studies report its effectiveness in reservoirs with higher gravity oils, improved oil production linked to acid diffusion during stimulation, and positive responses observed in carbonate formations.

Reported United States Research

Los Alamos National Laboratory Research: Inspired by the comprehensive review by Beresnev and Johnson [2], a joint industry project funded by the U.S. Department of Energy’s Office of Basic Energy Science, Division of Advanced Energy Projects, was initiated in 1995 by Peter M. Roberts at Los Alamos National Laboratory to investigate the fundamentals of vibration stimulation. This initiative evolved into the ongoing program titled “Seismic Stimulation for Enhanced Production of Oil Reservoirs” [30, 31].

This three-year partnership under the Natural Gas and Oil Technology Partnership, sponsored by DOE, involves collaboration with Lawrence Berkeley National Laboratory and the University of California at Berkeley. The project encompasses laboratory core-scale stimulation flow tests, monitoring of field tests conducted by industry partners, and development of theoretical models to describe the stimulated flow phenomena.

Laboratory experiments utilized a core test cell equipped with a magnetostrictive actuator to apply mechanical stress excitation to sandstone core samples under single-phase and two-phase fluid flow conditions. Several promising core tests have been performed [32]. Results indicate that mechanical stress excitation at frequencies of 100 Hz and below can significantly influence two-phase fluid flow in Berea sandstone, both under steady-state and simulated flood conditions. Preliminary analysis suggests that changes in wettability may be a dominant mechanism behind enhanced oil production. Additionally, in situ mobilization of clay fines was observed, leading to increased absolute permeability of the rock.

Currently, the project is monitoring industry-supported field stimulation tests in California’s Central Valley. Initial results show an increase in oil cut that correlates directly with the stimulation treatment. Downhole seismic and pressure data are being recorded to establish further correlations between wave field parameters and production enhancement.

Stanford University Research: As part of her doctoral thesis, Yan Pan investigated the use of intermediate frequency excitation for reservoir analysis [33]. The study developed five flow models incorporating Darcy’s law and Biot’s theory as special cases (models II and III, respectively). Although not focused explicitly on vibration stimulation, the research concluded that “elastic solid vibration positively affects fluid flow in porous media if the perturbation signal lies within the intermediate frequency range.” It further suggested that applying harmonic perturbations at optimal frequencies to reservoirs under suitable conditions could stimulate oil production.

CHAPTER 3 EXPERIMENT & EQUIPMETN

CHAPTER 3. Experimental & Procedures

3.1 EQUIPMENTS

- Digital Sound Level Meter: A Digital Sound Level Meter (DSLM) is a compact portable instrument, usually battery-powered, used to measure sound-pressure levels directly while incorporating frequency weighting networks that approximate the inverse of equal loudness contours at various levels. A sound level meter is also called a sound pressure level meter (SPL) [34]. This device used to measure that the vibration during the experimental.

Figure (3.1): Digital Sound Level Meter.

- Tubes with different length

Different tubes with different lengths (1 m, 1.5 m , 2 m ) has been collected, the lengths has been selected based on different pressure that need to applied during the experimental.

Figure (3.2): Pipes with different length.

- Piece of rubber

A piece from rubber was designed with two parallels channels covered with a piece of plastic in order to measure the vibration during the movement of the water.

Figure (3.3): Piece of rubber with two channels covered with a piece of plastic.

- Two bottles of water of 500 ml

Figure (3.4): Two bottles of water of 500 ml.

- Procedures

  - Run different well pattern by use CMG.

Different injection patters has been run in order to check weather the vibration can be created by water movement or not. A field-scale simulation model was built to investigate the vibration enhanced oil recovery performance. The simulation work was conducted by using CMG (computer management group ltd, Calgary, Canada). The compositional model was built by the CMG-IMX model. The CMG-WINPROP module was used for oil component lumping, according to the fluid properties of the target reservoir. To speed up the numerical simulation and ensure the accuracy of the calculation results

Based on the reservoir characteristics and properties, a model using the number of 21×21×3. The depth of the reservoir is about 2000 ft and water-oil contact about 2080 ft and the pinchout thickness is 0.0002ft, the lithology is mainly an average porosity

of 2%, and permeability I: 100 permeability J: 50 permeability K: Equals I * 0.1 . Rock compressibility (0.00125642409 1/psi) Pressure dependence of formation porosity / rock compressibility, Reference pressure for calculation of rock compressibility (1239.315 psi). Maximum time step size (150 day), Minimum time step size (0.001 day), Frist time step size after well change (1 day).

Table (3.1): Reservoir properties

    - The first scenario was conducted for the two injector and two produces. The position of the injectors and producers were opposite to each other as shown in the figure 3.5.

Figure (3.5): Three-dimensional finite differences grid.

    - The second scenario was conducted for the two injector and two produces in X direction as shown in the figure 3.6. The position of the injector were placed in front of other injector and producer were placed in front of other producer. All the data were the same for the first scenario as shown in the figure 3.5.

Figure (3.6): Three-dimensional finite differences grid.

  - Experimental Investigations

    - First scenario

Inject water from one direction with different lengths (1m, 1.5m and 2m) and measure the vibration that happened due to the water movements.

Figure (3.7): First scenario

Second scenario

Inject water from tow direction with different lengths (1m, 1.5m and 2m) and measure the vibration that happened due to the water movements.

Figure (3.8): Second scenario

Third Scenario

Injection water from two opposite direction with different lengths (1m, 1.5m and 2m) and shutdown the injection every 5 sec and then open it and measure the vibration that happened due to the water movements.

Figure (3.9): Third scenario

Fourth Scenario

Inject water from two barrel direction with different lengths (1m,1.5m and 2m) and shutdown the pipes every 5 sec and then open it and measure the vibration that happened due to the water movements.

Figure (3.10): Fourth Scenario

CHAPTER 4 RESULTS AND DISCUSSIONS

CHAPTER 4. RESULTS and DISCUSSIONS

  - Simulation Results

A 3 model has been built as shown in the chapter 3. The main purpose to run the CMG is to check the different injection pattern with same properties and the same distance will give the same recovery factory or not. Especially if the reservoir is homogenous. From our expectations that the different well locations for the same distance and the same reservoir properties will give the same recovery factor.

As shown in Figure 4.1, in simulating the first scenario, where the injection wells were opposite to the production wells, we injected water for the purpose of causing vibrations, including an increase in production. The recovery factor was 35 % and the water injection pattern were flow in the same directions into the producers well. Thus, no vibrations has been expected in this scenario.

Figure (4.1): First scenario oil recovery factor.

As Figure 4.2 shows when the simulated of the second scenario and changed the direction of an injection well with a production well while keeping the other wells as they were, and entered the same data as in the first scenario, so here in the normal condition and based on the material balance equation and based on the partial different equation for pressure difrential calculation, the recovery factor should be the same with the prevoius senario. But in this case the recovery factor was higher than the the fisrt senario, the gain was about 4 % from the first senario and that is reflect the effect of the vibration due to opposite movement of the inejction.

Figure (4.2): Second Scenario Oil Recovery Factor.

  - Experimental Results

Based on numerical investigation that has shown in the above, the experimental results has been investigate to check whether there is an vibration during the opposite directions of the movements. When we injection water in the first scenario from one direction at lengths of 1 m, 1.5 m, and 2 m at different hydrostatic pressure for each length, the following results has been obtained, which are the least results we got when doing the experiment. As shown in the results, the vibration is in direct proportional with the pressure, where the vibration increases with the increase of the pressure.

Table (4.1). Vibrations results when the injection from 1 side.

Vibration

Figure (4.3): Vibrations results when the injection from 1 side.

In the second scenario, the water injected from two direction at different side at lengths of 1 m, 1.5 m, and 2 m and with different continuous hydraulic pressure, for each length, the following results has been obtained, as shown in the results, the vibration is in direct proportional with the pressure, where the vibration increases with the increase of the pressure. The opposite direction has given higher vibration than the first scenario.

Table (4.2). Vibrations results when the injection from 2 different sides.

Vibration

Figure (4.4): Vibrations results when the injection from 2 different sides.

in the third scenario, the water was injected from two directions on different sides at lengths of 1m, 1.5m, and 2m, and alternately open and shut the injection every 5

seconds, while applying continuous hydraulic pressure at different levels for each length, the following results has been obtained, as shown in the results, the vibration is in direct proportional with the pressure, where the vibration increases with the increase of the pressure. The opposite direction with soaking alternating injection has given higher vibration than the one side and the opposite injection side.

Table (4.3). Vibrations results when the injection from 2 different sides with cycle injection.

Vibration

Figure (4.5): Vibrations results when the injection from 2 different sides with cycle injection.

In the fourth scenario, the water was injected from two directions on the same side at lengths of 1m, 1.5m, and 2m, and alternately opened and shut the pipe every 5

seconds, while applying continuous hydraulic pressure at different levels each length, the following results have been obtained, as shown in the results, the vibration is in direct proportional with the pressure, where the vibration increases with the increase of the pressure. The same direction with soaking alternating injection has lower vibration than the injection from 2 different sides with cycle injection.

Table (4.4). Vibrations results when the injection from 2 same sides with cycle injection.

Vibration

Figure (4.4): Vibrations results when the injection from 2 same sides with cycle injection.

4.3 Conclusion

- Vibration-enhanced oil recovery is a promising technology that has the potential to increase the recovery of oil from reservoirs.

- The application of vibrations, increases the permeability of the reservoir rock and improves the flow of oil.

- The application of vibrations, can lead to alter the wettability and initiate the movement of the crude oil.

References

[1]

D. W. a. G. P. W. Green, Enhanced oil recovery. Vol. 6. 1998: Henry L. and S. Doherty Memorial Fund of AIME, Society of Petroleum Engineers.…

[2]

S. Kostrov and W. Wooden, "Possible mechanisms and case studies for enhancement of oil recovery and production using in-situ seismic stimulation," in SPE Symposium on Improved Oil Recovery, 2008.

[3]

O. Kuznetsov, E. Simkin, G. Chilingar, M. Gorfunkel, and J. R. Jr, "Seismic techniques of enhanced oil recovery: experimental and field results," Energy Sources, vol. 24, pp. 877-889, 2002.

[4]

Q. Sun, Z. Li, S. Li, L. Jiang, J. Wang, and P. Wang, "Utilization of surfactant- stabilized foam for enhanced oil recovery by adding nanoparticles," Energy & Fuels, vol. 28, pp. 2384-2394, 2014.

[5]

C. Y. Ding J, Yan C ,et al. Experimental study of sand, control in a natural gas hydrate reservoir in the South China sea. Int J Hydrogen Energy 2019;44(42):23639e48.

[6]

Z. L. Pu C, Liu J. Innovations and challenges of vibration, coupled seepage mechanics in oil and gas reservoir development. Earth Sci 2017;42(8):1247e62.

[7]

P. a. A. M. McGuire, Compositional simulation and performance analysis of, the Prudhoe Bay miscible gas project. SPE reservoir engineering, 1992. 7(03): p-329 .

.334

[8]

R. E. Terry, J.B. Rogers, and B.C. Craft, Applied petroleum reservoir engineering. and P. Education.

[9]	https://www.corelab.com/ps/enhanced-oil-recovery-fluids, ed.

[10]

R. Schmidt, Thermal enhanced oil recovery current status and future needs., Chemical Engineering Progress;(USA), 1.(1)86 .990

[11]

Y. Zhu, et al., Geothermal power production from abandoned oil reservoirs using in situ combustion technology. Energies, 2019. 12(23): p. 4476.

[12]

J. J. Sheng, Modern chemical enhanced oil recovery: theory and practice. 2010:, Gulf Professional Publishing.

[13]	https://www.sciencedirect.com/science/article/abs/pii/S0306261919311547 .

[14]

R. a. I. M. B. Marchant, Microbial biosurfactants: challenges and, opportunities for future exploitation. Trends in biotechnology, 2012. 30(11): p.565-558 .

[15]

A. s. The Interstate Oil and Gas Compact Commission, Untapped Oil, Oklahoma City, OK (March 1995). .

[16]

I. A. e. a. E.-w. s. o. o. p. Beresnev, A review of methods and results,” Geophysics, (June1994) 59 ,No. 6, 1000.

[17]

V. N. e. a. R. O .R. R. W. Nikolaevskiy, Seismic Vibrations,” SPE Production & Facilities, (May 1996) 89.

[18]

O. L. e. a. I. o. r. b. a. Kouznetsova, of vibro-energy to waterflooded sandstones,” Journal of Petroleum Science and Engineering (1998), 19, 191.

[19]

V. N. e .a. V.-s. t. o. Belonenko, hydrocarbon yield and analogies of the Australian Pacific region and CIS,” Oil and Gas Australia (July 1996). .

[20]

V. N. V. S. T. f. I. Belonenko, Hydrocarbon Bed Recovery,” New Technologies for the 21st Century, Joint English/Russian Magazine (2000) 4, 14. .

[21]

V. S. I. t. t. I. O. Staff writer, Production on Baseline,” Petropages News (July 2000) 16.

[22]

Y. e. a. E. o. M. V. o. t. C. Ling, Pressure Curve and The Wettability of a Core,” Xi An Shi You Xue Yau Xue Bao) ,Sept. 1997) Vol. 12 No. 5, 23.

[23]

S. e. a. B. R. o. A. P. F. Changjin, to Increasing Crude Oil Production,” Shi You Xue Bao, (July) (1997) Vol. 18, No. 3, 63.

[24]

M. e. a. I. t. W. F. R. Jiangou, Efficiency of Cores by Mechanical Vibration,” Xi An Shi You Xue Yau Xue Bao, (July 1997) Vol. 12 No. 4, 19.

[25]

L. e. a. T. S. o. O. R. b. W. Mingyuan, Flooding with Sound Vibration,” Petroleum Science (March (1999) Vol. 2 No. 1, 48.

[26]

Z. e. a. C. W. P. M. T. Wenfei, Porous Media In Artificial Vibration Oil Producing,” Shi You Zuan Cai Gong Yi, (1997) Vol. 19 No. 1, 60.

[27]

T. J. T. e. a. P. P. a. t. R. S. A. Spanos, New EOR Approach,” presented at CIM Conference, Calgary ,Canada, (June 1999).

[28]

M. B. e. a. S. R. M. S. i. Dusseault, Heavy Oil Wells ”,presented at the 2000 SPE International Symposium on Formation Damage, Lafayette, LA (23-24 Feb.(2000).

[29]

M. B. e. a. P. P. T. U. a. D. o. Dusseault, Starting a New Technology,” Journal of Canadian Petroleum Technology, (April 2000) Vol. 39 No. 4.

[30]	A. e. a. S. S. o. O. P. i. Sharma, Mature Reservoirs,” Am. Assoc. Petrol. Geol. Annual

Convention, Extended Abstracts, (1998). Vol. 2: A591.

[31]

P. M. e. a. S. S. f. E. Roberts, Production of Oil Reservoirs,” NGOTP Project Reports ,

homepage: http//:www.ees4.lanl.gov/stimulation. .

[32]

P. M. e. a. L.-F. A. S. o. Roberts, Fluid Flow in Porous Media,” J. Acoust. Soc. Am., (1999) 105 ,2-Pt. 2: 1385.

[33]

Y. Pan, “Reservoir Analysis Using Intermediate Frequency Excitation,” presented as a dissertation for the Degree of Doctor of Philosophy, Stanford University (August 1999).

[34]

HBK. https://www.hbkworld.com/en/knowledge/resource-center/articles/2022/what- is-a-sound-level-meter#!ref_www.bksv.com.