State of Libya

Sabha University

Faculty of Engineering

Department of Renewable Energy

Project Proposal Entitled:

Study the Impact of Doubly-Fed Induction Generator wind turbines on voltage profile in transmission network

Students Name:

Adel abdulaali saleh albriky (22200095)

Saif al-lslam al-salihin mohammed (22160002)

Masouda ismail saad khalifa (22200015)

Supervised by

Dr.Ibrahim nasr

Academic Year

(2024-2025)

بسم الله الرحمن الرحيم

قال تعالى:

يَرۡفَعِ ٱللَّهُ ٱلَّذِينَ ءَامَنُواْ مِنكُمۡ وَٱلَّذِينَ أُوتُواْ ٱلۡعِلۡمَ دَرَجَٰتٖۚ وَٱللَّهُ بِمَا تَعۡمَلُونَ خَبِيرٞ (11)

Table of contents

Abstract

Chapter 1 Introduction

1-Introduction	1

1.2-Project Objectives	1

1.3-Literature Review	2

1.4-Impact of wind generation	4

Chapter 2  Electrical Power System Structure and Wind Generator Characteristics

2.1 Electrical Power System	6

2.1.1 Power Flow	8

2.1.2 Classification of Busbars	9

2.2 Wind Generation Development	10

2.3-Wind Turbine Operation Characteristics	13

2.4-Types of Wind Generators	15

2.4.1- Squirrel Cage Induction Generator (SCIG):	16

2.4.2 Doubly-Fed Induction Generator (DFIG):	17

2.5 Requirements for Wind Farm Integration	17

2.5.1 Voltage and Reactive Power Control	18

2.5.2 Voltage Level	19

2.6-Reactive Power Support from the DFIG	21

Chapter 3 TEST SYSTEM AND SIMULATION PROCEDURE…………..

3.1-TEST SYSTEM AND SIMULATION PROCEDURE	24

3.2-RESULTS AND DISCUSSIONS	28

3.2.1-Voltage Profile Analysis – Base Case (Normal Load)	28

3.2.1.1 Voltage Profile Analysis with DFIG – Base Case (Normal Load) at Busbar 14	29

3.2.1.2-Voltage Profile Analysis with DFIG – Base Case (Normal Load) at Busbar 9	31

3.2.1.3-Voltage Profile Analysis with DFIG – Base Case (Normal Load) at Busbar 9 were connected to Busbar 9 and Busbar 28 simultaneously	33

3.2.2-Voltage Profile Analysis – Load Demand increase by 50%	35

3.2.2.1-Voltage Profile Analysis with DFIG – Load Demand increase by 50% at Busbar 14	37

3.2.2.2-Voltage Profile Analysis with DFIG – Load Demand increase by 50% at busbar 9	40

3.2.2.3-Voltage Profile Analysis with DFIG – Load Demand increase by 50% were connected to Busbar 9 and Busbar 28 simultaneously	41

Conclusion	45

Recommendations:	46

References	49

Appendix A: IEEE 30-Busbar Test System	51

list of figure

Figure 2.1: Global cumulative installed wind capacity 2011 2024……………….12

Figure 2.2: The Power Coefficients-Tip Speed Ratio Performance Curve for Different Pitch Angles ……………………………………………………………………….15

Figure 2.3: Wind Turbine Output Curve………………………………………….15

Figure 2.4: Schematic Diagram of a Fixed-Speed Wind Turbine………………...16

Figure ‎2.5: Fixed Speed Wind Turbine Generator………………………………..17

Figure 2.6: Requirements for power factor variation range in relation to the voltage according to UK code………………………………………………………………21

Figure 2.7: P-Q Characteristic of the DFIG……………………………………….23

Figure 3.1 IEEE-30 Busbar system…………………………………………………....27

Figure 3.2: Voltage Profile (p.u) Base Case…………………………………….....29

Figure 3.3: Voltage Profile (p.u) Normal Load at Busbar 14………………………....31

Figure 3.4: Voltage Profile (p.u)-Normal Load at Busbar 9………………………..…33

Figure 3.5: Voltage Profile (p.u) -Normal Load at Busbar 9 and 28…………………..35

Figure 3.6: Voltage Profile (p u) 50% Load…………………………………….…37

Figure 3.7: Voltage Profile (p.u) 50% Load at Busbar 14……………………………..39

Figure 3.8: Voltage Profile (p.u) 50% Load at Busbar 9………………………………41

Figure 3.9: Voltage Profile (p.u) 50% Load at Busbar 9 and 28………………………43

List of Table

Table 2.1: Top 10 countries installed wind capacities	13

Table 3.1: The 10%   and 20% wind penetration level with M var of wind generator	25

Table 3.2 The 20% wind penetration level and different locations	26

Table 3.3: Bus Voltage Profile (p.u) Base Case	30

Table 3.4: Bus Voltage Profile (p.u) Normal Load at Busbar 14	32

Table 3.5: Bus Voltage Profile (p.u) – Normal Load at Busbar 9	34

Table 3.6: Bus Voltage Profile (p.u) – Normal Load at Busbar 9 and 28	36

Table 3.7: Bus Voltage Profile (p.u) 50% Load	38

Table 3.8: Bus Voltage Profile (p.u) 50% Load at Busbar 14	40

Table 3.9: Bus Voltage Profile (p.u) 50% Load at Busbar 9	42

Table 3.10: Bus Voltage Profile (p.u) 50% Load at Busbar 9 and 28	44

Abstract

Recently, the increasing interest in producing electricity using renewable resources is growing rapidly due to the ability of these resources to reduce greenhouse gases. As the use of wind generation in power systems is increasing rapidly it is having a more noticeable impact on the power systems operate. An important issue in the integration of large-scale wind farms is the impact of power systems on voltage profile. This project investigates the impact of doubly-fed induction generator on system voltage profile in transmission network.

The IEEE 30-Busbar system is used as a test case, and simulation is carried out using the Power World Simulator. Wind generation is connected to the transmission network at different buses, with different wind penetration level. The penetration level is varied from 10% to 20% to assess the impact on the system voltage profile. The results of this study show that system voltage profile is affected positively or negatively depending on the penetration level, location of wind generation connection, and type of wind generator.

# Chapter 1

# Introduction

1-Introduction

The increasing integration of renewable energy sources particularly wind power into modern transmission systems has brought significant opportunities and challenges for power system operators. The United Kingdom's grid for instance is undergoing substantial upgrades to ensure a reliable and continuous supply of electricity to millions of households and businesses. With government incentives promoting sustainability new technologies such as wind farms are being encouraged to support the transition toward a cleaner energy sector. As wind farms become major power injectors understanding their impact on the voltage profile of transmission systems is crucial for maintaining grid stability and reliability  [1]

Among the various wind turbine generator technologies, the Doubly Fed Induction Generator (DFIG) has gained prominence due to its operational flexibility ability to control both active and reactive power and cost-effectiveness .[2].The integration of DFIG-based wind farms introduces new dynamics into the grid particularly concerning voltage control and stability. This chapter introduces the background, objectives and significance of studying the impact of DFIG on the voltage profile in transmission systems and provides a review of relevant literature.

1.2-Project Objectives

The main objectives of this study are as follows:

- To study the impact of integrating wind power generation on voltage profiles within the electrical transmission networks.

- To analyze how voltage profiles can be improved when wind farms are connected to the power system.

1.3-Literature Review

In this section, we will review the most prominent previous studies that have investigated the impact of integrating wind power and DFIG generators on the voltage profile in electrical transmission systems. Through these studies, we present the main findings and recommendations reached by researchers in this field focusing on wind power penetration levels, reactive power requirements and the effect of wind farm distribution on voltage stability and network efficiency. This review aims to provide a comprehensive scientific background that supports the objectives of the current research and contributes to understanding the challenges and opportunities associated with integrating renewable energy sources into modern power grids.

Yang et al. (2008) analyzed the effects of different wind power penetration levels and distributions on the voltage profile and reactive power requirements in a modified IEEE 30-busbar system. The results indicated that integrating wind farms does not necessarily improve the overall system voltage profile, and the impact depends largely on the location and penetration level. Proper site selection can reduce reactive power losses and improve the voltage profile [8]

Jin and Huang (2014) proposed a novel voltage control approach for DFIG systems based on dynamically adjusted PI controller gains through a self-learning mechanism that adapts to real-time system sensitivity. Simulation results indicated that this approach enhances the ability of DFIG controllers to maintain terminal voltage under varying operating conditions. The study also reviewed advanced tuning techniques such as particle swarm optimization for improved voltage regulation and system stability in the presence of DFIG-based wind generation [7].

Elzemeity (2019) studied the impact of DFIG generator location and penetration level on system stability and voltage profile using eigenvalue analysis and PV curve evaluation. The results showed that increasing DFIG penetration levels led to a reduction in the voltage collapse margin at critical Busbars. The study emphasized the need for additional reactive power compensation, such as STATCOM

Liang et al. (2022) investigated the dynamic performance of DFIG networks under fault conditions, focusing on the role of voltage source converters and STATCOM devices in mitigating voltage drops. The study showed that using STATCOM during faults significantly improved voltage profiles at critical busbars, reduced current surges and stabilized the electromagnetic torque of DFIG generators, thus enhancing system resilience during disturbances [5].

devices to restore voltage stability margins at high penetration levels [6]

Huang et al. (2023) presented a recent study on the impact of DFIG generators on voltage stability in large-scale transmission systems. The researchers used an adaptive control model that incorporates deep learning algorithms to enhance the DFIG response to disturbances. The results demonstrated that the proposed model improved voltage stability margins by up to 22% compared to traditional control systems, especially at high penetration levels reaching 45% of system capacity [4].

Zhang et al. (2024) proposed a new integrated control framework for DFIG-based wind farms to improve the voltage profile in transmission networks. This framework combines local and system-level control using artificial intelligence techniques. Simulation tests on the IEEE 118-Busbar system showed that the proposed frame work maintained voltage profile stability even at high penetration levels up to 50%, and reduced reactive power losses by 18% compared to conventional control systems [3]

1.4-Impact of wind generation

The integration of wind generation into modern transmission systems has become a central focus for grid operators and policymakers driven by the need to reduce greenhouse gas emissions and diversify energy sources. Wind power offers significant environmental and economic benefits but its variable and intermittent nature introduces new operational challenges for transmission networks. One of the most prominent impacts is on system stability as fluctuations in wind speed can cause voltage and frequency variations, requiring advanced control strategies and additional resources to maintain reliable operation. Studies have shown that high levels of wind penetration may lead to congestion in existing transmission lines, necessitating infrastructure upgrades or the construction of new lines, especially since wind farms are often located in remote areas far from demand centers. Integrating wind power can affect the system’s reactive power balance, potentially impacting voltage profiles and increasing the need for compensation devices such as STATCOMs or capacitor banks. Despite these challenges, advances in wind turbine technologies such as the Doubly Fed Induction Generator (DFIG) have improved the ability of wind farms to support grid stability by enabling better control of active and reactive power. Overall, the successful integration of wind generation requires coordinated planning, investment in grid infrastructure, and the adoption of advanced control and monitoring systems to ensure both reliability and efficiency in modern power systems [9].

# Chapter 2

Electrical Power System Structure and Wind Generator Characteristics

2.1 Electrical Power System

Over the years, the electrical power system has evolved into a highly interconnected network composed of numerous generators and transmission lines supplying power to various types of loads. To ensure the system remains stable, it is essential to keep generation and demand in constant balance, while also accounting for system losses[10].

In recently, power systems have become more complex. This complexity stems from increased interconnections, many of which are operating close to their transmission stability limits due to economic and environmental constraints. Furthermore, the growing integration of renewable energy sources—particularly those connected through power electronic interfaces has introduced new dynamic characteristics that differ from those of conventional synchronous machines[10]. A traditional power system typically comprises three key segments: generation, transmission, and distribution.

The global demand for electrical energy continues to rise each year. Historically, fossil fuels such as coal, oil, and natural gas have been the dominant sources used to meet this demand. These fuels generate heat that is converted into mechanical energy via steam turbines, which in turn drive electricity generators. Nuclear energy follows a similar principle, while hydroelectric power converts gravitational potential energy from stored water into mechanical energy using turbines.[11]

However, reliance on fossil fuels has serious environmental consequences, notably the emission of pollutants like CO₂, SOₓ, and NOₓ. As a result, there is growing interest in renewable, cleaner sources such as wind and solar power. Wind energy, in particular, has gained significant attention for its sustainability and potential to reduce emissions[12]. Despite the increasing role of renewables, conventional generation sources are still essential—and may become even more so—to ensure a reliable and stable electricity supply.

The transmission system is responsible for carrying power from generation sites to load centers using high-voltage AC or DC networks. Step-down transformers connect these networks to distribution systems, which operate at medium or low voltage levels to deliver electricity over shorter distances to consumers. Traditionally, distribution systems were passive networks without local generation. Today, however, with more renewable generators being connected directly at the distribution level—ranging from a few kilowatts to several megawatts—the distribution grid has become more dynamic and active, altering the direction and nature of power flow.

Electricity demand tends to follow a daily, weekly, and seasonal cycle. Peak loads can be twice as high as the minimum (base) load, which necessitates sufficient generation capacity to handle fluctuations. The core goal of any power system is to meet demand at the lowest possible cost while ensuring reliability and stability in terms of voltage and frequency.

To meet varying load demands efficiently, power companies rely on two key operational strategies: economic dispatch (ED) and unit commitment (UC). ED involves determining the optimal power output of each generating unit at a given moment to minimize cost, while UC decides when to start up or shut down generators based on forecasted demand. These strategies are particularly challenged by wind power's unpredictability and variability, which make planning more complex.

2.1.1 Power Flow

Power flow analysis—also known as load flow analysis—is a critical tool used in the planning, operation, and optimization of electrical power systems. It provides engineers with essential information about the steady-state behavior of the system, particularly how voltage, current, and power are distributed across the network under various loading conditions[13].

The main goal of a power flow study is to determine the voltages at each Busbar in the system, the power flowing through each transmission line, and the active and reactive power supplied by each generator. This analysis helps identify overloaded lines, voltage drops, and areas where voltage levels might exceed acceptable limits.

To perform a power flow study, a mathematical model of the power system is developed based on network topology and component parameters. The system is represented by a set of nonlinear algebraic equations derived from Kirchhoff’s laws. These equations are then solved using numerical methods—such as the Gauss-Seidel method, Newton-Raphson method, or Fast Decoupled Load Flow (FDLF)—depending on the desired speed and accuracy.

Power flow results are crucial for:

- Evaluating the impact of new generation sources or load additions.

- Planning transmission expansions.

- Setting operational limits to prevent voltage instability.

- Analyzing contingencies, such as line outages or generator failures[13].

With the growing integration of renewable energy sources, especially intermittent ones like wind and solar, power flow analysis becomes even more important. It helps in assessing how these new sources affect system stability, voltage regulation, and reactive power management[13].

2.1.2 Classification of Busbars

In power system analysis, Busbars (or nodes) are essential points in the network where elements like generators, loads, or transmission lines are connected. Each Busbar is characterized by four key electrical quantities: voltage magnitude, voltage phase angle, active power (P), and reactive power (Q). Depending on which quantities are known and which need to be calculated[13],

Busbars are categorized into three main types:

1. Load Busbar (PQ Busbar):

Also known as a demand Busbar, this type represents Busbars where loads are connected. At a PQ Busbar, the active and reactive power demands are specified, while the voltage magnitude and angle are unknown and must be determined through power flow analysis. Most Busbars in a system fall into this category[4].

2. Generator Busbar (P-V Busbar):

This type of Busbar represents points where generators are connected and are often referred to as voltage-controlled busbars. At a P-V Busbar, the active power output and voltage magnitude are specified. The reactive power and voltage angle are calculated. These Busbars play a key role in voltage regulation across the system.

3. Slack Busbar (Swing Busbar):

The slack Busbar serves as a reference point for voltage angle (usually set to zero degrees) and balances the system’s power. Since power losses in the system are not known in advance, the slack Busbar adjusts its active and reactive power outputs to ensure overall power balance. Only one slack Busbar is defined in any power system study[13].

Proper classification of Busbars is critical for accurate and reliable power flow calculations. Each type of busbar plays a specific role in maintaining system balance, voltage levels, and operational efficiency

2.2 Wind Generation Development

The use of wind energy for electricity generation has seen rapid growth over the past few decades. Initially, wind power was limited to small-scale applications in remote areas or for individual use. However, advancements in technology, growing environmental concerns, and the push for sustainable energy sources have driven the expansion of large-scale wind farms connected to national power grids[14].

Wind energy is considered one of the most promising renewable sources [15] due to its abundance, cleanliness, and the fact that it doesn’t emit greenhouse gases during operation. Governments and energy companies around the world are investing heavily in wind power as part of their strategies to reduce carbon emissions and diversify energy sources.

The development of wind generation technology has focused on improving the efficiency, reliability, and capacity of wind turbines. Early designs were relatively small and operated at fixed speeds. Modern wind turbines, however, are much larger, can operate at variable speeds, and are equipped with advanced control systems and power electronics. These improvements allow them to produce more energy, adapt better to changing wind conditions, and integrate more smoothly into existing power systems.

In addition, offshore wind farms have become a growing trend due to stronger and more consistent wind speeds at sea, as well as reduced land use conflicts. Offshore projects, while more expensive and technically challenging, offer significant potential for large-scale clean energy production.

Despite the many benefits, wind generation poses certain challenges. Its intermittent and unpredictable nature can complicate system planning and stability. Therefore, integrating wind energy into the grid requires careful consideration of voltage control, reactive power support, and system flexibility.

Wind power has grown substantially from 239,000 MW in 2011 to 1.174000 MW in 2024 [16], as shown in Figure(2.1)  Figure 2.1: Global cumulative installed wind capacity 2011-2024

| Country | Installed Capacity (MW) | % of world Total |
| --- | --- | --- |
| China | 430,000 | 36.6% |
| United States | 151,000 | 12.9% |
| Germany | 69,000 | 5.9% |
| India | 46,000 | 3.9% |
| Spain | 31,000 | 2.6% |
| United Kingdom | 30,000 | 2.6% |
| Brazil | 26,000 | 2.2% |
| France | 22,000 | 1.9% |
| Sweden | 13,000 | 1.1% |
| Canada | 13,000 | 1.1% |
| Rest of the world | 343.000 | 29.2% |
| World Total | 1,174,000 | 100% |

Table 2.1: Top 10 countries installed wind capacities 2024

2.3-Wind Turbine Operation Characteristics

A wind is a machine that converts the wind's kinetic energy into rotary mechanical energy, which is then used to do work. In more advanced models, the rotational energy is converted into electricity, the most versatile form of energy, using a generator. The active power available from a wind generator is given by the equation(2.1) below [14]:

PWG=12ρ.A.Cpvw3                                            (2.1)

Where PWG  is the power output from the wind (watt),  ρ  is the air density(kg/m3),

A is the turbine swept area(m2), Cp is the coefficient of performance of the turbine(Dimensionless), and v is the wind speed(m/s)[14].

The value of  Cp can be determined from the following equation (2.2)

Cp=12λ-0.022β2-5.6e-0.17λ                   (2.2)

Where λ is the tip-speed(dimensionless) and β is the blade pitch angle (degrees).

The  Cp is not a constant for a given air foil, but rather is dependent on a parameter λ called the tip-speed ratio.

Figure 2.2: The Power Coefficients-Tip Speed Ratio Performance Curve for Different Pitch Angles.

Figure 2.3: Wind Turbine Output Curve.

2.4-Types of Wind Generators

Wind turbines convert kinetic energy from the wind into mechanical energy, which is then transformed into electrical energy through a generator. Over time, various types of wind generators have been developed to suit different operational needs and system requirements. These generators are generally classified based on the configuration of the generator and how it connects to the electrical grid. The most common types include:

2.4.1- Squirrel Cage Induction Generator (SCIG):

This is one of the simplest and most traditional types of wind generators. It operates at a fixed speed and requires reactive power support from the grid or external capacitors. SCIG-based turbines are easy to maintain and cost-effective, but they offer limited control over power output and grid interaction, making them less suitable for modern, flexible grid requirements[14].

Figure 2.4: Schematic Diagram of a Fixed-Speed Wind Turbine

2.4.2 Doubly-Fed Induction Generator (DFIG):

DFIG systems are among the most widely used in modern wind turbines. They allow variable-speed operation, which improves energy capture efficiency and provides better control over active and reactive power. The rotor of the DFIG is connected to the grid through a partial-scale power electronic converter, which enables independent control of generator speed and voltage. This makes DFIGs highly effective for grid integration and voltage support[15].

Figure 2.5: Fixed Speed Wind Turbine Generator

2.5 Requirements for Wind Farm Integration

As wind power becomes an increasingly important part of the global energy mix, integrating wind farms into existing power systems has introduced new technical challenges. Unlike conventional power plants, wind farms are highly variable and dependent on weather conditions, which affects their ability to deliver consistent power. To ensure stable and reliable grid operation, several key requirements must be met when connecting wind farms to the transmission system.

Successful integration depends on a combination of advanced technologies, grid support functionalities, and strict adherence to regulatory and technical standards. Among the most critical requirements are the ability to control voltage and reactive power, and to maintain acceptable voltage levels across the network[17].

2.5.1 Voltage and Reactive Power Control

Voltage control is a fundamental requirement for any power system, and wind farms are no exception. When connected to the grid, wind turbines must be able to contribute to voltage regulation in their local area. This is achieved through reactive power control, which involves supplying or absorbing reactive power as needed to maintain voltage within acceptable limits.

Modern wind turbines, especially those equipped with Doubly-Fed Induction Generators (DFIG) or full converter-based systems, are capable of dynamic reactive power control. These systems can respond quickly to grid demands and help support voltage stability during normal operation and fault conditions.

Grid codes in many countries now require wind farms to have reactive power capabilities similar to conventional generators. This includes maintaining a specific power factor, participating in voltage regulation, and providing fault ride-through (FRT) capability to stay connected during voltage dips.

Wind farms are required to be capable of regulating reactive power at their terminal in order to control the voltage [6] at the point of common coupling (PCC).

- A wind farm must be able to supply rated MW output at any point between the limits 0.95 lagging and 0.95 leading at the point common coupling (PCC).

- With all plants in service, the reactive power limits defined at rated MW at lagging power factor will apply at all active power output levels above 20% of the rated MW output.

- With all plants in service, the reactive power limits defined at MW at leading power factor will be applied at all active power output levels above 50% of the rated MW output. These reactive power limits will reduce linearly below 50% of active power output unless the requirement to maintain the reactive power limits defined at rated MW at leading power factor down to 20% active power output.

2.5.2 Voltage Level

Another key aspect of wind farm integration is the voltage level at the point of connection. Wind farms can be connected at different voltage levels—distribution, sub-transmission, or transmission—depending on their size and location. Large-scale wind farms are typically connected at higher voltage levels (e.g., 110 kV or above) to minimize losses and ensure efficient power transfer over long distances.

Choosing the appropriate voltage level is crucial for maintaining system performance, minimizing power losses, and ensuring compliance with grid standards. It also affects the design of transformers, protection systems, and control equipment required for the interconnection.

In some cases, especially for offshore wind farms, voltage levels are stepped up using offshore substations before transmitting power to the mainland grid. High-voltage direct current (HVDC) transmission may also be used for very long-distance power delivery, offering improved efficiency and control.

During standard operating conditions, wind farms are expected to either absorb or deliver reactive power based on the voltage level at the connection point. For high-voltage transmission levels such as 400 kV, this requirement is strictly enforced. Similarly, specific reactive power handling expectations exist for voltages at 275 kV and 132 kV. However, these guidelines tend to be more flexible at lower voltages like 33 kV, where the acceptable power factor range is outlined in Figure 2.6 which shows the UK Grid Code’s tolerance bands for reactive power behavior depending on the voltage level.

Figure 2.6: Requirements for power factor variation range in relation to the voltage according to UK codes.

2.6-Reactive Power Support from the DFIG

Doubly Fed Induction Generators (DFIGs) are equipped with voltage source converters [8] that enable flexible control over reactive power. Specifically, converter C1 is capable of generating reactive power, while converter C2 can absorb it. Together, they function as sources or sinks of reactive power to help regulate voltage at the grid connection point[15].

Figure 2.7 illustrates a typical P-Q (active-reactive power) characteristic of a DFIG-based variable-speed wind turbine. The extent of reactive power that a DFIG can provide is constrained by the capabilities of these converters, particularly their current ratings and the magnetizing current required by the induction machine.

Figure 2.7: P-Q Characteristic of the DFIG.

From this characteristic, two notable observations can be made:

1. Greater absorption than generation: The wind turbine generator can absorb more reactive power in the under-excited mode than it can generate in the over-excited mode.

2. Support without active power: Even when the wind turbine is not generating active power—for instance, during periods of low wind speed—it can still supply or absorb reactive power, provided the converter is active and connected to the grid.

While DFIGs do have some inherent capability to support reactive power, they may not always be sufficient to maintain voltage stability at the point of common coupling, especially when wind generation is highly variable. To comply with grid interconnection requirements particularly for large wind farms it is often necessary to install external reactive compensation equipment. Devices such as Flexible AC Transmission Systems (FACTS), Static Var Compensators (SVCs), and Static Synchronous Compensators (STATCOMs) are commonly used to provide additional support[15].

Chapter 3

TEST SYSTEM AND SIMULATION PROCEDURE

3.1-TEST SYSTEM AND SIMULATION PROCEDURE

This thesis investigates the impact of doubly-fed induction generator (DFIG) on voltage profile in transmission system when the reactive power limitation of wind generators is taken into consideration. The modified IEEE30-busbar simulated using Power World Simulator.

The test system was modified by connecting wind generation to the system at different busbars, with different connection scenarios at different wind penetration levels, and the farm consisted of several variable speed doubly fed induction generator (DFIG) wind turbines, each of 1.5 MW. In this assessment, the DFIG was modelled as a PV busbar and operated with maximum and minimum power factors of 0.95 leading (capacitive VAr) and 0.95 lagging (inductive VAr). The results of this study show that system voltage profile is affected positively or negatively depending on the penetration level, location of wind generation connection., and the test system was modified by connecting one wind generation to busbar 14, and on a first connection scenarios, and second connection scenarios wind generation was connected to bus 9 with different penetration level as shown in Table 5.1The single line diagram of the IEEE-30 bus system is shown in Figure3.1 The total generation capacity was 728.4 MW and the system peak load was 637.65 MW. Detailed data about the system are shown in Appendix [104]. AWind farms are connected to the transmission system at busbar 9 as first connection scenarios and second scenarios busbar 14, at two different scenarios, third connection scenarios was two locations as a combination with wind penetration levels at Busbar 9 and Busbar 28 at different MW outputs. The different connection scenarios of wind generation are:

- First scenarios:   one wind farm connected at busbar 14 as weak bus.

- Second scenarios:   one wind farm connected at busbar 9 as strong bus.

- Third scenarios:   two locations as a combination wind farms were connected to busbar 9 and busbar 28 simultaneously

The penetration level of the wind generator is determined as a percentage of the capacity of the conventional generators in the network [3.1].

Figure 3.1 IEEE-30 busbar system

The reactive power generation of the wind turbine is determined according to the power factor operating limits of 0.95, based on the following equation.

P.F=PS                                        (3.1)

Equation 3.1: power factor correction

S2=P2+Q2                                   (3.2)

Equation 3.2: power triangle equation

Where P.F is the power factor and P is the active power (Watt).

The S is apparent power (VA) and Q is reactive power (VAR)

.

Table 3.1: The 10%   and 20% wind penetration level with M var of wind generator

| Wind Penetration Level (%) | Wind Generator Reactive power± Q |
| --- | --- |
| 10% (72.84 MW) | ± 23.94 M var |
| 20% (145.68 MW) | ± 47.882 M var |

In the Third connection scenario, the wind farm is connected to busbars 9 and 28 simultaneously with a combination wind penetration level. The simulation will be processed under 10% and 20% penetration level of wind generation. Table 3. shows the wind penetration level and locations connected at these busbars.

Table 3.2 The 20% wind penetration level and different locations

| % Wind Penetration (WG9) | % Wind Penetration (WG28) | % of Combination Wind Penetration |
| --- | --- | --- |
|  |  | (WG9, WG28) |
| 10% (72.84 MW) 20% (145.68 MW) | 10% (72.84 MW) 20% (145.68 MW) | 5% (36.42 MW), 5% (36.42 MW) 10 % (72.84 MW), 10% 72.84 MW) |

the simulation was carried out by connecting DFIG at busbar 9 and busbar 14 separately then the third connection scenarios was two locations as a combination with wind penetration levels at busbar 9 and busbar 28.

Same data for wind penetration level and system demand were used so as to

compare the effect of   DFIG on the voltage profile at different locations in the transmission system.

There are two system demand scenarios used in this investigation for all penetration levels. The system demand scenarios are base case (normal loads) as first scenario, the system demand increased by 50% as second scenario. In this study assume that the output of DFIG is maximum value of each wind penetration level. The modified test system is analyzed using an AC Optimal Power Flow (OPF). The same load profile is used for all wind penetration levels and location, to enable us compare the results

3.2-RESULTS AND DISCUSSIONS

3.2.1-Voltage Profile Analysis – Base Case (Normal Load)

The results for the system voltage profile under wind penetration and dispersion are recorded and discussed. This voltage profile represents the initial operating condition of the 30-busbar system under normal load without wind generation. It can be seen from Figure 3.2 and table [3.3] that the voltages across most buses remain between 0.95 and 1.00 p.u, which is relatively stable.

Figure 3.2: Voltage Profile (p.u) Base Case

Table 3.3: Busbar Voltage Profile (p.u) Base Case

3.2.1.1 Voltage Profile Analysis with DFIG – Base Case (Normal Load) at busbar 14

When the DFIG is connected at weak busbar 14 and the wind penetration level increases from 10% to 20%, the voltage magnitudes of the system are decreased more for 20% wind penetration level as shown in Figure[3.3].

The wind farm does not improve the voltage profile and has a negative effect on voltage magnitudes of the network when the DFIG is connected at weak busbar 14. This low voltage problem can be solved by introducing adequate reactive power compensation in the network

Figure 3.3: Voltage Profile (p.u) Normal Load at Busbar 14

Table 3.4: Busbar Voltage Profile (p.u) Normal Load at Busbar 14

3.2.1.2-Voltage Profile Analysis with DFIG – Base Case (Normal Load) at busbar 9

However, when the wind generation is connected to a strong busbar 9, for all penetration levels. From the figure (3.4) we can see that the busbar voltage magnitude is increased due to inject both real and reactive power from DFIG into the network for both 10% to 20%, penetration levels compared to the base case with (no wind)

Figure 3.4: Voltage Profile (p.u)-Normal Load at busbar 9

Table 3.5: Busbar Voltage Profile (p.u) – Normal Load at Busbar 9

3.2.1.3-Voltage Profile Analysis with DFIG – Base Case (Normal Load) at Busbar 9 were connected to Busbar 9 and Busbar 28 simultaneously

When the DFIG is connected at busbars 9 and 28 and the wind penetration level increases from 10% to 20%, we can see in figure (3.5)   that the busbar voltage magnitude is close to the scenario of busbar 9, but there is a greater improvement in the distant busbars from bus 23 to 30 due to the injection of both active and reactive power in a distributed manner from the DFIG into the network for both penetration levels from 10 to 20 compared to the base case.

Figure 3.5: Voltage Profile (p.u) -Normal Load at busbar 9 and 28

Table 3.6: Busbar Voltage Profile (p.u) – Normal Load at Busbar 9 and 28

3.2.2-Voltage Profile Analysis – Load Demand increase by 50%

As the all load demands increased by 50%, some of the busbars voltage magnitude are decreased at without wind`. It can be seen from the table (3.7) that the busbar voltage magnitude of bus 21 is decreased to 0.92 p.u and mostly of busbars voltage magnitude decreasing due to increase the load demands.

Figure 3.6: Voltage Profile (p u) 50% Load

Table 3.7: Busbar Voltage Profile (p.u) 50% Load

3.2.2.1-Voltage Profile Analysis with DFIG – Load Demand increase by 50% at busbar 14

As the DFIG is connected to weak bus 14, bus voltage magnitude of some buses are deceased decreased for both penetration levels compared to the base case (no wind) and there are more low voltage on some busbars at 20% wind Penetration level.

the DFIG with voltage control mode might weaken the system voltage profile when DFIG is connected to weak busbar 14, according to the voltage profile point when increasing loading compared to the base case (no wind), as shown in Figures (3.7)

This means that the DFIG based wind farm with voltage controlled mode has a negative effect on system voltage magnitude when the DFIG is connected to a weak

Figure 3.7: Voltage Profile (p.u) 50% Load at busbar 14

Table 3.8: Busbar Voltage Profile (p.u) 50% Load at busbar 14

3.2.2.2-Voltage Profile Analysis with DFIG – Load Demand increase by 50% at busbar 9

Compared to the no wind the case, system voltage profile is improved by connecting the DFIG to strong busbar 9, as seen from the table (3.9). The system voltage value

was better   with higher the penetration level.

Compared to the DFIG at weak busbar 14, the case, system voltage profile is improved by connecting the wind farm (DFIG) to strong busbar 9.

Figure 3.8: Voltage Profile (p.u) 50% Load at busbar 9

1 Table 3.9: Bus Voltage Profile (p.u) 50% Load at busbar 9

3.2.2.3-Voltage Profile Analysis with DFIG – Load Demand increase by 50% were connected to busbar 9 and busbar 28 simultaneously

Compared to the no-wind case, the system voltage profile was improved by connecting the DFIG at both busbars 9 and 28, as shown in Table (3.10). The system voltage values were better with higher levels of wind penetration.

Compared to the case of the DFIG connected only at the strong busbar 9, the voltage profile at the distant buses was improved by connecting the wind farm (DFIG) at both strong buses. This configuration was able to accommodate higher penetration levels without negatively impacting the network.

Figure 3.9: Voltage Profile (p.u) 50% Load at busbar 9 and 28

Table 3.10: Bus Voltage Profile (p.u) 50% Load at busbar 9 and 28

Chapter 4

Conclusion and Recommendations

Conclusion

As a result, there has been an increase in the production of electricity from wind generation. For example, China’s total wind generation capacity jumped from 1,260 MW in 2005 to 62,733 MW by the end 2011. Moreover, there has been a rapid development in the technologies used for wind turbine generators. The capacity of individual turbines has grown from a few kilowatts to more than 13 MW. This project has discussed the system facilities used to connect wind farms in order to meet the requirements set by the grid operator. also been discussed This project, which has defined the relationship between wind speed and power output. Different wind turbine generators in current use have been presented.

The integration of DFIG into existing transmission network (IEEE-30 busbar system) and the impact on network has been presented in this project. Furthermore, case study results have been presented regarding the impact of wind generation on system voltage profile. The type of wind generator used was of variable speed with a doubly fed induction generator (DFIG) which was modelled on load flow studies as a PV busbar and operated in voltage controlled mode. . Different penetration levels from 10% to 20% were used and different wind connection scenarios were used in the network to analyse the impact of DFIG. From the results presented in this project, it can be concluded that the system voltage profile is affected positively or negatively depending on penetration level, of wind generator output, and location of wind generator connection. When DFIG was connected to a stronger busbar it was able to improve system voltage profile and the higher the penetration level the better. The reverse was found to be true when wind generation was connected to a weaker bus, in which case reactive power compensation (e.g. SVC) may still be necessary, especially if a large penetration of DFIG is connected to a weak area. The results obtained show that system voltage profile was better for a single wind farm connection location connected to a stronger busbar compared to multiple locations of wind farms for the same amount of wind penetration level.

Recommendations:

Based on the detailed analysis of voltage performance under various wind

penetration levels and generator placements in the IEEE 30-busbar system,

the following recommendations are proposed:

1. Reactive Power Compensation:

In cases where DFIG-based wind farms are connected to weak busbars, voltage support may become insufficient, especially under high wind penetration levels. It is recommended to integrate dynamic reactive power compensation devices such as STATCOM or SVC to enhance voltage stability.

2. Optimal Wind Farm Siting:

The location of wind farm connection has a critical impact on the system's voltage profile. Therefore, system planners should prioritize connecting wind farms to stronger busbars where voltage regulation is more stable, especially under increased load scenarios.

3.Penetration Level Evaluation:

While higher wind penetration can improve the voltage profile when connected properly, it may degrade system stability if not managed correctly. Utilities should evaluate the acceptable penetration limits for each region based on voltage sensitivity analysis.

4 Distributed Wind Integration Is Preferred

Connecting wind generation to multiple strong and weak buses (e.g., Busbars 9 and 28) yields better voltage support across the network compared to concentrating generation at a single location.

5 Use of DFIG Wind Turbines

DFIGs demonstrated strong voltage regulation capabilities, especially when

operated within the power factor limits (±0.95). Their reactive power control

should be optimized to support grid voltage profiles under variable loads

6 Future Studies:

It is recommended to study:

The impact of dynamic load variation and fault conditions

The inclusion of energy storage systems to mitigate variability

Economic analysis comparing costs of different wind penetration strategies

# Appendix A: IEEE 30-Busbar Test System

The test system consists of six generators, forty one branches and twenty one load

- Busbar Data and Load Flow Results

| Busbar No. | Busbar Voltage |  | Generation |  | Load |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Magnitude (p.u.) | Angle (Deg.) | Real (MW) | Reactive (MVAr) | Real (MW) | Reactive (MVAr) |
| 1 | 1 | 0 | 138.79 | -39.13 | 0 | 0 |
| 2 | 1 | 0 | 57.6 | 15.13 | 21.7 | 12.7 |
| 3 | 1 | 0 | 0 | 0 | 2.4 | 1.2 |
| 4 | 1 | 0 | 0 | 0 | 7.6 | 1.6 |
| 5 | 1 | 0 | 24.6 | 48.53 | 94.2 | 19 |
| 6 | 1 | 0 | 0 | 0 | 0 | 0 |
| 7 | 1 | 0 | 0 | 0 | 22.8 | 10.9 |
| 8 | 1 | 0 | 35 | 51.16 | 30 | 30 |
| 9 | 1 | 0 | 0 | 0 | 0 | 0 |
| 10 | 1 | 0 | 0 | 0 | 5.8 | 2 |
| 11 | 1 | 0 | 17.9 | 6.99 | 0 | 0 |
| 12 | 1 | 0 | 0 | 0 | 11.2 | 7.5 |
| 13 | 1 | 0 | 16.9 | 14.12 | 0 | 0 |
| 14 | 1 | 0 | 0 | 0 | 6.2 | 1.6 |
| 15 | 1 | 0 | 0 | 0 | 8.2 | 2.5 |
| 16 | 1 | 0 | 0 | 0 | 3.5 | 1.8 |
| 17 | 1 | 0 | 0 | 0 | 9 | 5.8 |
| 18 | 1 | 0 | 0 | 0 | 3.2 | 0.9 |
| 19 | 1 | 0 | 0 | 0 | 9.5 | 3.4 |
| 20 | 1 | 0 | 0 | 0 | 2.2 | 0.7 |
| 21 | 1 | 0 | 0 | 0 | 17.5 | 11.2 |
| 22 | 1 | 0 | 0 | 0 | 0 | 0 |
| 23 | 1 | 0 | 0 | 0 | 3.2 | 1.6 |
| 24 | 1 | 0 | 0 | 0 | 8.7 | 6.7 |
| 25 | 1 | 0 | 0 | 0 | 0 | 0 |
| 26 | 1 | 0 | 0 | 0 | 3.5 | 2.3 |
| 27 | 1 | 0 | 0 | 0 | 0 | 0 |
| 28 | 1 | 0 | 0 | 0 | 0 | 0 |
| 29 | 1 | 0 | 0 | 0 | 2.4 | 0.9 |
| 30 | 1 | 0 | 0 | 0 | 10.6 | 1.9 |

IEEE 30 busbar system

- Line Data

| Line Between Busbars |  |  | Line Impedance |  |  | Half Line Charging Susceptance (p.u.) |
| --- | --- | --- | --- | --- | --- | --- |
| From |  | To | R (p.u.) | X (p.u.) |  |  |
| 1 |  | 2 | 0.0192 | 0.0575 |  | 0.0528 |
| 1 |  | 3 | 0.0452 | 0.1852 |  | 0.0408 |
| 2 |  | 4 | 0.057 | 0.1737 |  | 0.0368 |
| 2 |  | 5 | 0.0472 | 0.1983 |  | 0.0418 |
| 2 |  | 6 | 0.0581 | 0.1763 |  | 0.0374 |
| 3 |  | 4 | 0.0132 | 0.0379 |  | 0.0084 |
| 4 |  | 6 | 0.0119 | 0.0414 |  | 0.009 |
| 4 |  | 12 | 0 | 0.256 |  | 0 |
| 5 |  | 7 | 0.046 | 0.116 |  | 0.0204 |
| 6 |  | 7 | 0.0267 | 0.082 |  | 0.017 |
| 6 |  | 8 | 0.012 | 0.042 |  | 0.009 |
| 6 |  | 9 | 0 | 0.208 |  | 0 |
| 6 |  | 10 | 0 | 0.35 |  | 0 |
| 6 |  | 28 | 0.0169 | 0.0599 |  | 0.013 |
| 8 |  | 28 | 0.0636 | 0.2 |  | 0.0428 |
| 9 |  | 10 | 0 | 0.055 |  | 0 |
| 9 |  | 11 | 0 | 0.208 |  | 0 |
| 10 |  | 17 | 0.0154 | 0.065 |  | 0 |
| 20 |  | 10 | 0.045 | 0.103 |  | 0 |
| 10 |  | 21 | 0.0348 | 0.0749 |  | 0 |
| 10 |  | 22 | 0.0727 | 0.1499 |  | 0 |
| 12 |  | 13 | 0 | 0.14 |  | 0 |
| 12 |  | 14 | 0.1231 | 0.2559 |  | 0 |
| 12 |  | 15 | 0.051 | 0.093 |  | 0 |
| 12 |  | 16 | 0.0945 | 0.1987 |  | 0 |
| 14 |  | 15 | 0.154 | 0.11 |  | 0 |
| 15 |  | 18 | 0.01 | 0.035 |  | 0 |
| 15 | 23 |  | 0.09 |  | 0.013 | 0 |
| 16 | 17 |  | 0.0621 |  | 0.092 | 0 |
| 18 | 19 |  | 0.045 |  | 0.015 | 0 |
| 19 | 20 |  | 0.015 |  | 0.038 | 0 |
| 21 | 22 |  | 0.0116 |  | 0.0236 | 0 |
| 22 | 24 |  | 0.08 |  | 0.093 | 0 |
| 23 | 24 |  | 0.085 |  | 0.15 | 0 |
| 24 | 25 |  | 0.035 |  | 0.1 | 0 |
| 25 | 26 |  | 0.086 |  | 0.13 | 0 |
| 25 | 27 |  | 0.086 |  | 0.13 | 0 |
| 28 | 27 |  | 0 |  | 0.14 | 0 |
| 27 | 29 |  | 0.2198 |  | 0.4153 | 0 |
| 27 | 30 |  | 0.17 |  | 0.31 | 0 |
| 29 | 30 |  | 0.164 |  | 0.301 | 0 |